Supplementary data for

Synergistic regulation of pollutant reduction and carbon mitigation in wastewater treatment plants under the rainfall-induced inflow and infiltration in the sewer network

Aliya Abulimitia,b, Yinfan Zhao b, Nan Zhaob, Xiuheng Wang b, c, Nanqi Ren b, c*

a School of Civil Engineering, Kashi University, Kashi 844000, China

b School of Environment, Harbin Institute of Technology, Harbin, 150090, PR China

c State key Laboratory of urban Water Resource and Environment, Harbin Institute of Technology, Harbin, 150090, P.R. China

* Corresponding author E-mail: rnq@hit.edu.cn

Supporting Information Contents:

The number of pages: 25

The number of texts: 8

The number of tables: 16

The number of figures: 5

Text A.1 Data preprocessing and temporal harmonization

The raw dataset included hourly influent flow rates, water quality indicators, chemical dosing rates, operational parameters, and meteorological variables. Prior to analysis, all datasets were subjected to quality control and harmonized to a consistent temporal resolution.

Outlier removal: Outliers were identified based on instrument-recording errors and process-related plausibility checks. Observations were removed when they were negative, physically impossible, clearly outside the operational range of the wastewater treatment plants, or attributable to sensor malfunction, maintenance activities, or data transmission errors. Typical examples included records in which the influent total nitrogen (TN) concentration was lower than the influent ammonium nitrogen (NH4+-N) concentration. In addition, negative or zero values in the influent flow series were truncated to the physical lower bound of 0.1 m3/h.

Missing-value imputation: Missing values in continuous time-series data were imputed using linear interpolation. Specifically, short-term gaps in influent flow rates, water quality concentrations, pH, and air temperature were filled by time-based linear interpolation. For longer gaps, the median value observed at the same hour on adjacent days was used when comparable operating conditions could be confirmed. Records were excluded from subsequent analysis when data gaps were excessively long or coincided with abnormal operating conditions. Missing rainfall values were not interpolated and were treated as no-rainfall periods in subsequent analyses.

Temporal harmonization: Because different variables were recorded at different temporal resolutions, all datasets were harmonized to an hourly scale. Variables originally recorded at sub-hourly resolution were aggregated into hourly averages or hourly totals according to their physical meaning. Rainfall was aggregated as hourly cumulative precipitation, whereas air temperature, water quality concentrations, and pH were expressed as hourly averages. Chemical dosing rates were originally recorded as daily totals and were allocated to the hourly scale in proportion to the hourly influent flow rate of the corresponding day. Operational parameters, such as dissolved oxygen setpoints and return flow ratios, were assumed to remain constant within the same day and were therefore assigned directly to each hourly record. Daily or lower-frequency measurements were matched to the corresponding hourly records only when temporal consistency could be ensured. The temporal coverage, original resolution, preprocessing methods, and missing-value treatment for each variable are summarized in Table A1.

Table A1 Data categories, time periods, resolutions, sources, and preprocessing methods.

| Data category | Variable | Time period | Original resolution | Unified resolution | Data source |
| --- | --- | --- | --- | --- | --- |
| Influent flow | Influent flow rate | Jan – Dec 2022 | Hourly | Hourly | Wastewater treatment plant (WWTP) monitoring system |
| Water quality | Influent and effluent COD | May – Sep 2022 | Hourly / routine records | Hourly |  |
|  | Influent and effluent TN | May – Sep 2022 | Hourly / routine records | Hourly |  |
|  | Influent and effluent TP | May – Sep 2022 | Hourly / routine records | Hourly |  |
|  | Influent and effluent NH4+-N | May – Sep 2022 | Hourly / routine records | Hourly |  |
|  | pH | May – Sep 2022 | Hourly | Hourly |  |
| Chemical dosing | Chemical dosing rate | May – Sep 2022 | Daily | Hourly |  |
| Operational parameters | Key process parameters | May – Sep 2022 | Daily / operation records | Hourly |  |
| Meteorological data | Rainfall, air temperature | May – Sep 2022 | Hourly | Hourly | China Meteorological Data Service Centre |

Text A.2 Development of baseline flow prediction models

Text A.2.1 Data preprocessing and identification of rainfall-affected periods

To construct the baseline domestic wastewater flow, this study developed time-series prediction models based on hourly influent flow data from the WWTPs. The raw data were first processed by standardizing timestamp formats, resampling to an hourly scale, and sorting chronologically. Invalid and missing values in the influent flow series were filled using linear interpolation. If a small number of missing values remained after interpolation, forward filling and backward filling were further applied to ensure the continuity of the hourly time series.

Rainfall data were mainly used to identify rainfall events and their subsequent impact periods. An hour was classified as a rainfall period when the rainfall depth in that hour was greater than 0. For adjacent rainfall events, if the intervening dry interval was short, the events were treated as part of the same continuous rainfall-affected process. Because background groundwater infiltration in the study area was relatively weak, abnormal increases in influent flow during the rainy season were assumed to be mainly caused by rainfall and post-rainfall sewer network responses. Therefore, positive residuals between the observed influent flow and the predicted baseline flow were used to represent rainfall-related inflow and infiltration (I/I) increments.

The dataset was split chronologically into training and testing subsets, with the first 80% of the time-series data used for model training and the remaining 20% used for model testing. This chronological split avoided temporal information leakage caused by random partitioning and was appropriate for continuous time-series data such as WWTP influent flow.

Text A2.2 Random forest model

The random forest (RF) model was used to capture nonlinear relationships between influent flow and temporal features, historical statistical features, and lagged flow features. The model inputs  included basic temporal features, categorical temporal features, historical statistical features, and lagged flow features (Table A2). The basic temporal features included hour, day, month, year, day of week, day of year, and week of year. The categorical temporal features included weekday indicator, weekend indicator, season, and intraday time period. The historical statistical features included the 24 h and 72 h rolling mean influent flows. The lagged flow features included influent flow at 1 h, 2 h, 3 h, 6 h, 12 h, and 24 h before the prediction time.

To reduce the influence of differences in numerical scales among variables, Min-Max normalization was applied to influent flow, rolling mean flow, lagged flow, and other numerical variables. The RF regression model was configured with 200 decision trees, a maximum tree depth of 15, a minimum of 5 samples required for node splitting, and a minimum of 2 samples per leaf node. The square-root strategy was used for feature selection at each split, and a fixed random seed was set to ensure reproducibility. The main model parameters are summarized in Table A3. Model outputs were inverse-transformed to restore the predicted values to the original flow scale.

Text A.2.3 Long short-term memory network model

The long short-term memory network (LSTM) model was used to characterize temporal dependencies in the influent flow series. To enhance the ability of the model to identify periodic variations, sine–cosine encoding was applied to the hour and weekday variables, generating four periodic features: hour sine, hour cosine, weekday sine, and weekday cosine. The model inputs included normalized influent flow and these four periodic temporal features (Table A2).

Considering the pronounced diurnal pattern of WWTP influent flow, a 24 h time step was used in the LSTM model; that is, the influent flow and periodic features from the previous 24 h were used to predict the influent flow at the current time. The model adopted a two-layer stacked LSTM architecture. The first LSTM layer contained 64 units and returned the full sequence, while the second LSTM layer contained 32 units to further extract temporal features. A dropout layer with a dropout rate of 0.2 was applied after each LSTM layer to reduce the risk of overfitting. The output layer was a fully connected layer that predicted the normalized influent flow at the current time.

The model was trained using the Adam optimizer, with mean absolute error (MAE) as the loss function. The maximum number of training epochs was set to 100, the batch size was set to 32, and 10% of the training data were used as the validation set. Early stopping was applied during training. Specifically, training was terminated when the validation loss did not decrease for 10 consecutive epochs, and the model parameters corresponding to the lowest validation loss were retained. The main model parameters are listed in Table A3.

Text A.2.4 Prophet model

The Prophet model was used to represent the trend, daily seasonality, and weekly seasonality of influent flow. Considering the pronounced seasonal characteristics of the regional climate and influent flow fluctuations, a season-specific modelling strategy was adopted. The year was divided into spring, summer, autumn, and winter (Table A2). Spring included April and May, summer included June to August, autumn included September and October, and winter included November to March of the following year. During seasonal transition periods, the predictions from adjacent seasonal models were smoothly connected to avoid abrupt changes in the baseline flow caused by seasonal switching.

For each season, the Prophet model included daily and weekly seasonal components and used an additive seasonality mode. The changepoint prior scale was set to 1, the seasonality prior scale was set to 10, and both daily and weekly seasonality were included. During model training, an iterative residual-filtering strategy was used to identify and remove abnormal disturbance points. Specifically, a Prophet model was first trained using the samples from the corresponding season to generate the predicted baseline flow. Residuals were then calculated as the difference between observed influent flow and predicted baseline flow, and abnormal points were identified using a k-σ criterion. After removing the abnormal points, the model was retrained. This process was repeated for a maximum of 10 iterations, with k set to 1.1.

Multiple stopping criteria were used during the iterative modelling process. Iteration was terminated early when the Anderson–Darling test of the residual distribution satisfied the predefined stability condition, when the maximum positive residual was lower than the predefined threshold, or when the correlation between predictions from two consecutive iterations exceeded 0.999. The main model parameters are provided in Table A3. After model training, the season-specific models were used to generate a complete hourly baseline flow series. Missing baseline flow values caused by seasonal transitions or local data gaps were filled using time-based interpolation, followed by forward and backward filling, to obtain a continuous baseline domestic wastewater flow series.

In the final identification of I/I, only positive anomalous residuals in which the observed influent flow exceeded the predicted baseline flow were treated as rainfall-related flow increments. This prevented negative residuals from being misclassified as extraneous water intrusion. To reduce misclassification caused by single-hour fluctuations, a 3 h moving-window majority voting approach was applied to smooth the wet-period labels.

Test A.2.5 Model evaluation metrics and selection criteria

Model prediction performance was evaluated using mean absolute error (MAE) and root mean square error (RMSE):

MAE=1ni=1nyi-yi#1

RMSE=1ni=1nyi-yi2#2

where (n) is the number of samples, yi is the observed influent flow, m3/h, and yi is the model-predicted influent flow, m3/h. MAE reflects the average absolute magnitude of prediction errors, whereas RMSE is more sensitive to large errors.

Because the objective of this study was not merely short-term flow forecasting but the construction of a baseline domestic wastewater flow, model selection was based not only on prediction error but also on the ability of each model to characterize daily and weekly periodicity, prediction stability, dependence on lagged flow variables, and interpretability.

Table A2 Input variables of the baseline flow prediction models

| Model | Input variable category | Specific variables | Purpose |
| --- | --- | --- | --- |
| RF | Basic temporal features | Hour, day, month, year, day of week, day of year, week of year | Characterize flow variations at different temporal scales |
|  | Categorical temporal features | Weekday indicator, weekend indicator, season, intraday time period | Distinguish differences between weekdays and weekends, seasons, and intraday periods |
|  | Historical statistical features | 24 h rolling mean, 72 h rolling mean | Represent short- and medium-term flow variation trends |
|  | Lagged flow features | Qt-1,Qt-2,Qt-3,Qt-6,Qt-12,Qt-24 | Characterize temporal dependence in influent flow |
| LSTM | Flow sequence | Normalized flow during the previous 24 h | Represent the flow variation process over the preceding 24 h |
|  | Periodic temporal features | Hour sine, hour cosine, weekday sine, weekday cosine | Characterize daily and weekly periodicity |
| Prophet | Time-series variables | Timestamp and hourly influent flow | Construct trend, daily seasonality, and weekly seasonality components |
|  | Seasonal grouping variables | Spring, summer, autumn, winter | Account for seasonal differences in flow fluctuations |
|  | Residual identification variable | Residual between observed influent flow and predicted baseline flow | Iteratively identify and remove abnormal disturbance points |

Table A3. Main parameter settings of the baseline flow prediction models

| Model | Parameter | Setting |
| --- | --- | --- |
| RF | Data split | 80% training and 20% testing in chronological order |
|  | Normalization | Min-Max normalization |
|  | Number of trees | 200 |
|  | Maximum tree depth | 15 |
|  | Minimum samples for split | 5 |
|  | Minimum samples per leaf | 2 |
|  | Maximum features | Square root |
|  | Random state | 42 |
| LSTM | Data split | 80% training and 20% testing in chronological order |
|  | Validation split | 10% of the training data |
|  | Normalization | Min-Max normalization for flow |
|  | Sequence length | 24 h |
|  | Input dimension | 5 variables |
|  | LSTM layers | 64 units and 32 units |
|  | Activation function | tanh |
|  | Dropout rate | 0.2 |
|  | Optimizer | Adam |
|  | Loss function | MAE |
|  | Epochs | 100 |
|  | Batch size | 32 |
|  | Early stopping | Patience = 10 |
| Prophet | Seasonal modelling | Spring, summer, autumn, and winter |
|  | Seasonality mode | Additive |
|  | Daily seasonality | Included |
|  | Weekly seasonality | Included |
|  | Changepoint prior scale | 1 |
|  | Seasonality prior scale | 10 |
|  | Maximum iterations | 10 |
|  | Residual filtering | k-σ criterion, k = 1.1 |
|  | Positive residual threshold | Used for final I/I identification |
|  | Wet-period smoothing | 3 h moving-window majority voting |
|  | Stopping criteria | Residual stability, high prediction correlation, or maximum iteration |

Text A.3 Development of the WWTP mechanistic model

Text A.3.1 Model configuration

WWTP B features parallel AAO and AO treatment lines with a flow distribution ratio of 1:1. The model consists of screens, grit chambers, AAO, AO, secondary clarifier, and sand filters. Screens, grit chambers, and sand filters were simplified as non-reactive separation units. The AAO and AO bioreactors were represented by multiple completely mixed reactors in series to approximate plug-flow behavior: the AAO line was divided into anaerobic, anoxic, and aerobic zones (with symmetric structure, identical dimensions and operating conditions for the same functional zones); the AO line was divided into anoxic and aerobic zones. Reactor volumes followed the as-built dimensions. The secondary clarifier was simulated using a three-layer model coupled with biological reactions and fixed sludge withdrawal; Polyaluminium chloride (PAC) was dosed before the clarifier for chemical phosphorus removal. Part of the settled sludge was returned to the bioreactors, and the remainder was sent to thickening and dewatering.

The sludge thickener also used a three-layer model with Polyacrylamide (PAM) addition; dewatering was simplified as a non-reactive unit with solid removal efficiency based on plant records. Key operating parameters: internal recycle ratio 200% for AAO (aerobic to anoxic) and 300% for AO (aerobic to anoxic); external recycle ratio 75%; DO controlled at 3 mg/L in aerobic zones. Hydraulic connections were modeled using non-reactive pipe modules. Main reactor volumes and operating conditions are summarized in Table A4 and Table A5.

Table A4 Configuration of the WWTP B.

| Section | Parameter | Value | Unit |
| --- | --- | --- | --- |
| AAO | Pre‑anaerobic zone | 2293.8 | m3 |
|  | Anaerobic zone | 2293.8 | m3 |
|  | Anoxic zone | 10000.0 | m3 |
|  | Transition zone | 3336.0 | m3 |
|  | Aerobic zone | 18766.9 | m3 |
| AO | Anoxic zone | 10155.5 | m3 |
|  | Aerobic zone | 39539.0 | m3 |
| Secondary clarifier |  | 10112.0 | m3 |

Table A5 Key operating conditions for model calibration.

| Configuration | Flow description | Value | Unit |
| --- | --- | --- | --- |
| General conditions | Temperature | 20 | ℃ |
|  | pH | 7.2 | pH unit |
| AAO aerobic zone | DO | 3.0 | g O2/m3 |
| AO aerobic zone | DO | 3.0 | g O2/m3 |
| Internal recycle (AAO) | Aerobic→anoxic | 200 | % |
| Internal recycle (AO) | Aerobic→anoxic | 300 | % |
| External recycle | Secondary clarifier→bioreactors | 75 | % |

Text A.3.2 Sensitivity analysis

The model parameters include influent characteristics, reactor geometry, operational parameters, and influent fractionation and stoichiometric parameters. The first three categories were set according to plant records. For influent fractionation and stoichiometric parameters, the SUMO‑4N model contains 384 kinetic/stoichiometric parameters and 15 wastewater fractionation parameters. To reduce computational burden, parameters that do not vary with external conditions were kept at default values. A total of 35 kinetic parameters and 15 fractionation parameters were selected for local sensitivity analysis.

A one‑at‑a‑time perturbation method was applied, and the relative sensitivity factor (RSF) was calculated as:

RSF=Yi1-Yi0Yi0Xi1-Xi0Xi0#3

where Xi0 and Xi1 are the parameter values before and after perturbation, respectively, and Yi0 and Yi1 are the corresponding model outputs. Parameters with |RSF|≥0.1were considered sensitive and were prioritized during calibration. The sensitivity analysis results for key parameters are summarized in Table A6.

Table A6 Sensitivity analysis results of key SUMO parameters.

| Parameter | Unit | ¦RSF¦ | Influence factor |
| --- | --- | --- | --- |
| μAOB | g O2/m3 | 0.16, 0.71, 1.43 | TN, TP, NH4+-N |
| KO2,AOB | g O2/m3 | 0.48 | NH4+-N |
| KO2,NHx,AOB | g O2/m3 | 0.71 | TP |
| KNHx,NH2OH,AOB | g N/m3 | 0.71, 0.48 | TP, NH4+-N |
| μNOB | 1/d | 0.24 | TN |
| KSB | g COD/m3 | 0.15, 0.71, 0.48 | TN, TP, NH4+-N |
| KSB,NO | g COD/m3 | 0.71 | TP |
| KSB,NO2 | g COD/m3 | 0.11 | TN |
| KO2,OHO | g O2/m3 | 0.48 | NH4+-N |
| KO2,OHO,NO2 | g O2/m3 | 0.13 | TN |
| qPAO,PP | 1/d | 0.71 | TP |
| qHYD | 1/d | 0.71, 0.48 | TP, NH4+-N |
| frVSS,TSS | % | 0.29, 0.10 | TP, NH4+-N |
| frSCCOD,TCOD | % | 0.14 | TP |
| frSCOD,TCOD | % | 0.14 | TP |
| frVFA,SCCOD | % | 0.14 | TP |
| frSU,SCCOD | % | 2.35, 0.15, 0.57, 0.10 | COD, TN, TP, NH4+-N |
| frXU,TCOD | % | 0.29, 0.29 | TP, NH4+-N |
| frSPO4,TP | % | 0.14, 0.10 | TP, NH4+-N |

Text A.3.3 Steady-state calibration

Steady-state calibration was performed to bring the model to a stable operating condition. The simulation period was 30 days with a 1 h recording interval. Input data consisted of monthly average influent flow, water quality, and operating conditions from May to September 2022. After steady-state simulation using the default kinetic parameters, the model was further calibrated by manually adjusting stoichiometric and kinetic parameters based on engineering experience.

Currently, no universal standard exists for digital model accuracy in wastewater treatment. The WERF guidelines recommend that relative errors for steady-state simulation be controlled between 5% and 20%. The steady-state calibration results are shown in Table A7. The monthly average simulated effluent values agreed well with observed data, with all relative errors below 10%, indicating satisfactory model performance for subsequent dynamic simulation.

Table A7 Steady-state calibration results.

| Month | COD |  | TN |  | NH4+-N |  | TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Observed (mg/L) | Simulated (mg/L) | Observed (mg/L) | Simulated (mg/L) | Observed (mg/L) | Simulated (mg/L) | Observed (mg/L) | Simulated (mg/L) |
| May | 25.455 | 25.592 | 11.997 | 12.039 | 0.615 | 0.620 | 0.264 | 0.266 |
| June | 26.131 | 26.417 | 12.398 | 12.119 | 0.782 | 0.800 | 0.213 | 0.215 |
| July | 26.590 | 26.138 | 12.529 | 12.490 | 0.911 | 0.896 | 0.197 | 0.195 |
| Aug | 22.553 | 22.921 | 12.866 | 12.701 | 0.957 | 0.962 | 0.157 | 0.155 |
| Sep | 24.377 | 24.394 | 12.586 | 12.580 | 0.711 | 0.701 | 0.183 | 0.178 |

# Text A.4 Scenario-based performance evaluation

Text A.4.1 GHG emission

Our study exclusively evaluated GHG emissions during the operational stage of wastewater treatment and in-plant sludge treatment, as the operational stage contributed to the vast majority of life-cycle emissions[1]. This included direct emissions and indirect emissions from energy and chemical consumption.

Direct emission: The SUMO‑4N model implemented in the Dynamita SUMO platform was used to simulate direct GHG emissions during the operation of the WWTP. The model describes the metabolic pathways of key microbial groups in biological wastewater treatment and simulates the production and release of CH4, N2O and CO2 in each treatment unit.

CH4 production is mainly associated with the degradation of organic matter under anaerobic conditions and is influenced by dissolved oxygen, nitrate, ammonium, and volatile fatty acids[2]. In the SUMO‑4N model, CH4 primarily originates from the conversion of organic substrates by methanogens in anaerobic tanks. Part of the dissolved CH4 can be transported into subsequent anoxic or aerobic tanks via internal recirculation and hydraulic conveyance, and is then stripped from the liquid phase to the gas phase under aeration disturbance and surface diffusion.

Direct N2O emissions are mainly related to the metabolism of ammonia‑oxidizing bacteria (AOB)[3]. The SUMO‑4N model considers two main N2O generation pathways: (i) N2O formation through the oxidation of hydroxylamine (NH2OH) during nitrification, and (ii) N2O generation via AOB denitrification under conditions of insufficient dissolved oxygen and nitrite accumulation.

Direct CO2 emissions in the SUMO‑4N model consider CO2 generated from the oxidation of fossil carbon in the influent. The fossil carbon fraction in influent total organic carbon was set to the default value of 10% to estimate CO2 emissions from fossil carbon oxidation[4]. In addition, the transformation of the external carbon sources by biological processes, although relatively complex, largely follows the assumption that two-thirds of the carbon source is used for catabolism and one-third for anabolism. In this study, methanol was assumed to be the carbon source. Therefore, the CO2 emission from carbon source conversion can be estimated using a conversion coefficient of 0.92 kg CO2-eq/kg CH3OH[5].

To deal with the different nature of the generated GHG emissions (CO2, CH4, and N2O) on Global Warming potential (GWP), they are converted into units of CO2 equivalents (CO2-eq).

Ef-ec=0.92×Wm#4

Edirect=28×ECH4+265×EN2O+Ef-CO2+Ef-ec#5

where Edirect is the direct emission from WWTPs (kg/h), ECH4 is the CH4 emission simulated by SUMO (kg/h), EN2O is the N2O emission simulated by SUMO (kg/h), Ef-CO2 is the fossil‑source CO2 simulated by SUMO (kg/h), and Ef-ec is the fossil‑source CO2 caused by external carbon source addition (kg/h), Wm is the daily consumptions of methanol (kg/d). The GWPs of CH4 and N2O are 28 and 265 kg CO2‑eq/kg, respectively (IPCC, 2019)[6].

Indirect GHG emissions: Indirect GHG emissions mainly include those from electricity and chemical consumption. The indirect emissions of the WWTP (Eindirect, kg CO2-eq/d) were calculated using Eq. (6) – (8).

Eelectricity=EF×Uelectricity#6

Echemical=EF1×Wm+EF2×Wp+EF3×Ws+EF4×Wn#7

E indirect=Eelectricity+Echemical#8

where Eelectricity and Echemical are the CO2 equivalents of consumed electricity and chemicals (kg CO2-eq/d), EF is the emission factor for electricity (kg CO2/kWh), taken from the provincial average CO2 emission factor for electricity in 2023 calculated by the Ministry of Ecology and Environment and the National Bureau of Statistics. Uelectricity is the energy consumption of the WWTP (kWh/d). EF1, EF2 EF3 and EF4 are the emission factors for methanol, PAC, PAM, and NaClO, respectively (kg CO₂‑eq/kg), taken from the Guidelines for Carbon Emission Accounting of Urban Wastewater Treatment Plants. Wm, Wp, Ws and Wn are the daily consumptions of methanol, PAC, PAM, and Sodium hypochlorite (NaClO, kg/h), respectively. All emission factors are listed in Table A8.

Table A8 Emission factors for electricity and chemicals.

| Name | Emission factor（kg CO2-eq /kg） |
| --- | --- |
| Electricity | 0.53 |
| Methanol | 0.61 |
| PAC | 1.62 |
| PAM | 1.48 |
| NaClO | 0.99 |

Text A.4.2 Operating cost accounting

Operating cost accounting focused on electricity and chemical costs, because these two components are directly affected by hydraulic loading and operational adjustment[7]. Other cost components, including labor, maintenance, depreciation, construction, and sludge disposal costs, were not included. The unit costs of electricity and each chemical are shown in Table A9.

Table A9 Unit costs of electricity and chemicals

| Item | Unit cost |
| --- | --- |
| Electricity | 0.619 CNY/kWh |
| Methanol | 2640 CNY/ton |
| PAC | 1350 CNY/ton |
| PAM | 15000 CNY/ton |
| NaClO | 850 CNY/ton |

Text A.4.3 Rainfall‑induced I/I event identification, scenario classification and model input setup

Rainfall event identification: To construct rainfall‑induced I/I scenarios, independent rainfall events were identified from hourly rainfall data. Two rainfall periods separated by a dry interval of at least 6 consecutive hours were treated as independent events. To avoid interference from negligible rainfall, only events with a total rainfall depth greater than 1 mm were defined as effective rainfall events and used for subsequent rainfall‑induced I/I analysis. For each effective event, total rainfall depth, rainfall duration, and maximum hourly rainfall intensity were extracted.

Scenario classification: The rainfall‑induced I/I scenarios were classified according to the event‑total rainfall depth, following the thresholds of the Chinese precipitation grade standard (GB/T 28592‑2012). Because the study focuses on the impact of individual rainfall events on WWTP operation, scenario classification based on event‑total rainfall depth is more appropriate than using 24 h or 12 h accumulated rainfall. Five rainfall‑induced I/I scenarios were included in the SUMO simulations: no rain (dry‑weather baseline), light rain (1.0–9.9 mm), moderate rain (10.0–24.9 mm), heavy rain (25.0–49.9 mm), and rainstorm (50.0–99.9 mm). Severe rainstorm (100.0–249.9 mm) and extraordinary rainstorm (≥250.0 mm) were excluded from the regular scenario simulations. The exclusion reasons are: according to the local rainfall intensity equation, rainfall events reaching these grades correspond to return periods generally greater than 100 years, and the resulting rainfall‑induced I/I volume may increase the WWTP influent flow to more than 1.5 times the design capacity. Under such extreme conditions, the treatment system may not maintain stable normal operation, and flow restriction or bypass discharge is likely to be triggered. Therefore, these extreme events were considered beyond the scope of this study, which focuses on controllable WWTP operation under rainfall‑induced I/I disturbances.

Baseline selection and rainfall‑induced I/I volume estimation: To enable a fair comparison among different rainfall‑induced I/I scenarios, a dry‑weather day with no rainfall in the preceding 48 hours was selected as the baseline day. The influent flow rate and pollutant concentrations of this baseline day were used as the dry‑weather reference values Qbasic and Cko. The rainfall‑induced I/I volume for each rainfall scenario (QI/I) was estimated using the rainfall–I/I quantitative relationship established in Section 3.1 of the main text. For the incompletely separate sewer system studied, the total I/I volume of a rainfall event was linearly correlated with the event‑total rainfall depth per unit area of the combined sewer catchment. The specific regression coefficients and the derivation of the relationship are presented in Section 3.1. The I/I volumes of the five included scenarios were calculated using this relationship together with the event‑total rainfall depth of each grade.

Influent concentration adjustment: When I/I enters the sewer network, the WWTP influent flow increases while the pollutant load from domestic sewage remains approximately constant. Consequently, influent pollutant concentrations are diluted. In addition, stormwater runoff carries pollutants washed from the ground surface, which may partly offset the dilution effect. To account for these two opposing effects, the influent concentration of each pollutant under a given rainfall‑induced I/I scenario was calculated using a mass‑balance mixing equation:

Ck=Ck0×Qbasic+Ck,II×QIIQ#9

Where Ck is the influent concentration of pollutant k under the rainfall‑induced I/I scenario (mg/L); Qbasic is the dry‑weather baseline influent flow rate (m3/h); Ck0 is the dry‑weather baseline influent concentration of pollutant k (mg/L); QII is the rainfall‑induced I/I flow rate (m3/h) for the scenario; Ck,II is the concentration of pollutant k in the infiltrated water (stormwater runoff plus possible groundwater), mg/L. The values of Ck,II for COD, TN, TP, and NH4+‑N were adopted from a literature compilation of urban stormwater runoff quality[8] (Table A10).

Table A10 The values of Ck,II

| Flow description | Value | Unit |
| --- | --- | --- |
| COD | 267.92 | mg/L |
| TN | 4.35 | mg/L |
| TP | 0.57 | mg/L |
| NH3-N | 1.9 | mg/L |

Constant operating parameters across scenarios: To isolate the independent effect of rainfall‑induced I/I on WWTP performance, all process operating parameters were kept identical to the dry‑weather baseline scenario across the five rainfall‑induced I/I scenarios. Specifically： Internal recycle ratio: 200% for the AAO line and 300% for the AO line; External recycle ratio (return sludge ratio): 75% for both lines; Aeration control: DO setpoint at 3 mg/L in aerobic zones; Carbon source and phosphorus removal chemical (PAC) dosages: kept constant (not adjusted with influent flow or concentration); Flocculant (PAM) and disinfectant (sodium hypochlorite) dosages: adjusted proportionally to the influent flow rate, at rates of 4 g/t and 10.5 g/t of influent, respectively.

# Text A.5 Multi‑objective optimization method for synergistic reduction of pollutants and GHG emissions

Text A.5.1. Decision variables and optimization settings

The heavy-rain scenario was selected as the representative optimization condition because it caused marked increases in nitrogen fluctuation, operating cost, and GHG emissions. Dissolved oxygen (DO), internal recirculation ratio (Rint), and external recirculation ratio (Rext) were selected as decision variables because they directly affect nitrification, denitrification, sludge concentration, recirculation energy consumption, and process-related GHG emissions.

The variable ranges were determined based on actual plant operation, engineering experience, and preliminary SUMO simulations. A Box-Behnken design was used to construct response surface surrogate models, and NSGA‑II was used to identify Pareto‑optimal solutions. The objectives are to minimize the comprehensive water‑quality (WQ) index (combining peak TN and NH4+‑N), total GHG emissions, and total operating cost.

Table A11 Decision‑variable ranges and NSGA‑II parameters used for multi‑objective optimization.

| Category | Parameter | Value/Range |
| --- | --- | --- |
| Decision variable | DO | 1.0-4.0 mg/L |
|  | Rint | 100%-400% |
|  | Rext | 40%-100% |
| Box–Behnken level | DO | 1.0, 2.5, 4.0 mg/L |
|  | Rint | 100%, 250%, 400% |
|  | Rext | 40%, 70%, 100% |
| NSGA‑II | Population size | 100 |
|  | Offspring number | 50 |
|  | Crossover probability | 0.9 |
|  | Mutation probability | 1/3 |
|  | Maximum generations | 200 |

Text A.5.2. Response surface surrogate models

To reduce the computational burden of directly coupling SUMO with NSGA‑II, second‑order polynomial surrogate models were developed for each process line. The general form of the response surface model was:

Y=β0+β1DO+β2Rint+β3Rext+β11DO2+β12Rint2+β13Rext2+β1DORint+β2DORext+β3RextRint#10

Where represents each response variable, including WQ, operating cost, or GHG emissions. The coefficients 𝛽 were estimated from the Box-Behnken simulation results. The variables DO, Rint and Rext are expressed in mg/L, %, and %, respectively. Operating cost is expressed in 104 CNY, and GHG emissions are expressed in t CO2‑eq. Linear terms describe individual parameter effects, quadratic terms describe nonlinear responses, and interaction terms describe coupled effects between two parameters.

Text A.5.3. Fitted surrogate model equations

The fitted surrogate models used for multi‑objective optimization are listed below. Terms with negligible coefficients were not retained in the final expressions.

For the AAO process:

TNAAO=25.13-0.5DO-0.0060Rint-0.150Rext+0.13DO2+0.00007Rint2+0.0008Rext2+0.003DORint-0.0099DORext+0.0002RextRint#11

NH4+-NAAO=10.46-1.20DO-0.0054Rint-0.120Rext+0.12DO2+0.00051Rint2+0.00012DORint-0.0033DORext+0.0003RextRint#12

CostAAO=7.02+0.05DO-0.002Rint+0.0033Rext+0.0098DO2+0.0001Rint2+0.000057Rext2+0.00007DORint-0.000134DORext+0.00003Rint#13

GHGAAO=77.54-0.89DO-0.17Rint-0.11Rext+0.27DO2+0.00084Rint2+0.0011Rext2+0.00254DORint+0.0016DORext+0.000Rint#14

For the AO process:

TNAO=31.94-3.08DO-0.049Rint-0.250Rext+0.70DO2+0.0001Rint2+0.0016Rext2+0.006DORint-0.016DORext-0.00015RextRint#15

NH4+-NAO=8.71-1.17DO-0.0017Rint-0.110Rext+0.12DO2+0.00047Rint2-0.000014DORint+0.0039DORext+0.000011RextRint#16

CostAO=11.39+0.017DO-0.067Rint-0.0041Rext+0.025DO2+0.0003Rint2+0.00007Rext2+0.00007DORint-0.00014DORext-0.00031Rint#17

GHGAO=136.20-4.85DO-0.58Rint-0.55Rext+0.91DO2+0.0026Rint2+0.0032Rext2+0.0058DORint+0.0028DORext-0.0028Rint#18

## Text A.5.4. Model adequacy of response surface models

The response surface models showed acceptable fitting performance for subsequent optimization. The coefficients of determination were generally close to 1, and the coefficients of variation were below 10%, indicating good model reliability. The predicted R2 values of the TN models were lower than those of the other responses, suggesting that TN was affected by more complex interactions under I/I disturbance (Table A12). Nevertheless, the overall fitting statistics and significance tests indicated that the models were sufficient for identifying operational trends and Pareto‑optimal regions.

Table A12 Goodness‑of‑fit statistics of response surface surrogate models.

| Model | Std. Dev. | Mean | C.V. (%) | R2 | Adjusted R2 | Predicted R2 | Adeq. Precision |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AAO‑TN | 0.2615 | 12.60 | 2.0700 | 0.9842 | 0.9557 | 0.7471 | 18.9634 |
| AO‑TN | 0.9499 | 14.22 | 6.6800 | 0.9670 | 0.9075 | 0.7716 | 11.1570 |
| AAO‑NH4+‑N | 0.0607 | 2.84 | 2.1400 | 0.9983 | 0.9953 | 0.9729 | 58.9699 |
| AO‑NH4+‑N | 0.0878 | 2.26 | 3.8900 | 0.9954 | 0.9872 | 0.9270 | 36.8073 |
| AAO‑Cost | 0.0145 | 10.62 | 0.1366 | 1.0000 | 1.0000 | 0.9999 | 900.7151 |
| AO‑Cost | 0.0196 | 17.64 | 0.1108 | 1.0000 | 1.0000 | 1.0000 | 1623.6872 |
| AAO‑GHG | 0.1851 | 102.03 | 0.1815 | 1.0000 | 1.0000 | 0.9998 | 58.6727 |
| AO‑GHG | 1.04 | 161.29 | 0.6442 | 0.9999 | 0.9999 | 0.9992 | 262.4666 |

# Text A.6 Machine learning-based quantification of inflow and infiltration

Fig. A1 24-hour distribution characteristics of Influent data

Fig. A2 Weekly distribution characteristics of influent data

Table A13 Summary of performance evaluation of three models

| Model | RMSE | MAE | R1 | R2 |
| --- | --- | --- | --- | --- |
| RF | 686.27 | 514.08 | 0.94 | 0.96 |
| LSTM | 664.74 | 431.56 | 0.92 | 0.98 |
| Prophet | 705.44 | 521.26 | 0.83 | 0.81 |

Note: R1, correlation between the predicted and observed values; R2, correlation between the predicted values and the observed values lagged by one time step.

Fig. A3 Relationship between predicted wet periods, I&I volume, and rainfall for the WWTP in the separate sewer catchment: (a) May; (b) June; (c) July; (d) August; (e) September; (f) An example rainfall event.

Fig. A4 Relationship between predicted wet periods, I&I volume, and rainfall for the WWTP in the incompletely separate sewer catchment: (a) May; (b) June; (c) July; (d) August; (e) September; (f) An example rainfall event.

Text. A7 Effects of inflow and infiltration on pollutant removal and GHG–cost performance

The rainfall-induced I/I scenarios were constructed based on the empirical rainfall–I/I relationship established in Section 3.1. In that section, rainfall-induced I/I was first identified as the excess influent flow above the Prophet-predicted baseline domestic wastewater flow. Event-scale analysis showed that total rainfall depth was the dominant driver of I/I, and the normalized I/I volume increased by 154.7 m3/km2 for each 1 mm increase in rainfall depth.

For scenario simulation, four rainfall events representing light rain, moderate rain, heavy rain, and rainstorm were selected according to the rainfall classification criteria. Their 24-h rainfall depths were 9.1, 20.8, 35.4, and 53.2 mm, respectively (Table A14). The corresponding event-scale I/I volumes were estimated using Eq. (5) in the main text. The I/I volume was then distributed over the 24-h rainfall period according to the hourly rainfall pattern, and a 4-h delay was applied to represent the hydraulic response time between rainfall input and influent flow increase at the WWTP. Finally, the rainfall-induced I/I flow was superimposed on the baseline domestic wastewater flow to generate the scenario-specific 24-h influent flow profiles used in the SUMO simulations (Fig. A5).

Table A14 Rainfall depths used for rainfall-induced I/I scenario construction

| Scenario | Light rain | Moderate rain | Heavy rain | Rainstorm |
| --- | --- | --- | --- | --- |
| 24-h rainfall depth (mm) | 9.1 | 20.8 | 35.4 | 53.2 |

Fig. A5. Scenario-specific 24-h influent flow and hourly rainfall under rainfall-induced I/I conditions: (a) 24-h influent flow profiles of the WWTP under different rainfall-induced I/I scenarios with a 4-h hydraulic response delay; (b) hourly rainfall depth over 24 h.

To further quantify the integrated effects of rainfall-induced I/I on effluent quality, operating cost, and carbon emissions, sensitivity coefficients were calculated for each indicator using Eq. (6). The results are summarized in Table A13.

|  | （19） |
| --- | --- |

where S is the sensitivity coefficient of a given indicator to rainfall-induced I/I; ∆Y is the change in the indicator; Y0 is the value under the baseline dry-weather scenario; Yk is the value under the k-th I/I scenario; ∆Q is the change in influent flow, namely the I/I volume; Q0 is the influent flow under the baseline scenario; and Qk is the influent flow under the k-th scenario. Indicators were classified as extremely sensitive when S>2, highly sensitive when 1<S≤2, moderately sensitive when 0.5<S≤1, and weakly sensitive when S≤0.5. A positive S indicates a positive response to influent flow, whereas a negative S indicates an inverse response.

Table A15 Sensitivity coefficients of effluent quality, operating cost, and carbon-emission indicators under rainfall-induced I/I scenarios

| Indicator |  | Sensitivity coefficient |
| --- | --- | --- |
| Effluent quality | COD | -0.11 |
|  | TN | -0.02 |
|  | TP | -0.10 |
|  | NH4+-N | 1.52 |
| Operating cost | Electricity cost | 3.85 |
|  | Chemical cost | 0.25 |
|  | Unit wastewater treatment cost | 0.66 |
| GHG emission | Direct CO2 emissions | 0.08 |
|  | Direct CH4 emissions | 0.13 |
|  | Direct N2O emissions | 0.59 |
|  | Electricity-related indirect emissions | 3.85 |
|  | Chemical-related indirect emissions | 0.18 |
|  | Unit wastewater carbon emissions | 0.49 |

Text. A8. Significance ranking of operational variables

Analysis of variance was used to identify the dominant factors affecting each response. For both treatment lines, Rext was the dominant factor affecting nitrogen‑related water‑quality indicators (Table A16), whereas Rint was the dominant factor affecting operating cost and GHG emissions. This indicates that wet‑weather operation should control internal recirculation to reduce energy consumption while using external recirculation to stabilize nitrogen removal.

Table A16 Dominant operational factors identified from response surface analysis.

| Response | AAO process | AO process | Interpretation |
| --- | --- | --- | --- |
| TN | Rext＞Rint＞DO | Rext＞DO＞Rint | External recirculation strongly affects sludge concentration and nitrogen‑removal stability. |
| NH4+‑N | Rext＞DO＞Rint | Rext＞DO＞Rint | External recirculation and DO jointly regulate nitrification capacity. |
| Operating cost | Rint＞Rext＞DO | Rint＞DO＞Rext | Internal recirculation dominates pumping energy demand. |
| GHG emissions | Rint＞Rext＞DO | Rint＞Rext＞DO | Indirect emissions from electricity consumption dominate the carbon response. |

[1]	XI J, GONG H, GUO R, et al. Characteristics of greenhouse gases emission from wastewater treatment plants operation in China (2009–2016): A case study using operational data integrated method (ODIM) [J]. Journal of Cleaner Production, 2023, 402: 136829.

[2]	DAELMAN M R, VAN VOORTHUIZEN E M, VAN DONGEN U G, et al. Methane emission during municipal wastewater treatment [J]. Water research, 2012, 46(11): 3657-70.

[3]	POCQUET M, WU Z, QUEINNEC I, et al. A two pathway model for N2O emissions by ammonium oxidizing bacteria supported by the NO/N2O variation [J]. Water research, 2016, 88: 948-59.

[4]	LI L, WANG X, MIAO J, et al. Carbon neutrality of wastewater treatment-A systematic concept beyond the plant boundary [J]. Environmental Science and Ecotechnology, 2022, 11: 100180.

[5]	ABULIMITI A, WANG X, REN N. Exploring the pathway for the energy-environment synergistic WWTPs through comprehensive efficiency assessment [J]. Journal of Water Process Engineering, 2025, 76: 108269.

[6]	BAASANSUREN J, FUKUDA M, NGARIZE S, et al. 2019 refinement to the 2006 IPCC guidelines for national greenhouse gas inventories [Z]. IPCC Switzerland. 2019

[7]	LUO L, DZAKPASU M, YANG B, et al. A novel index of total oxygen demand for the comprehensive evaluation of energy consumption for urban wastewater treatment [J]. Applied Energy, 2019, 236: 253-61.

[8]    YANG M, PAN X, LIU H, etal. Urban non-point pollution characteristics in china: a meta-analysis [J]. Ecology and Environmental Sciences, 2020, 29 (8): 1634-1644. (in Chinese)