Table S1. Bimodal and Multimodal NIRS Study Inventory

Configuration, synchronization, fusion architecture, and ablation results for combined-modality NIRS studies.

| Study ID | First Author (Year) | Modalities Combined | NIRS Configuration (channels, anatomical site) | Companion Modality Configuration | Synchronization Method | Fusion Level and Architecture | Ablation Result (multimodal vs unimodal) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S54 | Zimmermann R (2013) | fNIRS + biosignals (ECG-derived HR, blood pressure, respiratory flow, skin conductance response) | 8 light paths total (4 per probe at 2.0/2.5/3.5/4.0 cm separations); 2 wavelengths (692/834 nm); contralateral left primary motor cortex (M1, ~C3) and ventral premotor cortex (PMv, ~FC5) | g.USBamp biosignal amplifier at 600 Hz: bipolar ECG (2 channels), nasal thermistor respiratory flow, finger CNAP continuous blood pressure, finger skin conductance response | Hardware trigger signals from a Simulink-controlled NI USB-6008 DAQ sent to the fNIRS Auxiliary Input Module to mark task onsets/offsets; biosignals acquired through the same Simulink model on the host PC | Feature-level fusion within a dual hidden Markov model (HMM) framework; per-class HMMs (rest vs active) trained on multivariate observations combining fNIRS feature signals (linear-combination weights from M1 and PMv) with the four biosignal channels; full covariance Gaussian mixture emissions | fNIRS-only mean accuracy 79.4% vs combined fNIRS+biosignals 88.5% (p = 0.02); sensitivity 78.3% to 87.2% (p = 0.008); specificity 80.5% to 89.9% (p = 0.062) |
| S39 | Blokland Y (2014) | fNIRS + EEG | 2 fNIRS channels (one per hemisphere) using 2 transmitters (764/858 nm) and 1 receiver per channel with 35 mm target and 10 mm reference inter-optode distances for short-channel scalp regression; placed around C3 and C4 over sensorimotor cortex | 8-channel passive Porti EEG system (TMSi) at 2048 Hz; electrodes at C3, FC3, C5, CP3, C4, FC4, C6, CP4 (10/20 system) co-located with the fNIRS optodes on a custom integrated cap | Co-located EEG electrodes and fNIRS optodes on a single custom cap; offline alignment of trial-locked 3 s segments using shared task event timing | Decision-level (late) fusion via naive-Bayesian combination of calibrated per-modality logistic-regression classifiers; EEG classifier on alpha/beta power features (0–15 s window) plus three separate fNIRS classifiers (HbO, HHb, combined HbO+HHb) on 3–18 s windows; class probabilities summed in log-odds space | Patients with tetraplegia, attempted movement vs rest: EEG 73%, fNIRS 70%, combined 79%; imagined movement: EEG 63%, fNIRS 65%, combined 70%. Healthy controls, actual movement vs rest: EEG 84%, fNIRS 77%, combined 87%; imagined movement: EEG 77%, fNIRS 59%, combined 79%. Combined > EEG-only significant for all conditions except control motor imagery |
| S91 | Abtahi M (2020) | fNIRS + EEG + body kinematics (IMU-based motion capture for gross movements + WearUp instrumented glove for fine finger movements) | NIRScout system (NIRx) with 8 sources and 8 detectors yielding 18 channels (2 channels excluded due to source malfunction, 16 used); 2 wavelengths (760/850 nm); 7.81 Hz sampling; placed over motor cortex per international 10-20 system | g.USBAMP EEG (g.tec) at 256 Hz with 13 electrodes (FC3, FC4, C1, Cz, C2, C3, C4, CP1, CP2, CP3, CP4, P3, P4); YEI 3-Space non-optical motion capture with 17 IMU sensors at 50 Hz upsampled to 256 Hz (3 sensors used per task); WearUp glove with 2 woven flexible sensors at ~50 Hz | Custom Brain-Body Monitoring application programming interface (BBM API) built on Python wrappers (Threespace for IMUs, Lab Streaming Layer for fNIRS via NIRStar) to coordinate start/stop and timestamp streams; EEG synchronized via trigger output from g.tec amplifier; signals resampled and aligned to 256 Hz | Feature-level fusion: per-trial 1 s averaged features concatenated across modalities (16 fNIRS HbO channels + 39 EEG band-power features [theta, alpha, beta over 13 channels] + 1 kinematic feature = 56 features) into support vector machine classifiers; 12 SVM kernels (linear, polynomial, radial) compared | PD vs neurotypical classification with all-activities-fused data: fNIRS-only 81.23%, EEG-only 92.79%, fNIRS+EEG hybrid 92.27%, all-modalities fused 93.40% accuracy; per-activity classification ≥83% accuracy across all 8 motor tasks |
| S27 | AL-Quraishi MS (2021) | fNIRS + EEG | Hitachi OT-R40 multichannel system; 48 fNIRS channels (4×4 probe over each hemisphere) at 10 Hz; 2 wavelengths (695/830 nm); 3 cm inter-optode distance; bilateral sensorimotor cortex covering primary motor, premotor, and supplementary motor areas around Cz with foot-area representation | BrainMaster Discovery 24E EEG at 256 Hz with 19 electrodes (FCz, FC1–FC4, Cz, C1–C6, CPz, CP1–CP6) over bilateral motor cortex referenced to linked earlobes (A1–A2); 10/10 placement; impedance < 5 kΩ | Custom MATLAB code on the host PC sent simultaneous trigger signals to the EEG system (parallel port) and fNIRS system (serial port) at task start/end markers within each trial; integrated EEG-fNIRS cap (MCScap) co-localized electrodes and optodes | Feature-level fusion via channel-pair selection guided by Pearson correlation between alpha-band (8–11 Hz) ERD on EEG and oxy-hemoglobin on fNIRS; concatenated time-domain EEG features (MAV, RMS, waveform length, AR coefficients) with fNIRS statistical features (variance, kurtosis, skewness) on 1 s sliding windows; classified with a support vector machine | Right vs left ankle joint movement classification: fNIRS-alone 85.61%, EEG-alone 89.39%, fNIRS+EEG all channels 92.13%, fNIRS+EEG correlated channels only 93.01% accuracy (p = 0.0018 vs fNIRS, p = 0.0083 vs EEG) |
| S48 | Ortega P (2021) | fNIRS + EEG | NIRScout (NIRx) with 24 channels (12 per hemisphere from 10 sources and 8 detectors) at 12.5 Hz; 2 wavelengths (760/850 nm); ~3 cm inter-optode distance; bilateral sensorimotor cortex | ActiChamp EEG amplifier (BrainProducts) initially at 4 kHz, downsampled to 500 Hz; 24 EEG electrodes co-aligned with the fNIRS layout (one electrode placed between each fNIRS source-detector pair) over bilateral sensorimotor cortex; independent EOG recording for ICA-based ocular artifact removal | Co-located multimodal probe layout with EEG electrodes physically interleaved between fNIRS source-detector pairs on a single cap; downsampled to a common 12.5 Hz grid for the Hilbert-feature input stream and 80 Hz for the raw-EEG input stream | Attention-based deep learning fusion: a custom convolutional neural network with residual connections and self-attention layers (cnnatt) takes parallel fNIRS (HbO/HbR) and EEG (Hilbert envelope and instantaneous phase across delta, theta, alpha, beta, low-/mid-/high-gamma) inputs through modality-specific 1D convolutional layers, concatenates them via skip connections and a self-attention mechanism, and outputs continuous bilateral force; LSTM and mixture-density-network variants also evaluated | Continuous bimanual grip force trajectory reconstruction (FVAF[%], left/right hand): linear EEG-only 34.2/33.9, linear fNIRS-only 38.2/42.3, linear multimodal 46.9/49.9, cnnatt multimodal 54.2/55.6, cnnatt-LSTM multimodal 58.6/59.6; force-onset detection accuracy improved from 78.7/79.0% (linear multimodal) to 86.1/86.4% (cnnatt) and 87.0/87.1% (cnnatt-LSTM) |
| S30 | Jiang Y-C (2022) | fNIRS + EEG (with auxiliary EOG and neck sEMG used only for artifact rejection) | NIRScout continuous-wave system (NIRx) with 12 sources and 12 detectors yielding 37 channels at 5.21 Hz; 2 wavelengths (785/830 nm); ~3 cm inter-optode distance; covering bilateral premotor, primary motor (M1), and primary somatosensory (S1) cortices | g.Hlamp EEG amplifier (g.tec) at 1200 Hz with 36 electrodes (10–10 system, F7–F8, FT7–FT8, T7–T8, TP7–TP8 rows) over PMC/M1/S1 referenced at FCz; plus 4 EOG electrodes and 4 neck sEMG electrodes (bilateral trapezius and sternocleidomastoid) on the same g.Hlamp at 1200 Hz for artifact identification | Psychophysics Toolbox 3 (PTB-3) on a single visual-cue PC controlled task presentation and broadcast event triggers simultaneously to the EEG and fNIRS acquisition systems for hardware-level synchronization | Feature-level fusion using task-related component analysis (TRCA): per-modality TRCA spatial filters trained on each bimanual cyclical task; resulting low-dimensional task-similarity correlation indicators (3 EEG + 3 fNIRS per trial) concatenated and fed to LDA, SVM, and random forest classifiers | Three-class single-trial classification of bimanual cyclical tasks (in-phase / 90° out-of-phase / anti-phase) with 15 training trials and SVM: EEG-only 74.8%, fNIRS-only 82.2%, fused EEG+fNIRS 90.1% accuracy (one-way ANOVA across modalities p ≤ 0.005) |
| S49 | Syed AU (2023) | fNIRS + sEMG | NIRSport (NIRx) continuous-wave system at 7.81 Hz; 8 sources and 8 detectors yielding 20 channels; 3 cm inter-optode distance; placed on the motor cortex according to the 10–5 international system (between nasion-inion and pre-auricular reference points, Cz center); fOLD-toolbox-guided probe placement at 15% specificity threshold | Thalmic Labs Myo armband at 200 Hz: 8 dry sEMG electrodes plus a 9-axis IMU (3-axis gyroscope and accelerometer), worn around the bicep brachii (or residual limb of trans-humeral amputees); range −30 to +70 mV; z-score-based channel selection (48 mV threshold healthy, 16 mV threshold amputees) | Both modalities recorded in parallel during pre-cued tasks following a 30 s baseline, with 10 s task and 20 s rest periods controlled by the same paradigm script; LabVIEW used for sEMG storage and the NIRStar/nirsLAB pipeline for fNIRS, with synchronized session-level start markers | Feature-level fusion in a custom convolutional neural network: 150 ms windows on sEMG (peak, mean, wavelength) and 2 s windows on fNIRS (peak, minimum, mean ΔHbO and ΔHbR) feed three sequential 1-D convolutional blocks (kernel sizes 7, 5, 3; 128/128/64 filters with max-pooling), then global average pooling, two fully connected layers (512, 128) and an 8-way softmax; preprocessed by a product-of-peaks-and-power (PP) transform to stabilize non-stationary inputs; combined with weighted decision logic that switches reliance between modalities when sEMG quality drops | Eight-class upper-limb intention classification (EE, EF, WS, WP, WE, WF, HO, HC) on 20 healthy and 4 trans-humeral amputee participants: sEMG-only 91.81%, fNIRS-only 93.65%, fused sEMG+fNIRS 98% maximum and 94.5% average accuracy; hybrid scheme yielded ~6% absolute accuracy improvement over the best single modality, with paired t-test p = 0.0001 versus sEMG and fNIRS alone; real-time prosthetic-arm control validated with 0/30 to 1/30 misclassifications per motion in healthy and 0/15 to 1/10 in amputee subjects |
| S82 | Wu F (2025) | fNIRS + EEG | NIRSport continuous-wave system (NIRx) at 10.2 Hz; 12 sources and 12 detectors yielding 38 channels; 2 wavelengths (760/850 nm); 3 cm inter-optode distance; covering bilateral premotor cortex, primary motor cortex (M1) and primary somatosensory cortex (S1) | BrainAmp DC amplifier (Brain Products) at 1000 Hz; 10 EEG electrodes (F3, Fz, F4, FC1, FC2, C3, Cz, C4, CP1, CP2) over PMC/M1/S1 referenced to FCz with ground at Fpz; co-located with fNIRS optodes on a single cap arranged according to 10-20/10-10 systems | Custom Python script dispatched a synchronization trigger to both EEG and fNIRS systems concurrently with the onset of each trial; co-located cap with NIRSite-guided montage to align electrodes and optodes spatially; data downsampled to a common reference for cross-modal feature extraction | Feature-level fusion using task-related component analysis (TRCA): per-modality TRCA spatial filters trained on each gait task to extract reproducible task-locked components; band-specific (theta/alpha/beta) ERSP and ΔHbO time courses from each 5 s task segment summarized into 20-dimensional feature vectors per modality; concatenated EEG-TRCA + fNIRS-TRCA vectors classified by an XGBoost three-class model under leave-one-subject-out cross-validation; neurovascular coupling additionally quantified by HRF-convolved EEG cross-correlated with HbO over a ±6 s lag window | Three-class single-trial classification of automated vs discrete gait tasks (continuous walking / isolated gait phase / single-limb stance) on 18 healthy adults: EEG-Avg 49.02%, fNIRS-Avg 47.06%, EEG-Avg+fNIRS-Avg ~58%, EEG-TRCA ~69%, fNIRS-TRCA ~57%, fused EEG-TRCA+fNIRS-TRCA 74.51% accuracy (p = 0.042 vs EEG-Avg, p = 0.038 vs fNIRS-Avg); TRCA significantly increased within-class similarity and between-class discriminability for both modalities (p < 0.05) and revealed task-specific alpha-band coupling (stronger negative coupling in IGPT vs CW, p = 0.046) |
| S22 | He X (2026) | fNIRS + EEG | NirSmart commercial system (China) at 10 Hz; multi-source/detector montage forming a 14×14 reconstructed image after 2D scalp projection; covering bilateral pre-motor and supplementary motor area (BA6), primary motor cortex (BA4), and somatosensory association cortex (BA7); MNI coordinates obtained via Patriot 3D digitizer (Polhemus); HbO/HbR/HbT/HbD parameters computed via the modified Beer–Lambert law | g.tec 32-channel EEG system (Austria) at 250 Hz, co-mounted in a custom integrated cap with fNIRS optodes; Cz-centered placement at the intersection of sagittal and coronal planes; bipolar Ag–AgCl EMG electrodes on biceps brachii and triceps brachii sampled at 1000 Hz (NI DAQ-6341) for myoelectric-controlled interface (MCI) cursor control | Custom-written program output marker signals to synchronize EEG, EMG and fNIRS streams; LabVIEW handled task presentation, EMG processing and target trajectory generation while a synchronization program time-stamped events for the fNIRS-EEG system; one-second moving window with 0.125 s step on EEG features aligned with delayed fNIRS amplitudes | Feature-level (early) weighted integration model: Hilbert-transform-derived time-phase-frequency features (instantaneous amplitude, phase, frequency, power) extracted from EEG, normalized to [−1, 1] and channel-selected via Joint Mutual Information (JMI); per-time-window classification accuracies from an extreme learning machine (ELM) form a dynamic time-weight vector θ that modulates spatially aligned 14×14 fNIRS hemodynamic topographies (delays t = 0–4 s) to produce an fNIRS Guided Attention map (FGAmap); compared against constant-weight, neurovascular-coupling (HRF-convolved correlation) and PSD-band-power baselines | Within-class similarity for one-dimensional and two-dimensional MCI tracking tasks on 15 healthy participants: dynamic-weighted EEG-fNIRS 0.66 ± 0.17 (1D) and 0.62 ± 0.05 (2D), versus NVC (Beta-HbO) 0.58 ± 0.10 / 0.56 ± 0.08, PSD (beta band, EEG only) 0.38 ± 0.05 / 0.31 ± 0.09, and constant-weight (fNIRS only) 0.50 ± 0.09 / 0.53 ± 0.03; one-way ANOVA confirmed significant superiority of the dynamic weighted model (1D: p = 0.018, F = 8.38; 2D: p = 0.011, F = 10.46); only the dynamic model produced significant 1D-vs-2D differences in cortical activation (t[5,7] = 3.87, t[6,8] = 3.73; both p < 0.05) and in high-activation channel count (t = 4.44, p < 0.01) |
| S03 | Huang J (2026) | NIRS + sEMG | 60-channel NIRS module integrated within a custom hybrid sEMG-NIRS array (NMEP-HBO SyncACQ System) at 11 Hz; 18 emitter and 20 receiver probes with 30 mm emitter-to-receiver spacing; 21.25 cm × 12.75 cm flexible/elastic rubber probe belt covering lumbar erector spinae from L2 to L5; 2D HbO/HbR feature topographic maps (224 × 224 px) generated by cubic-spline interpolation; preprocessing in Homer3 with 0.01–0.2 Hz band-pass and targeted PCA motion-artifact correction | 60-channel surface electromyography on the same custom array at 2050 Hz, 24-bit precision, common-mode rejection >95 dB; 6×10 transparent, flexible, stretchable electrode array with 21.2 mm inter-electrode distance; sEMG electrodes interlaced (diamond-and-grid) with NIRS probes for spatial co-localization; reference/ground electrodes on the sacral vertebrae; 10–500 Hz band-pass + 50 Hz notch filtering and ICA-based ECG-artifact removal; MDF and RMS feature topographic maps generated per 1000 ms window | Hybrid sEMG-NIRS sensor array enables co-located, simultaneous acquisition with sEMG electrodes positioned at the midpoint of each NIRS source-detector pair; transparent stretchable electrodes allow NIRS light to pass through; both modalities recorded by the integrated NMEP-HBO SyncACQ System with shared session timing; 1000 ms windows align the two sampling rates and Borg RPE scores time-locked for label generation | Feature-level fusion via the dual-stream convolutional hybrid attention network (DCHANet): modality-specific streams with three sequential conv blocks (kernels 7/5/3, 32/64/128 channels), a parallel multi-scale conv block (max-pooling for sEMG, average-pooling for NIRS) and a Spatial-Channel Attention Module (SCAM, l = 3 for sEMG, l = 7 for NIRS); 384-channel feature maps from each stream are flattened, position- and modality-embedded, concatenated and passed through a hierarchical Joint Self-Attention Layer (JSAL) followed by sEMG and NIRS Multi-Head Cross-Attention layers, then fully connected (256, 128) and softmax (2 attention heads, L2 = 0.3, ~11.17 M parameters) | Lower-back muscle-fatigue classification on 11 healthy male adults (FAT-3 / FAT-15 datasets): subject-wise multimodal DCHANet 97.93% / 91.06% accuracy versus sEMG-only (FAT-3 ~80.86%, FAT-15 ~34.7%; both p < 0.001) and NIRS-only (FAT-3 ~95.86%, p = 0.035; FAT-15 ~89.30%, p = 0.007); cross-subject 96.80% / 88.53% with similar pattern; outperformed HOG-KNN/SVM/RF (FAT-15: +12.81%/+4.64%/+6.17%, all p < 0.001) and Deep Multimodal CNN (+1.9%, p = 0.013); ablation showed feature-level attention fusion superior to baseline (DCANet, −1.88% subject-wise, −2.16% cross-subject) and decision-level fusion (DCANet-DF, −1.12% / −1.23%) |
| S59 | Lin J (2026) | EEG + fNIRS | Nirsmart system (Danyang Huichuang) at 11 Hz; 17 sources and 16 detectors yielding 43 channels; 2 wavelengths (760/850 nm); 3 cm inter-optode distance; covering prefrontal lobe, motor cortex (sensorimotor cortex, supplementary motor area, premotor cortex) and occipital lobe; depth-weighted minimum norm estimate inverse model on a five-tissue MNI-ICBM152 head with Monte Carlo light-fluence simulation (1e8 photons) | Neuroscan SynAmps2 64-channel EEG at 1000 Hz arranged on the international 10-10 system, electrode impedance < 10 kΩ; sLORETA inverse modeling on a multi-layer BEM head model (OpenMEEG) with the same MNI-ICBM152 template and 15 000-vertex cortical mesh | EEG and fNIRS co-acquired with hardware-level synchronization via a trigger box that simultaneously timestamped the start of each trial on both systems; co-mounted sensors on a custom acquisition cap; both source-reconstructed signals projected onto the Brodmann atlas to enable alignment in a unified cortical source space (50 overlapping ROIs) | Feature-level fusion via an EEG-fNIRS multilayer brain network: intra-layer edges built from wPLI for EEG and Pearson correlation for fNIRS, binarized at 10–30% edge-density thresholds; inter-layer edges defined by subject-specific neurovascular-coupling strength estimated through least-squares deconvolution of a non-canonical HRF (15 s kernel, λ = 0.1) linking EEG beta-band time-frequency power to fNIRS HbO; multiplex network metrics (multiplex node degree, multirichness coefficient, multiplex clustering coefficient, multiplex participation coefficient) extracted on a 33×33 frontal-parietal sub-network and used as features in random-forest and SVR regression models | Subacute hemiplegic stroke recovery study (19 patients: 11 rTMS, 8 sham; 13 healthy controls): rTMS group showed significant improvement in ipsilateral-hemisphere NVC strength (PCC 0.249 → 0.355, p = 0.010) and hemispheric laterality (−0.109 → 0.050, p = 0.024), and significantly higher ΔPCC vs sham (p = 0.020); multilayer multiplex node degree (p = 0.021) and multiplex clustering coefficient improvements were significantly larger than sham (p = 0.006); ΔPCC and ΔMCC correlated with ΔFMA-UE (R = 0.600, p = 0.040; R = 0.618, p = 0.030); multilayer fusion reduced normalized mean absolute error in clinical-score prediction by 20.36% on average versus unimodal EEG and fNIRS baselines under leave-one-out cross-validation |

Note. NIRS Configuration captures channel count, source-detector geometry where reported, and anatomical site (e.g., bilateral sensorimotor cortex; lumbar erector spinae). Companion Modality Configuration captures the parallel field for the non-NIRS modality (e.g., 64-ch EEG at 1000 Hz; 60-ch sEMG at 2050 Hz). Fusion Level and Architecture follows the early / feature / decision / attention taxonomy used in Section 3.4.4 and names the concrete fusion architecture where applicable (e.g., feature-level concatenation; decision-level naive-Bayes combination; attention-based cnnatt; multilayer brain network). Ablation Result reports headline performance for the multimodal pipeline versus each unimodal baseline where reported; use NR if not reported. Sample size and population are provided in Table 1 and are not duplicated here. ECG, electrocardiography; EEG, electroencephalography; HRV, heart-rate variability; IMU, inertial measurement unit; LSL, Lab Streaming Layer; NR, not reported; sEMG, surface electromyography. Studies are listed chronologically by publication year.