Electronic Supplementary Material

Explainable AI-driven discovery of optimal modular architectures of CO2 reduction reactor clusters

Kaihao Fu1,3, Xinyuan Li2, Ping Li1, Wenze Guo2, Chenxi Cao ()2, Wangli He2, Wenli Du2, Feng Qian2

1 Key Laboratory of Green Chemical Engineering and Industrial Catalysis, School of Chemical Engineering, East China University of Science and Technology, Shanghai 200237, China

2 Key Laboratory of Smart Manufacturing in Energy Chemical Process, Ministry of Education, East China University of Science and Technology, Shanghai 200237, China

3 Suzhou Laboratory, Suzhou 215123, China

E-mail: caocx@ecust.edu.cn

S1 Dataset generation

S1.1 Random structure dataset.

As a stratified sampling technique, Latin Hypercube Sampling (LHS) divides each dimension of the input space into intervals of equal probability and then samples randomly within each interval, thereby achieving more uniform coverage of the input space with fewer samples. Given that the geometric dimensions are continuous variables with constraints, while module topology and connection schemes are discrete variables, the following sampling strategy was adopted: First, an excessive number of random geometric dimension combinations [H1, H2, H3, H4, H5] were sampled using LHS in the unconstrained geometric dimension space (55,000 samples), from which approximately 5,000 samples satisfying the geometric constraints were retained. Subsequently, discrete variables Nk and Gk were sampled using the LHS method. A total of 528 possible module topology were described using an index N, and the three types of inter-stack connection schemes were also represented by index (Z = 1, U = 2, S = 3), yielding approximately 5,000 combinations of module topology and inter-stack connection schemes [N, G3, G4, G5]. These geometric and topological structures were then randomly combined, with the topological index N being restored to [N1, N2, N3, N4, N5]. When Nk = 1, indicating the absence of a particular architectural hierarchy of the reactor module, Hk and Gk were set to 0. For instance, in a single-stack structure where N3 = 1, the inter-stack distribution channel H3 does not exist. Finally, the equivalent transport resistance network model was used to solve the performance features of the approximately 5,000 random structures, ultimately forming the random structure dataset. The frequency distribution histogram of the structural parameters is shown in Figure S1.

Figure S1 The frequency histogram of the random reactor module architecture dataset: (a) H1, (b) H2, (c) H3, (c) H4, (d) H5, (e) N, (g) G3, (h) G4, (i) G5.

S1.2 Pareto-optimal structure data set

Figure S2 Performance of Pareto-optimal reactor module designs.

Figure S3 Feature correlation matrix of Pareto-optimal designs.

S2 Machine learning Model details

S2.1 Calculation efficiency between different model

Figure S4 Comparison of computation time between machine learning model and equivalent transport resistance model: (a) Architecture [2, 2500, 1, 1]; (b) All samples

S2.2 Hyperparameters and performance of machine learning model

Table S1 Optimized hyperparameters with 5-fold cross-validation.

| Model | Parameter | Value | Metrics |
| --- | --- | --- | --- |
| XGB regression model for reaction performance | max_depth | 4 | R2train = 0.99; R2test = 0.90 |
|  | n_estimators | 250 |  |
|  | learning_rate | 0.2 |  |
| XGB regression model for cost | max_depth | 6 | R2train = 0.99; R2test = 0.99 |
|  | n_estimators | 250 |  |
|  | learning_rate | 0.05 |  |
| XGB regression model for carbon footprint | max_depth | 6 | R2train = 0.99; R2test = 0.90 |
|  | n_estimators | 200 |  |
|  | learning_rate | 0.2 |  |
| XGB classification model | max_depth | 4 | F1,train = 1.00; F1,test = 0.97 |
|  | n_estimators | 30 |  |
|  | learning_rate | 0.7 |  |

Table S2 Model error analysis

| Objective | Train set |  |  | Test set |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MAE | RMSE | MAPE | MAE | RMSE | MAPE |
| Ct | 3274.39 CNY/a | 4440.25 CNY/a | 2.16% | 7005.55 CNY/a | 10080.16 CNY/a | 4.07% |
| X | 0.25% | 0.41% | 0.40% | 0.61% | 1.43% | 1.05% |
| mCO2,net | 0.14t/a | 0.20 t/a | 0.75% | 1.07 t/a | 2.31 t/a | 6.79% |

Fig. S5 Regression performance of different models

Table S3 Performance of different models

| Model | Train |  |  |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R2 | RMSE | MAE | MAPE(%) | R2 | RMSE | MAE | MAPE(%) |
| XGBoost | 0.9916 | 0.4101 | 0.2482 | 0.4001 | 0.8955 | 1.426 | 0.6063 | 1.045 |
| Linear Regression | 0.3854 | 3.508 | 2.021 | 3.514 | 0.4393 | 3.304 | 1.995 | 3.369 |
| SVM | 0.7836 | 2.081 | 0.6715 | 1.294 | 0.7361 | 2.267 | 0.8065 | 1.489 |
| Random Forest | 0.9499 | 1.001 | 0.5269 | 0.8991 | 0.7794 | 2.072 | 0.91923 | 1.621 |

S3 Cost Model details

S3.1 Cost of equipment investment

The cost of equipment includes the cost of microreactor materials CC, the cost of catalyst Ccat, and the cost of insulation materials Cin:

(S1)

The cost of microreactor materials can be calculated as:

(S2)

where CB is the base cost of the same kind of equipment, QC and QB are the design capacity of the equipment and base equipment, respectively. M is a cost exponent of equipment, fM, fP, fT are correction factors of material, pressure, and temperature, respectively. For reactors, the design capacity is the reactor volume or mass; for heat exchangers, the design capacity is the area of heat transfer. QC is calculated by:

(S3)

where ρm is the material density, Vmc is the material consumption. The parameters used in this work and the base cost and capacity of microreactor are shown in Table S18.

Table S4 Parameters used for reactor cost calculations

| CB | QB | M | fM | fP | fT |
| --- | --- | --- | --- | --- | --- |
| 2142 CNY | 1.6 kg | 0.82 | 3.4 | 1 | 2.1 |

The cost of insulation can be calculated by:

(S4)

where δ is the thickness of insulation layer, S is the heat dissipation area, and Pi is the price of insulation material per volume. The calculation of δ is based on GB/T8175, guide for design of thermal insulation of equipment and pipes:

(S5)

where A1 is a constant, A1 = 1.8975×10-3; fn is the heating price, fn = 4 CNY; t is the annual operating hours, t = 8000 h; Ta is the operation temperature, which can be taken as qualitative temperature; S is the annual apportionment rate of the loan for insulation project investment, taken as 0.2638, α surface heat dissipation coefficient is generally taken as 11.63 W/(m2·K); λ is the thermal conductivity of insulation material, Pi is its unit volume cost, aluminum silicate insulation cotton was selected, thermal conductivity of 0.035 W/(m·K), unit cost of 260 CNY.

The cost of Ni/Al12O19 catalyst is calculated according to the following formula, and the unit catalyst price pcat is 1000 CNY/kg. For 2500 channels with about 30 g catalyst, the price is about:

(S6)

S3.2 Cost of reactor operation

The operation cost mainly comes from the energy consumption Cop (CNY/a) generated in fluid transportation, and the annual operation cost can be calculated as：

(S7)

where t is the annual operation time, taking 8000 h and p as the unit price, the industrial electricity price can take 0.5 CNY/kWh. Pa is the mechanical power of fluid transport installation, which can be calculated by:

(S8)

(S9)

(S10)

where η is the pumping efficiency, taken as 0.6; Pe is the pump power; g is gravitational acceleration; ρ and qv are the density and volume flow rate of fluid; z2 - z1 is the height difference of the fluid, which is ignored in this work; P2 - P1 is the pressure drop; u2 and u1 are the velocities before and after the fluid enters the conveying equipment.

S3.3 Geometry of individual microreactor stacks

Table S5 Structural diagram of individual microreactor stacks

| Single stack structure | Vertical view | Side view |
| --- | --- | --- |

Table S6 Detailed parameters of individual microreactor stacks

| Description | Symbol | Expression |
| --- | --- | --- |
| Length of 1st-level distributor | L1 | L1 = 2H0 |
| Length of 2nd-level distributor | L2 | L2 = 2H1 |
| Length of stack | La | La = N1L1 + H0 + 2H2 + 2ε1 |
| Width of stack | Lb | Lb = L0 + 2H1 + 2ε1 |
| Thickness of stack | Lc | Lc = N2L2 + H1 |
| Heat dissipation area | Sstack | Sstack = 2(LaLb + LaLc + LbLc) |
| Volume of reaction channel | Vch,0 | Vch,0 = H02L0 |
| Channel volume of 1st-level | Vch,1 | Vch,1 = N1(Vch,0 + 2H12L1) |
| Channel volume | Vch,stack | Vch,stack = Nk(Vch,1 + 2πHk2Lk/4) |
| Envelop volume | Ven,stack | Ven,stack = LaLbLc |
| Material consumption volume | Vmc,stack | Vmc,stack = Ven,stack – Vch,stack |
| Necessary margin on the plate | ε1 | ε1 = 0.02 m |

S3.4 Geometry of microreactor stack arrays

Table S7 Structural diagram of microreactor stack arrays

| Structure | Diagram |  |
| --- | --- | --- |
| Overall schematic structure (5th-level) | 3D view |  |
| Z-type parallel connection | Vertical view | Side view |
| U-type parallel connection | Vertical view | Side view |
| Series connection | Vertical view | Side view |

Table S8 Detailed parameters of microreactor stack arrays

| Description | Symbol | Expression |
| --- | --- | --- |
| Z-type parallel connection structure |  |  |
| Length of 3rd-level distributor | L3 | L3 = min (La + ε2, Lb + ε2) |
| Length of 4th-level distributor | L4 | L4 = max (La + ε2, Lb + ε2) |
| Length of 5th-level distributor | L5 | L5 = Lc + 2H4 + ε2 |
| Length of stacks | Ld | Ld = N3L3 + ε2 |
| Width of stacks | Le | Le = N4L4 + ε2 |
| Thickness of stacks | Lf | Lf = N5L5 + ε2 |
| Channel volume | Vch,k | Vch,k = Nk(Vch,k-1 + 2πHk2Lk/4) |
| Connection tube material consumption | Vtube,k | Vtube,k = 2LkNkπ(Hk,out2 − Hk2)/4 |
| U-type parallel connection structure |  |  |
| Length of 3rd-level distributor | L3 | L3 = min (La + ε2, Lb + ε2) |
| Length of 4th-level distributor | L4 | L4 = max (La + ε2, Lb + ε2) |
| Length of 5th-level distributor | L5 | L5 = Lc + 2H4 + ε2 |
| Length of stacks | Ld | Ld = N3L3 |
| Width of stacks | Le | Le = N4L4 |
| Thickness of stacks | Lf | Lf = N5L5 |
| Channel volume | Vch,k | Vch,k = Nk(Vch,k-1 + 2πHk2Lk/4) |
| Connection tube material consumption | Vtube,k | Vtube,k = 2LkNkπ(Hk,out2 − Hk2)/4 |
| Series connection structure |  |  |
| Length of 3rd-level distributor | L3 | L3 = ε2 + 2ε1 |
| Length of 4th-level distributor | L4 | L4 = ε2 + 2ε1 |
| Length of 5th-level distributor | L5 | L5 = 2H4 + ε2 |
| Length of stacks | Ld | Ld = min (La + ε2, Lb + ε2) N3 + ε2 |
| Width of stacks | Le | Le = max (La + ε2, Lb + ε2) N4 + ε2 |
| Thickness of stacks | Lf | Lf = (Lc + 2H4 + ε2) N5 + ε2 |
| Channel volume | Vch,k | Vch,k = Nk(Vch,k-1 + πHk2Lk/4) |
| Connection tube material consumption | Vtube,k | Vtube,k = LkNkπ(Hk,out2 − Hk2)/4 |
| All structure |  |  |
| Heat dissipation area | S | S = N3N4N5Sstack |
| Envelop volume | Ven | Ven = LdLeLf |
| Material consumption volume | Vmc | Vmc = Vmc,tube + N3N4N5Vmc,stack |
| Margin between stacks | ε2 | ε2 = 0.25 m |
| Wall thickness of connection tubes | dtube | dtube = 0.002 m |

S4 Supplementary results

Table S9 Optimal architectural patterns for Nt = 2500 and σ = 0.

| Clustering result | Performance features for different module architectures |  |  |
| --- | --- | --- | --- |
| Nt = 2500, σ= 0 |  |  |  |
|  | Small-sized single-stack reactors | Small-sized few-stack parallel modules | Large-sized single-stack reactors |

Table S10 Optimal architectural patterns for Nt = 5000 and σ = 0.

| Clustering result | Performance features for different module architectures |  |  |
| --- | --- | --- | --- |
| Nt = 5000, σ = 0 |  |  |  |
|  | Small-sized single-stack reactors | Small-sized few-stack parallel modules |  |
|  | Large-sized single-stack reactors | Large-sized few-stack parallel modules |  |

Table S11 Optimal architectural patterns for Nt = 5000 and σ = 0.48

| Clustering result | Performance features for different module architectures |  |  |
| --- | --- | --- | --- |
| Nt = 5000, σ= 0.48 |  |  |  |
|  | Small-sized single-stack reactors | Small-sized few-stack parallel modules | Small-sized multi-stack parallel modules |
|  | Large-sized single-stack reactors | Large-sized few-stack parallel modules | Large-sized multi-stack parallel modules |