Electronic Supplementary Material

Optimization of kinetic mechanism for hydrogen combustion based on machine learning

Shuangshuang Cao1, Houjun Zhang1, Haoyang Liu3, Zhiyuan Lyu3, Xiangyuan Li2, Bin Zhang ()3, You Han ()1

1 School of Chemical Engineering and Technology, Tianjin University, Tianjin 300072, China

2 School of Chemical Engineering, Sichuan University, Chengdu 610065, China

3 School of Aeronautics and Astronautics, Shanghai Jiao Tong University, Shanghai 200240, China

E-mails: zhangbin1983@sjtu.edu.cn (Zhang B); yhan@tju.edu.cn (Han Y)

Fig. S1. The prediction performance of the Optimized mechanism for different weight coefficient of IDTs and LFSs for 5:5, 6:4, 7:3

In the process of optimizing mechanism for H2 combustion, the weight coefficients of ignition delay time (IDTs) and laminar flame speed (LFSs) in the objective function are very crucial. We hope that the optimized mechanism will show good prediction performance in both IDTs and LFSs. If the weight coefficient of a physical quantity is too high, the prediction performance of the other physical quantity will become worse. Considering the number of experimental data and the magnitude of error of IDTs and LFSs, several weight coefficient sets of IDTs and LFSs of 5:5,6:4 and 7:3 were selected. Five indexes of the total mean error of IDTs and LFSs, the mean and maximum error of IDTs, and the mean and maximum error of LFSs were selected to determine the weight coefficient used in the objective function. Fig. S1 shows the performance of the Optimized mechanism obtained using different weight coefficients under 10000 sample data points. As shown in Fig. S1 , the Optimized mechanism obtained by the weight coefficient of 6:4 has minimum values in the total mean error, the mean error of LFSs, and the maximum error of IDTs and LFSs.Therefore, the weight coefficient of 6:4 is the most appropriate weight coefficient in the optimization process.

Fig. S2. Relative error for mechanism during iteration.

As illustrated in Fig. S2, the objective function, prediction mean error of IDTs and LFSs present the fluctuating downward trend with iterations in the first 20 iterations. The values of the objective function have changed very little after 40 iterations. It can be seen that the terminal condition is satisfied at the 79th iteration.

Table S1. Ignition delay time measurements of hydrogen-oxygen mixtures

| Number of data | Diluents | Temperature (K) | Pressure (atm) | Equivalence ratio |
| --- | --- | --- | --- | --- |
| Herzler et al. (2009) [1] |  |  |  |  |
| 30 | AR | 956-1456 | 0.87-4.12 | 0.5-1 |
| Zhang et al. (2012) [2] |  |  |  |  |
| 11 | AR | 1065-1267 | 10-20 | 0.5 |
| Kéromnès et al. (2013) [3] |  |  |  |  |
| 102 | AR | 966-1739 | 1.43-34.67 | 0.3-1 |
| 21 | N2 | 974-1954 | 0.96-16 | 0.5 |
| Pang et al. (2009) [4] |  |  |  |  |
| 15 | AR | 963-1118 | 3-3.7 | 0.42-1 |
| Pavlov et al. (2011) [5] |  |  |  |  |
| 15 | AR | 963-1902 | 0.64-1.34 | 1 |
| Pan et al. (2014) [6] |  |  |  |  |
| 28 | AR | 915-1457 | 1.2-16.7 | 1 |
| Tang et al. (2013) [7] |  |  |  |  |
| 34 | AR | 930-1465 | 1.12-10.67 | 0.5-2 |
| Pan et al. (2014) [8] |  |  |  |  |
| 24 | AR | 937-1313 | 1.19-10.54 | 0.5-2 |
| Petersen et al. (1996) [9] |  |  |  |  |
| 64 | AR | 1189-1930 | 33-87 | 1 |
| Cheng et al. (1984) [10] |  |  |  |  |
| 105 | AR | 1004-1427 | 1.35-2.9 | 0.5;1 |
| Cohen et al. (1967) [11] |  |  |  |  |
| 20 | AR | 941-1583 | 0.25-1.44 | 1 |
| Skinner et al. (2004) [12] |  |  |  |  |
| 3 | AR | 1004-1075 | 5 | 2 |
| Schott et al. (2004) [13] |  |  |  |  |
| 17 | AR | 1086-1836 | 0.74-2 | 0.25 |
| Slack et al. (1977) [14] |  |  |  |  |
| 15 | N2 | 984-1184 | 2 | 1 |
| Petersen et al. (2004) [15] |  |  |  |  |
| 28 | AR | 1009-1511 | 1 | 1-1.5 |
| Bhaskaran et al. (1973) [16] |  |  |  |  |
| 13 | N2 | 1067-1323 | 2.5 | 1 |
| Wang et al. (2003) [17] |  |  |  |  |
| 10 | N2 | 1019-1173 | 4 | 0.42 |
| 37 | N2/H2O | 1080-1252 | 4-16 | 0.42 |
| Asaba et al. (1965) [18] |  |  |  |  |
| 15 | AR | 1602-2554 | 0.27-0.46 | 0.5 |
| Naumann et al. (2011) [19] |  |  |  |  |
| 105 | AR | 911-2136 | 0.68-17.57 | 0.1-4 |
| Pan et al. (2014) [20] |  |  |  |  |
| 28 | AR | 921.3-1465 | 1.19-4.25 | 0.5-2 |
| Drakon et al. (2017) [21] |  |  |  |  |
| 18 | AR | 960-1020 | 1.93-2.64 | 1 |

Table S2. Laminar flame speed measurements of hydrogen-oxygen mixtures

| Number of data | Diluents | Temperature (K) | Pressure (atm) | Equivalence ratio |
| --- | --- | --- | --- | --- |
| Kwon et al. (2001) [22] |  |  |  |  |
| 8 | N2 | 298 | 1 | 0.6-4.5 |
| 13 | HE | 298 | 0.5-3 | 0.6-4.5 |
| Tse et al. (2000) [23] |  |  |  |  |
| 16 | N2 | 298 | 1 | 0.45-4.02 |
| 39 | HE | 298 | 3-20 | 0.5-3.5 |
| Bradley et al. (2007) [24] |  |  |  |  |
| 22 | N2 | 365 | 1-10 | 1 |
| Burke et al. (2011) [25] |  |  |  |  |
| 27 | HE | 295 | 1-25 | 0.3-0.7 |
| Koroll et al. (1993) [26] |  |  |  |  |
| 14 | N2 | 298 | 1 | 1 |
| Taylor et al. (1991) [27] |  |  |  |  |
| 21 | N2 | 296 | 1 | 0.22-4.99 |
| Takahashi et al. (1983) [28] |  |  |  |  |
| 9 | N2 | 298 | 1 | 0.93-4.38 |
| Vagelopoulos et al. (1994) [29] |  |  |  |  |
| 6 | N2 | 298 | 1 | 0.3-0.55 |
| Egolfopoulos et al. (1991) [30] |  |  |  |  |
| 56 | N2 | 298 | 1 | 0.15-2.2 |
| Hermanns et al. (2007) [31] |  |  |  |  |
| 27 | N2 | 298 | 1 | 0.8-3.3 |
| Lamoureux et al. (2003) [32] |  |  |  |  |
| 15 | N2 | 298 | 1 | 0.26-3.57 |
| Aung et al. (1998) [33] |  |  |  |  |
| 59 | N2 | 298 | 0.5-4 | 0.45-4 |
| Burke et al. (2010) [34] |  |  |  |  |
| 19 | AR | 295 | 1-25 | 2.5 |
| 26 | HE | 295 | 1-25 | 0.85-1 |
| Tang et al. (2006) [35] |  |  |  |  |
| 3 | N2 | 298 | 1 | 0.8-1.6 |
| Qiao et al. (2007) [36] |  |  |  |  |
| 20 | N2 | 298 | 1 | 1 |
| 21 | N2/AR | 298 | 1 | 1 |
| 7 | N2/HE | 298 | 1 | 1 |
| Dong et al. (2009) [37] |  |  |  |  |
| 18 | N2 | 293 | 1 | 1 |
| Santner et al. (2013) [38] |  |  |  |  |
| 11 | HE | 393 | 1-10 | 0.85 |
| 3 | HE/H2O | 393 | 1-10 | 0.85 |
| Lyu et al. (2017) [39] |  |  |  |  |
| 2 | N2 | 373 | 3 | 1 |
| 2 | AR | 373 | 3 | 1 |
| 41 | HE | 373 | 1-3 | 0.6-4 |
| Qin et al. (2000) [40] |  |  |  |  |
| 11 | N2 | 298 | 1 | 0.6-3 |
| Huang et al. (2006) [41] |  |  |  |  |
| 10 | N2 | 300 | 1 | 0.6-1.5 |
| Tang et al. (2008) [42] |  |  |  |  |
| 4 | N2 | 298 | 1 | 0.8-1.6 |

Fig. S3. Comparison of reaction rate constants of different mechanisms and experimental data for

R1-high pressure (a), R1-low pressure (b), R5 (c), R7 (d), R8-high pressure (e), R8-low pressure (f) and R9 (g).

Fig. S4. Species concentrations simulations using the Origin, Optimized, Kéromnès and NUIG mechanisms for experimental data for Case 1 (a), Case 2 (b), Case 3 (c) Case 4 (d), Case 5 (e) and Case 6 (f).

References

1.  Herzler J, Naumann C. Shock-tube study of the ignition of methane/ethane/hydrogen mixtures with hydrogen contents from 0% to 100% at different pressures. Proceedings of the Combustion Institute, 2009, 32(1): 213-220

2.  Zhang Y, Huang Z, Wei L, Zhang J, Law C K. Experimental and modeling study on ignition delays of lean mixtures of methane, hydrogen, oxygen, and argon at elevated pressures. Combustion and Flame, 2012, 159(3): 918-931

3.  Kéromnès A, Metcalfe W K, Heufer K A, Donohoe N, Das A K, Sung C J, Herzler J, Naumann C, Griebel P, Mathieu O et al. An experimental and detailed chemical kinetic modeling study of hydrogen and syngas mixture oxidation at elevated pressures. Combustion and Flame, 2013, 160(6): 995-1011

4.  Pang G A, Davidson D F, Hanson R K. Experimental study and modeling of shock tube ignition delay times for hydrogen–oxygen–argon mixtures at low temperatures. Proceedings of the Combustion Institute, 2009, 32(1): 181-188

5.  Pavlov V A, Shatalov O P. Measuring the ignition delay time for hydrogen-oxygen mixtures behind the front of an incident shock wave. Kinetics and Catalysis, 2011, 52(2): 157-165

6.  Pan L, Zhang Y, Zhang J, Tian Z, Huang Z. Shock tube and kinetic study of C2H6/H2/O2/Ar mixtures at elevated pressures. International Journal of Hydrogen Energy, 2014, 39(11): 6024-6033

7.  Tang C, Man X, Wei L, Pan L, Huang Z. Further study on the ignition delay times of propane-hydrogen-oxygen-argon mixtures: effect of equivalence ratio. Combustion and Flame, 2013, 160(11): 2283-2290

8.  Pan L, Hu E, Zhang J, Zhang Z, Huang Z. Experimental and kinetic study on ignition delay times of DME/H2/O2/Ar mixtures. Combustion and Flame, 2014, 161(3): 735-747

9.  Petersen E L, Röhrig M, Davidson D F, Hanson R K, Bowman C T. High-pressure methane oxidation behind reflected shock waves. Symposium (International) on Combustion, 1996, 26(1): 799-806

10. Cheng R K, Oppenheim A K. Autoignition in methanehydrogen mixtures. Combustion and Flame, 1984, 58(2): 125-139

11.  Cohen A, Larsen J. Explosive mechanism of the H2-O2 reaction near the second ignition limit. Technical report BRL 1386. 1967

12. Skinner G B, Ringrose G H. Ignition delays of a hydrogen-oxygen-argon mixture at relatively low temperatures. The Journal of Chemical Physics, 2004, 42(6): 2190-2192

13. Schott G L, Kinsey J L. Kinetic studies of hydroxyl radicals in shock waves. II. induction times in the hydrogen‐oxygen reaction. The Journal of Chemical Physics, 2004, 29(5): 1177-1182

14. Slack M W. Rate coefficient for H + O2 + M = HO2 + M evaluated from shock tube measurements of induction times. Combustion and Flame, 1977, 28: 241-249

15. Petersen E L, Kalitan D M, Rickard M J A. Reflected shock ignition of SiH4/H2/O2/Ar and SiH4/CH4/O2/Ar mixtures. Journal of Propulsion and Power, 2004, 20(4): 665-674

16. Bhaskaran K A, Gupta M C, Just T. Shock tube study of the effect of unsymmetric dimethyl hydrazine on the ignition characteristics of hydrogen-air mixtures. Combustion and Flame, 1973, 21(1): 45-48

17. Wang B L, Olivier H, Grönig H. Ignition of shock-heated H2-air-steam mixtures. Combustion and Flame, 2003, 133(1): 93-106

18. Asaba T, Gardiner W C, Stubbeman R F. Shock-tube study of the hydrogen-oxygen reaction. Symposium (International) on Combustion, 1965, 10(1): 295-302

19. Naumann C, Herzler J, Griebel P, Curran H J, Kéromnès A. Results of ignition delay times for hydrogen-rich and syngas fuel mixtures measured. 7th Framework Programme FP7-Energy-2008-Tren-1 Energy-2008-6-Clean Coal Technologies. 2011

20. Pan L, Hu E, Deng F, Zhang Z, Huang Z. Effect of pressure and equivalence ratio on the ignition characteristics of dimethyl ether-hydrogen mixtures. International Journal of Hydrogen Energy, 2014, 39(33): 19212-19223

21. Drakon A, Eremin A, Matveeva N, Mikheyeva E. The opposite influences of flame suppressants on the ignition of combustible mixtures behind shock waves. Combustion and Flame, 2017, 176: 592-598

22. Kwon O C, Faeth G M. Flame/stretch interactions of premixed hydrogen-fueled flames: measurements and predictions. Combustion and Flame, 2001, 124(4): 590-610

23. Tse S D, Zhu D L, Law C K. Morphology and burning rates of expanding spherical flames in H2/O2/inert mixtures up to 60 atmospheres. Proceedings of the Combustion Institute, 2000, 28(2): 1793-1800

24. Bradley D, Lawes M, Liu K, Verhelst S, Woolley R. Laminar burning velocities of lean hydrogen-air mixtures at pressures up to 1.0 MPa. Combustion and Flame, 2007, 149(1): 162-172

25. Burke M P, Dryer F L, Ju Y. Assessment of kinetic modeling for lean H2/CH4/O2/diluent flames at high pressures. Proceedings of the Combustion Institute, 2011, 33(1): 905-912

26. Koroll G W, Kumar R K, Bowles E M. Burning velocities of hydrogen-air mixtures. Combustion and Flame, 1993, 94(3): 330-340

27. Taylor SC. Burning velocity and the influence of flame stretch. Dissertation for the Doctoral Degree. Leeds: University of Leeds, 1991, 137

28. Takahashi F, Mizomoto M, Ikai S. Alternative energy sources III. New York: Hemisphere Publishing, 1983, 5: 447-57

29. Vagelopoulos C M, Egolfopoulos F N, Law C K. Further considerations on the determination of laminar flame speeds with the counterflow twin-flame technique. Symposium (International) on Combustion, 1994, 25(1): 1341-1347

30. Egolfopoulos F N, Law C K. An experimental and computational study of the burning rates of ultra-lean to moderately-rich H2/O2/N2 laminar flames with pressure variations. Symposium (International) on Combustion, 1991, 23(1): 333-340

31. Hermanns R T E, Konnov A A, Bastiaans R J M, de Goey L P H. Laminar burning velocities of diluted hydrogen-oxygen-nitrogen mixtures. Energy & Fuels, 2007, 21(4): 1977-1981

32. Lamoureux N, Djebaı̈li-Chaumeix N, Paillard C E. Laminar flame velocity determination for H2-air-He-CO2 mixtures using the spherical bomb method. Experimental Thermal and Fluid Science, 2003, 27(4): 385-393

33. Aung K T, Hassan M I, Faeth G M. Effects of pressure and nitrogen dilution on flame/stretch interactions of laminar premixed H2/O2/N2 flames. Combustion and Flame, 1998, 112(1): 1-15

34. Burke M P, Chaos M, Dryer F L, Ju Y. Negative pressure dependence of mass burning rates of H2/CO/O2/diluent flames at low flame temperatures. Combustion and Flame, 2010, 157(4): 618-631

35. Tang W, Brezinsky K. Chemical kinetic simulations behind reflected shock waves. International Journal of Chemical Kinetics, 2006, 38(2): 75-97

36. Qiao L, Gu Y, Dahm W J A, Oran E S, Faeth G M. A study of the effects of diluents on near-limit H2-air flames in microgravity at normal and reduced pressures. Combustion and Flame, 2007, 151(1): 196-208

37. Dong C, Zhou Q, Zhao Q, Zhang Y, Xu T, Hui S. Experimental study on the laminar flame speed of hydrogen/carbon monoxide/air mixtures. Fuel, 2009, 88(10): 1858-1863

38. Santner J, Dryer F L, Ju Y. The effects of water dilution on hydrogen, syngas, and ethylene flames at elevated pressure. Proceedings of the Combustion Institute, 2013, 34(1): 719-726

39. Lyu X, Hu E, Peng C, Meng X, Zuohua H. Measurements on laminar burning velocities of hydrogen/oxygen/diluents at elevated pressure and temperature. Journal of Aerospace Power, 2017, 32(7): 1599-1604 (in Chinese)

40. Qin X, Kobayashi H, Niioka T. Laminar burning velocity of hydrogen-air premixed flames at elevated pressure. Experimental Thermal and Fluid Science, 2000, 21(1): 58-63

41. Huang Z, Zhang Y, Zeng K, Liu B, Wang Q, Jiang D. Measurements of laminar burning velocities for natural gas-hydrogen-air mixtures. Combustion and Flame, 2006, 146(1): 302-311

42. Tang C, Huang Z, Jin C, He J, Wang J, Wang X, Miao H. Laminar burning velocities and combustion characteristics of propane-hydrogen-air premixed flames. International Journal of Hydrogen Energy, 2008, 33(18): 4906-4914