Electronic Supplementary Material

Extractive distillation of cycloalkane monomers from the direct coal liquefaction fraction

Shuo-Shuo Zhang1,2, Xing-Bao Wang ()1,2, Wen-Ying Li ()1

1 State Key Laboratory of Clean and Efficient Coal Utilization, Taiyuan University of Technology, Taiyuan 030024, China

2 College of Chemical Engineering and Technology, Taiyuan University of Technology, Taiyuan 030024, China

E-mails: wangxingbao@tyut.edu.cn (Wang X-B); ying@tyut.edu.cn (Li W-Y)

Supporting Information:

Fig. S1  Flowsheet of distillation column optimization by sequential iteration method

Fig. S2  Deviation range of predicted values of QSPR model

Table S1  Main components and contents in direct coal liquefaction naphtha

Table S2  Pearson correlation coefficients among molecular descriptors

Table S3  The necessary formulas and data for economic evaluation

Table S4  Results of total process optimization parameters

Table S5 Experimental values, predicted values and molecular descriptors of RVn-heptane/MCH in naphtha under different extractants (training set)

Table S6  Experimental values, predicted values and molecular descriptors of RVn-heptane/MCH in naphtha under different extractants (validation set)

Table S7  The energy consumption, economic performance and environmental protection analysis of the different extractive distillation separation processes

Table S8  Economic evaluation of salicylic acid as extractant

Fig. S1  Flowsheet of distillation column optimization by sequential iteration method

Fig. S2  Deviation range of predicted values of QSPR model.

| Table S1 Main components and contents in direct coal liquefaction naphtha |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| n | Molecular formula | Molecular structural formula | Relative mlecular mass | Compound Name | Relative content/wt.% | Boiling point/℃ |
| 1 | C5H12 |  | 72 | 2-Methylbutane | 2.00 | 30 |
| 2 | C5H12 |  | 72 | n-Pentane | 3.96 | 36 |
| 3 | C5H10 |  | 70 | Cyclopentane | 3.17 | 49.2 |
| 4 | C6H14 |  | 86 | n-Hexane | 4.73 | 68.5 |
| 5 | C6H12 |  | 84 | Methylcyclopentane | 5.01 | 71.8 |
| 6 | C6H12 |  | 84 | Cyclohexane | 16.47 | 80.7 |
| 7 | C7H16 |  | 100 | 3-Methylhexane | 1.11 | 90.7 |
| 8 | C7H14 |  | 98 | Dimethylcyclopentane | 3.16 | 97.2 |
| 9 | C7H16 |  | 100 | n-Heptane | 2.77 | 98.4 |
| 10 | C7H14 |  | 98 | Methylcyclohexane | 22.40 | 101.1 |
| 11 | C7H14 |  | 98 | Ethylcyclopentane | 1.70 | 103.5 |
| 12 | C8H16 |  | 112 | Trimethylcyclopentane | 1.31 | 143.5 |
| 13 | C8H18 |  | 114 | 2-Methylheptane | 0.43 | 117.8 |
| 14 | C8H16 |  | 112 | Dimethylcyclohexane | 12.09 | 125.9 |
| 15 | C8H16 |  | 112 | 1-Ethyl-1-methylcyclopentane | 7.75 | 150.8 |
| 16 | C8H16 |  | 112 | Isopropylcyclopentane | 0.16 | 125.2 |
| 17 | C8H16 |  | 112 | Ethylcyclohexane | 7.63 | 129.4 |
| 18 | C9H18 |  | 126 | Trimethylcyclohexane | 0.38 | 143.5 |
| 19 | C10H18 |  | 138 | Decahydronaphthalene | 0.34 | 187 |
| 20 | C8H10 |  | 106 | p-xylene | 0.16 | 138 |
| 21 | C9H20 |  | 128 | Nonane | 1.50 | 151.7 |
| 22 | C9H16 |  | 124 | hexahydroindene | 1.20 | 161 |
| 23 | C9H18 |  | 126 | Isopropyl cyclohexane | 0.33 | 155 |
| 24 | C9H18 |  | 126 | Butylcyclopentane | 0.21 | 156.1 |

| Table S2 Pearson correlation coefficients among molecular descriptors |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | α/a.u. | ELUMO/eV | EHOMO/eV | V/Angstrom3 | ρ/g·cm−3 | logPoct/water/kg·mol−1 | SASA/Angstrom2 | AF/Angstrom2 |
| α/a.u. | 1 |  |  |  |  |  |  |  |
| ELUMO/eV | 0.006 | 1 |  |  |  |  |  |  |
| EHOMO/eV | 0.174 | −0.112 | 1 |  |  |  |  |  |
| V/Angstrom3 | 0.943 | 0.094 | 0.039 | 1 |  |  |  |  |
| ρ/g·cm−3 | −0.16 | −0.221 | −0.119 | −0.246 | 1 |  |  |  |
| logPoct/water/kg·mol−1 | 0.368 | −0.198 | 0.221 | 0.225 | −0.097 | 1 |  |  |
| SASA/Angstrom2 | 0.903 | −0.077 | 0.023 | 0.969 | −0.091 | 0.232 | 1 |  |
| AF/Angstrom2 | 0.91 | −0.027 | −0.011 | 0.994 | −0.262 | 0.179 | 0.972 | 1 |

| Table S3 The necessary formulas and data for economic evaluation |  |
| --- | --- |
| Parameters | Economic calculation |
| Condensers | Heat-transfer coefficient: KC = 0.568 kW·K-1∙m-2 Heat-transfer area:AC=QC△t×kC m2 Cost=7296×AC0.65 |
| Reboilers | Heat-transfer coefficient: KR = 0.852 kW·K-1∙m-2 Heat-transfer area: AR=QR△t×kR m2 Cost=7296×AR0.65 |
| Column vessel | H=0.61×(NT-2)×1.2 shell Cost=17640×D1.066×H0.802 Diameter: D is calculated by Aspen tray sizing TCC =shell cost + HX cost |
| Energy cost | LP stream (160 ℃): 7.78 $·GJ−1 MP stream (184 ℃): 8.22 $·GJ−1 HP stream (254 ℃): 9.88 $·GJ−1 Cooling water (40 ℃): 0.354 $·GJ−1 |
| Total operating cost Payback period | TOC = 3.6x10−3EnergyC × t × Q EnergyC is the unit price of public works; t is the annual operation time, which is 8000 h; Q is the duty of heat exchanger, kW. 3 years |
| TAC | (Total capital cost/Payback period ) + Total operating cost |

| Table S4 Results of total process optimization parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Variables | Light hydrocarbon removal tower | Cyclohexane tower | Heavy hydrocarbon removal tower | EDC tower | ERC tower | MCH tower |
| NT | 40 | 44 | 40 | 44 | 24 | 88 |
| NF | 18 | 12 | 15 | 4 18 | 10 | 42 |
| RR | 7 | 6 | 16 | 5 | 5 | 40 |
| D/kmol·h−1 | 35 | 20 | 29 | 7.9 | 21 | 20 |

| Table S5 Experimental values, predicted values and molecular descriptors of RVn-heptane/MCH in naphtha under different extractants (training set) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compounds | Molecular Formula | RVsim | RVpred | RD | α/a.u. | EHOMO/eV | ρ/g·cm−3 | logPoct/water/kg·mol−1 |
| Benzyl alcohol |  | 1.22 | 1.25 | 2.3% | 101.56 | −8.02 | 1.20 | 1.15 |
| Tetraglycol |  | 1.19 | 1.18 | −0.4% | 139.69 | −9.00 | 1.24 | −0.02 |
| 1, 2-Propylene glycol |  | 1.21 | 1.22 | 0.7% | 50.20 | −9.12 | 1.13 | 0.00 |
| N-butoxyethanol |  | 1.16 | 1.18 | 1.4% | 96.49 | −8.79 | 1.08 | 1.57 |
| Hexylene glycol |  | 1.19 | 1.20 | 0.8% | 84.53 | −8.89 | 1.11 | 0.63 |
| Tetraglycol dimethyl ether |  | 1.15 | 1.15 | −0.3% | 162.55 | −8.82 | 1.19 | 1.30 |
| Trimethylene glycol dimethyl ether |  | 1.19 | 1.17 | −1.7% | 130.32 | −8.82 | 1.18 | 1.09 |
| Diglycol dimethyl ether |  | 1.18 | 1.19 | 1.0% | 98.09 | −8.81 | 1.15 | 0.86 |
| Ethylene carbonate |  | 1.25 | 1.26 | 0.9% | 48.20 | −10.32 | 1.52 | −1.12 |
| Propylene carbonate |  | 1.22 | 1.23 | 0.7% | 64.13 | −10.25 | 1.42 | −0.61 |
| Dibutyl phthalate |  | 1.14 | 1.15 | 0.9% | 233.47 | −8.68 | 1.23 | 0.00 |
| N-Formylmorpholine |  | 1.30 | 1.28 | −1.2% | 87.09 | −8.08 | 1.31 | −0.30 |
| 2-pyrrolidone |  | 1.25 | 1.27 | 1.9% | 65.49 | −8.30 | 1.23 | −0.29 |
| NMP |  | 1.27 | 1.25 | −1.5% | 84.10 | −8.21 | 1.20 | 0.62 |
| Furfural |  | 1.30 | 1.30 | 0.2% | 74.56 | −8.07 | 1.37 | 0.14 |
| Tetraethylamylamine |  | 1.24 | 1.25 | 0.5% | 148.75 | −7.46 | 1.09 | −3.21 |
| Sulfolane |  | 1.29 | 1.27 | −1.5% | 84.13 | −9.16 | 1.41 | −1.10 |
| 3-Methyl sulfolane |  | 1.28 | 1.25 | −2.3% | 100.30 | −9.15 | 1.35 | −1.10 |
| DMSO |  | 1.33 | 1.32 | −0.9% | 57.23 | −7.77 | 1.27 | −1.09 |
| 4-Tert-butyl phenol |  | 1.22 | 1.24 | 1.3% | 136.34 | −7.24 | 1.16 | 2.19 |
| Salicylic acid |  | 1.46 | 1.49 | 0.3% | 100.36 | −7.79 | 1.44 | −0.43 |
| o-Toluic acid |  | 1.29 | 1.23 | 0.6% | 115.39 | −8.30 | 1.31 | 1.39 |
| 1,2,4- Trichlorobenzene |  | 1.28 | 1.28 | −0.1% | 118.01 | −8.34 | 1.64 | 3.56 |
| Butylbenzene |  | 1.19 | 1.17 | −2.1% | 125.02 | −7.96 | 1.05 | 3.94 |
| 1,2-Dichlorobenzene |  | 1.26 | 1.27 | 0.6% | 101.86 | −8.36 | 1.51 | 3.03 |
| 1,3,5-Tritylene |  | 1.20 | 1.19 | −0.7% | 113.05 | −7.76 | 1.06 | 3.39 |

| Table S6 Experimental values, predicted values and molecular descriptors of RVn-heptane/MCH in naphtha under different extractants (validation set) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compounds | Molecular Formula | RVsim | RVpred | RD | α/a.u. | EHOMO/eV | ρ/g·cm−3 | logPoct/water/kg·mol−1 |
| Diglycol |  | 1.18 | 1.24 | 5.0% | 71.06 | −8.90 | 1.23 | −0.48 |
| 1,4-Butanediol |  | 1.20 | 1.15 | −4.5% | 68.65 | −9.11 | 1.13 | 0.06 |
| Triethylene glycol monomethyl ether |  | 1.22 | 1.21 | −1.0% | 117.00 | −8.98 | 1.21 | 0.42 |
| 1,4-Butyrolactone |  | 1.14 | 1.16 | 1.7% | 51.56 | −9.32 | 1.31 | −0.01 |
| γ-Valerolactone |  | 1.14 | 1.14 | −0.3% | 73.91 | −9.26 | 1.26 | 0.42 |
| Nitrobenzene |  | 1.35 | 1.34 | −0.1% | 101.06 | −8.88 | 1.40 | 1.36 |
| 5-Methyl-2-hexanone |  | 1.13 | 1.13 | −0.3% | 111.12 | −9.21 | 1.01 | 0.00 |
| 4-Fluorophenol |  | 1.44 | 1.42 | −1.6% | 73.90 | −7.53 | 1.44 | 1.00 |
| 1,2,4-Tritylene |  | 1.20 | 1.22 | 1.6% | 126.12 | −7.59 | 1.07 | 3.27 |

| Table S7 The energy consumption, economic performance and environmental protection analysis of the different extractive distillation separation processes |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extractants | TOC/106$ | TCC/106$ | TAC/106$ | QR/KW | QC/KW | CO2 emissions /kg·y−1 | SO2 emissions /kg·y−1 | NOx emissions /kg·y−1 |
| Salicylic acid | 4.6 | 5.3 | 6.3 | 18904.9 | -12939.3 | 8632.8 | 259.7 | 129.9 |
| 3-Methyl sulfolane | 4.4 | 6.6 | 6.6 | 18197.9 | -13447.3 | 8310.0 | 250.0 | 125.0 |
| DMP | 4.6 | 6.5 | 6.7 | 18985.9 | -13456.3 | 8669.8 | 260.8 | 130.4 |
| NMP | 5.2 | 5.1 | 6.9 | 21204.9 | -18100.3 | 9683.1 | 291.3 | 145.7 |

| Table S8 Economic evaluation of salicylic acid as extractant |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variables | T1 | T2 | T3 | T4 | T5 | T6 | SUM |
| d/m | 1.72 | 1.04 | 1.51 | 1.17 | 1.03 | 2.82 | 6.33×106 $ |
| H/m | 27.80 | 30.72 | 27.80 | 32.19 | 6.58 | 62.91 |  |
| QR/KW | 2763 | 1136 | 2102 | 1757 | 4048 | 7099 |  |
| Qc/KW | −2342 | −1131 | −2086 | −76 | −212 | −7092 |  |
| Cost of column vessel/$ | 451439 | 286896 | 393524 | 337879 | 82575 | 1474440 |  |
| AR/m2 | 97.29 | 25.00 | 105.73 | 128.89 | 1425.35 | 215.49 |  |
| AC/m2 | 130.91 | 120.69 | 51.01 | 1.72 | 4.15 | 138.73 |  |
| Cost of heat exchanger/$ | 316421 | 223621 | 244930 | 182043 | 837109 | 419872 |  |
| TOC/$ | 638205 | 264066 | 488617 | 391419 | 1153995 | 1650668 |  |
| TCC/$ | 767860 | 510517 | 638454 | 519921 | 919684 | 1894312 |  |
| TAC/$ | 894158 | 434239 | 701435 | 564726 | 1460557 | 2282105 |  |
| TAC/106$ | 0.89 | 0.43 | 0.70 | 0.56 | 1.46 | 2.28 |  |
| CO2 emissions /kg·y−1 | 1261.73 | 518.67 | 959.87 | 802.32 | 1848.50 | 3241.72 | 9022.38 kg·y−1 |
| SO2 emissions /kg·y−1 | 37.96 | 15.60 | 28.88 | 24.14 | 55.61 | 97.52 |  |
| NOx emissions /kg·y−1 | 18.98 | 7.80 | 14.44 | 12.07 | 27.81 | 48.76 |  |