Electronic Supplementary Material

Azeotropic distillation process for methanol-ethyl acetate-water separation: design and control

Prakhar Srivastava, Prit Dadhania, Aayush Gupta, Nitin Kaistha ()

Department of Chemical Engineering, Indian Institute of Technology, Kanpur 208016, UP, India

E-mail: nkaistha@iitk.ac.in

List of Figures

S1         Variation of distribution coefficient with solvent rate . . . . . . . . . . . . . .	2

S2	Nominal design and operating condition of proposed design without HI . . .	3

S3	C1 column composition and temperature profile of proposed design	. . . . .	4

S4	C2 column composition and temperature profile of proposed design	. . . . .	5

S5	C3 column composition and temperature profile of proposed design	. . . . .	6

S6	Optimization of dominant design variable of EHAD design (a) water composi-

tion (b) C1 column EtAc composition (c) Water split rate	. . . . . . . . . .	8

S7         Pressure Swing Design ……………………………………………………               12

List of Tables

S1 NRTL interaction parameter adapted from Srivastava et al.1	2

S2 Equipment dimensioning, cost relationships, and unit pricing data	9

S3 Major equipment cost and dimensional details of adapted EHAD design	10

S4	 The parameters and their values used for environmental evaluation ……………………….	11

NRTL interaction parameter

Table S1: NRTL interaction parameter adapted from Srivastava et al.1

| Comp i | Comp j | Source | Aij | Aji | Bij | Bji | Cij |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MeOH | Water | Regressed | 3.6493 | -2.471 | -1556.88 | 1460.82 | 0.13 |
| MeOH | EtAc | Regressed | 4.5620 | -5.518 | -1196.92 | 2216.77 | 1.11 |
| Water | EtAc | Regressed | -3.750 | -3.640 | 2229.930 | 1699.11 | 0.36 |

Proposed Design

LLX Design

Distribution Coefficient (D) is defined as the ratio of the concentration of solute in the raffinate phase to the concentration of solute in the extract phase.

We have incorporated the distribution coefficient (D) as a quantitative indicator for mass

transfer.

Figure S1: Variation of distribution coefficient with solvent rate

A sensitivity analysis was conducted by varying the solvent (water) flow rate while keeping the number of stages fixed. As shown in Figure S1, the distribution coefficient exhibits a maximum value of 3.02 to 200 kmol/hr. The TAC optimum water flow rate is lower at 138 kmol/hr. At this flow rate, the distribution coefficient is 2.75, indicating effective mass transfer. Increasing the distribution coefficient further would require raising the water flow rate to approximately 200 kmol/h, which incurs a higher solvent recovery and recycle cost.

Without heat integration (HI) design

Figure S2: Nominal design and operating condition of proposed design without HI

Column Profiles

Figure S3: C1 column composition and temperature profile of proposed design

Figure S4: C2 column composition and temperature profile of proposed design

Figure S5: C3 column composition and temperature profile of proposed design

Dominant design variables of EHAD design

Similar to our proposed design, we used engineering heuristics to fix the degrees of freedom, after which the economically dominant design variables were identified and fixed using the Aspen Plus sensitivity tool.

The three dominant design variables are as follows: water composition in the C2 column distillate xWater, EtAc composition in the C1 bottom stream xEtAc, and the water split rate

FSplit to the C1 column. To obtain near-optimum value of TAC, we performed iterative cycles. During each cycle, we systematically adjusted each of the three design variables within a specified range, keeping the others constant. Each variable was updated to the value that minimized TAC, and the iteration process continued until the reduction in TAC between cycles was smaller than a predefined tolerance.

Figures S6 a, b, and c show how TAC changes in relation to the recommended design values that correspond to the minimum TAC. The design value for xWater in the C2 distillate is 0.28; increasing this value further results in a rise in TAC as the composition approaches the distillation boundary, which raises the distillate rate (D2). For xEtAc, values above the optimum result in a higher TAC due to an increase in the C3 column reboiler duty, while values below the optimum TAC rise due to the higher reboiler duty in the C1 column. The optimal water split rate FSplit is 427.49 kmol/h; above this value, TAC increases due to a higher solvent water recycle rate, which increases the reboiler duties in both the C1 and C3 columns. Below this optimum, TAC increases due to the rise in the C2 column reboiler duty.

Figure S6: Optimization of dominant design variable of EHAD design (a) water composition

(b) C1 column EtAc composition (c) Water split rate

Costing and sizing details

Table S2: Equipment dimensioning, cost relationships, and unit pricing data

| Column | Diameter | Tray Sizing utility (Fair’s method) |
| --- | --- | --- |
|  | Height | Tray with 2 ft spacing 20 % Extra length for sump |
|  | Capital cost | $17640(D)1.066(L)0.802 (D,L in m) |
| Condenser | HTC | 0.568 kW/(K.m2) |
|  | Capital cost | $7296(A)0.65 |
| Reboiler | HTC | 0.852 kW/(K.m2) |
|  | Capital cost | $7296(A)0.65 |
| FEHE | HTC | 0.85 kW/(K.m2) |
| LLX | HETS | 2 ft |
|  | Capital cost | $17640(D)1.066(L)0.802 (D,L in m) |
| Utilities | HP Steam | $17.7 /GJ |
|  | MP Steam | $14.85 /GJ |
|  | LP Steam | $14.05 /GJ |
|  | Cooling water | $0.354 /GJ |

HTC=Heat transfer coefficient A= area

Table S3: Major equipment cost and dimensional details of adapted EHAD design

|  | Measurement |  | CC $106 | Energy kW | EC $106/yr |
| --- | --- | --- | --- | --- | --- |
| C1 Column | Height (m) Diameter (m) | 6.58 0.85 | 0.067 |  |  |
|  | Condenser (m2) | 23.16 | 0.0562 | 584.7 | 0.0054 |
|  | Reboiler (m2) | 22.1 | 0.0546 | 1310.7 | 0.4773 |
| C2 Column | Height (m) Diameter (m) | 13.89 0.76 | 0.1081 |  |  |
|  | Condenser (m2) | 25.24 | 0.0595 | 616.6 | 0.0057 |
|  | Reboiler (m2) | 11.1 | 0.0349 | 737.9 | 0.2687 |
| C3 Column | Height (m) Diameter (m) | 19.02 0.79 | 0.1458 |  |  |
|  | Condenser (m2) | 58.26 | 0.1025 | 1295.5 | 0.0119 |
|  | Reboiler (m2) | 31.69 | 0.0689 | 1450.8 | 0.5283 |
| Decanter |  |  | 0.0453 |  |  |
| FEHE | Area (m2) | 1.65 | 0.0101 | 37.8 |  |
| Cooler 1 | Area (m2) | 2.28 | 0.0124 | 29.1 | 0.0003 |
| Cooler 2 | Area (m2) | 50.74 | 0.0937 | 648.5 | 0.0059 |
| Cooler 3 | Area (m2) | 4.57 | 0.0196 | 58.4 | 0.0005 |
| Cooler 4 | Area (m2) | 12.7 | 0.038 | 162.1 | 0.0015 |
| TCC |  |  | 0.9166 |  |  |
| YOC |  |  |  |  | 1.3055 |
| TAC |  |  |  | $1.611×106 /yr |  |

CC=Capital cost, EC=Energy Cost, TCC=Total Capital cost

YOC=Yearly Operating Cost

CO2 analysis

(1)

(2)

All the values given in the equation are given in Table S4.2

Table S4: The parameters and their values used for environmental evaluation

| Variable | Definition | Value | Unit |
| --- | --- | --- | --- |
| Qfuel | energy consumption of the heavy oil fuel | - | kJ/s |
| NHV | net heating value | 39771 | kJ/kg |
| C% | carbon content | 86.5 | kg/kg |
| α | the molar mass of CO2 and C | 3.67 | - |
| Qtotal | reboiler energy consumption | - | kW |
| λsteamLP | latent heat of the steam | 2081.63 | kJ/kg |
| λsteamMP | latent heat of the steam | 1999.69 | kJ/kg |
| λsteamHP | latent heat of the steam | 1725.33 | kJ/kg |
| hsteamLP | enthalpy of the steam | 2756.37 | kJ/kg |
| hsteamMP | enthalpy of the steam | 2778.64 | kJ/kg |
| hsteamHP | enthalpy of the steam | 2800.32 | kJ/kg |
| Tflame | flame temperature | 2073.15 | K |
| Tstack | stack temperature | 433.15 | K |
| Tambient | ambient temperature | 298.15 | K |

Structure of PSD Design

Figure S7: Pressure Swing Design

References

- Srivastava, P.; Gupta, A.; Kaistha, N. Synthesis and Integrated Design of a Compact Azeotropic Process for EtAc–MeOH–Water Separation. Industrial & Engineering Chemistry Research 2024, 63, 21466–21478.

- Mukherjee, R.; Mondal, B.; Jana, A. K. A Carbon-Neutral Highly Productive SolarPowered Desalination Process: Improving Economic Performance with Carbon Pricing.

Industrial & Engineering Chemistry Research 2024, 63, 9107–9123.