Electronic Supplementary Material

In-depth multi-component analysis of bio-aviation fuel derived from waste cooking oil using comprehensive two-dimensional gas chromatography mass spectrometry

Yang Xu1, Xuan Guo ()1, Meng Wang ()2, Yunming Fang1

1 College of Chemical Engineering, Beijing University of Chemical Technology, Beijing 100029, China

2 College of Life Science and Technology, Beijing University of Chemical Technology, Beijing 100029, China

E-mails: guoxuan@buct.edu.cn (Guo X); wangmeng@mail.buct.edu.cn (Wang M)

Fig.S1. Two-dimensional plot of bio-aviation fuel (a) split ratio 1:50, (b) heating rate 5 °C/min and (c) modulation period 4 s.

Table S1. Detailed instrument parameters for GC×GC-MS

|  | GC×GC | MS |  |  |
| --- | --- | --- | --- | --- |
| Carrier gas | He, 1.5 mL/min; constant flow |  | Mode | TIC |
| Injection | 0.1 μL split 1:100 at 250 °C |  | Transfer line temperature | 300 °C |
| Columns | 1st HP-5MS (30 m × 0.25 mm × 0.25 μm) |  | Mass range | 33-500 amu |
|  | 2nd BPX50 (2.5 m × 0.1 mm × 0.1 μm) |  |  |  |
| Temperature program | 35 °C (3 min)_3 °C/min_170 °C (1 min) |  | Ion source | 230 °C |
| Modulation period | 6 s; 0.6 s hot pulse; 300 °C |  | EI ionization | - eV |

Table S2. Information of name, CAS of identified compounds in bio-aviation fuel

| No. | Compound | CAS |  |  | Retention Time |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1D a) /min | 2D b) /s |  |
| 1 | n-Octane |  | 111-65-9 | 12.70 |  |  | 1.26 |
| 2 | n-Nonane |  | 111-84-2 | 17.80 |  |  | 1.35 |
| 3 | n-Decane |  | 124-18-5 | 23.20 |  |  | 1.41 |
| 4 | n-Undecane |  | 1120-21-4 | 28.40 |  |  | 1.44 |
| 5 | n-Dodecane |  | 112-40-3 | 33.40 |  |  | 1.47 |
| 6 | n-Tridecane |  | 629-50-5 | 38.10 |  |  | 1.50 |
| 7 | n-Tetradecane |  | 629-59-4 | 42.50 |  |  | 1.50 |
| 8 | n-Pentadecane |  | 629-62-9 | 46.60 |  |  | 1.56 |
| 9 | n-Hexadecane |  | 544-76-3 | 50.60 |  |  | 1.56 |
| 10 | 1,2-Dimethylcyclohexane |  | 583-57-3 | 14.20 |  |  | 1.44 |
| 11 | 1,2,4-Trimethylcyclohexane |  | 2234-75-5 | 16.80 |  |  | 1.44 |
| 12 | 1,1,3,5-tetramethylcyclohexane |  | 4306-65-4 | 18.60 |  |  | 1.41 |
| 13 | decahydronaphthalene |  | 91-17-8 | 28.60 |  |  | 2.07 |

a) 1D is first dimension retention time; b) 2D is second dimension retention time

Table S3. Detailed quantitation of alkanes and cycloalkanes in bio-aviation fuel

| Alkanes | Concentration ± CI a) /wt.% | RSD b) /% |
| --- | --- | --- |
| C8 | 0.962±0.123 | 11.5 |
| C9 | 0.885±0.177 | 7.8 |
| C10 | 2.984±0.404 | 2.7 |
| C11 | 13.570±0.424 | 3.5 |
| C12 | 32.638±1.689 | 5.3 |
| C13 | 25.597±1.134 | 4.0 |
| C14 | 8.961±0.897 | 9.1 |
| C15 | 4.089±0.202 | 4.3 |
| C16 | 0.771±0.038 | 4.4 |
| Cycloalkanes | Concentration ± CI a /wt.% | RSD b /% |
| C11 | 0.662±0.021 | 3.1 |
| C12 | 1.131±0.113 | 9.4 |
| C13 | 0.265±0.006 | 2.7 |
| decahydronaphthalenes | 2.755±0.142 | 6.3 |

a) Confidence interval at 95% of the confidence level; b) RSD, relative standard deviation

Table S4. Average of volume and RSD of FAMEs (5 ppm) in bio-aviation fuel

| FAMEs | C16:0 | C16:1 | C18:0 | C18:1 | C18:2 | C18:3 |
| --- | --- | --- | --- | --- | --- | --- |
| Average of volume | 25153384 | 27517799 | 27404606 | 15689690 | 10185818 | 3603060 |
| RSD /% | 2 | 3 | 2 | 5 | 8 | 2 |

Table S5. Average of volume and RSD of FAMEs (2 ppm) in bio-aviation fuel

| FAMEs | C16:0 | C16:1 | C18:0 | C18:1 | C18:2 | C18:3 |
| --- | --- | --- | --- | --- | --- | --- |
| Average of volume | 10159064 | 27517798 | 8862876 | 8014821 | 4007288 | 2809666 |
| RSD % | 6 | 5 | 7 | 7 | 11 | 6 |