# Electronic Supplementary Material

Real-time yield prediction in microchannel gas-liquid sulfonation via augmented convolutional long short-term memory-based soft measurement

Yingjin Wang, Yingxin Mu, Shaokui Fu, Muxuan Qin, Wenjin Zhou, Wei Zhang ()

Shanxi Key Laboratory of Chemical Product Engineering, College of Chemistry and Chemical Engineering, Taiyuan University of Technology, Taiyuan, Shanxi 030024, China

E-mail: zhangwei01@tyut.edu.cn

Table S1 HPLC analytical conditions

| Chromatographic conditions | Parameters |
| --- | --- |
| Column | C18 column（250×4.6×5 μm） |
| Mobile phase | Methanol |
| Detector | Differential refractive index detector |
| Speed of moving fluid | 0.017 cm3*s-1 |
| Column temperature | 40 ℃ |
| Feed volume | 15 μl |

Table S2 Detailed information on network structure and parameters

| Phase | Layer | Output |
| --- | --- | --- |
| Input | Input | (B, 3, T, 200, 60) |
| Spatio-temporal feature extraction | ConvLSTM(kernel=3x3, padding=1) | (B, 32, T, 200, 60) |
|  | BatchNorm3d | (B, 32, T, 200, 60) |
|  | ConvLSTM(kernel=3x3, padding=1) | (B, 64, T, 200, 60) |
|  | BatchNorm3d | (B, 64, T, 200, 60) |
|  | Dropout3d(p=0.25) | (B, 64, T, 200, 60) |
| Frame-by-frame spatial feature deepening | TD(Conv2d) (kernel=3x3, padding=1) | (B, 64, T, 200, 60) |
|  | TD(BatchNorm2d) | (B, 64, T, 200, 60) |
|  | TD(ReLU) | (B, 64, T, 200, 60) |
|  | TD(MaxPool2d) (kernel=2x2) | (B, 64, T, 100, 30) |
|  | TD(Conv2d) (kernel=3x3, padding=1) | (B, 64, T, 100, 30) |
|  | TD(BatchNorm2d) | (B, 64, T, 100, 30) |
|  | TD(ReLU) | (B, 64, T, 100, 30) |
|  | TD(MaxPool2d) (kernel=2x2) | (B, 64, T, 50, 15) |
| Feature Integration and Classification | TD(Flatten) | (B, T, 48000) |
|  | Select Last Time | (B, 48000) |
|  | Fully Connect | (B, 128) |
|  | ReLU | (B, 128) |
|  | Dropout(p=0.5) | (B, 128) |
|  | Fully Connect | (B, 64) |
|  | ReLU | (B, 64) |
|  | Dropout(p=0.5) | (B, 64) |
| Output | Output | (B, 3) |

Table S3 Summary of 5-fold cross-validation performance (T=12)

| Fold | Best validation loss | Best validation accuracy/% |
| --- | --- | --- |
| 1 | 0.0624 | 96.88 |
| 2 | 0.0491 | 100 |
| 3 | 0.0251 | 100 |
| 4 | 0.0759 | 96.77 |
| 5 | 0.0773 | 93.55 |
| Average value | 0.0580 | 97.44 |

Fig. S1 Model architecture flowchart