Hand landmarks in MediaPipe library

source: https://ai.google.dev/edge/mediapipe/solutions/vision/hand_landmarker

In order to use firmate library, you must install Arduino IDE software:

https://www.arduino.cc/en/software

You should open StandardFirmata on Arduino Nano by accessing

FileExamplesFirmataStandardFirmata

Then, run SketchUpload command after setting ToolsboardArduino AVR boardsArduino Nano, ToolsProcessor: ATmega328P or ATmega328P (Old bootloader), and Tools Port (my case COM3).

For fvsw.py, connect the two pins of the LED on Arduino Nano, the short leg to the GND pin and the long leg to the digital pin #2.

For fservo.py, connect the three pins (brown, red, and orange) of a servo motor (SG90) on Arduino Nano, the brown wire to the GND pin, the red wire to the 5V pin, and the orange wire to the digital pin #2.

The fvsw.py application determines whether the LED is on or off by identifying whether the tip of the index finger is within the zone (area) specified by the two-dimensional coordinates where these coordinates are generated by MediaPipe.

fservo.py identifies the horizontal axis of the index finger tip to determine the angle of the servo motor where MediaPipe generates the horizontal data and pyFirmata sends the date to firmata on Arudino.