Supplementary Materials

Spatiotemporal simulation, source identification, and ecological risk assessment of potentially toxic elements in river sediments from the Jinsha River Basin, southwest China

Lanchu TAO 1,2, Denghui WEI 3,4, Jinhai YU 3,4, Qingsong CHEN 1,2, Yunhui ZHANG () 3,4

1 Kunming General Survey of Natural Resources Center, China Geological Survey, Kunming 650100, China

2 Innovation Base for Eco-geological Evolution, Protection, and Restoration of Southwest Mountainous Areas, Geological Society of China, Kunming

650100, China

3 Faculty of Geosciences and Engineering, Southwest Jiaotong University, Chengdu 611756, China

4 Yibin Research Institute, Southwest Jiaotong University, Yibin 644000, China

Spatiotemporal simulation, source identification, and ecological health risk assessment of heavy metals in river sediments from the Jinsha River Basin, Southwest China

2.7 Human health risk assessment

Three main exposure pathways (ingestion, inhalation, and dermal contact) and two health risks (non-carcinogenic and carcinogenic) were considered, and the population was divided into two groups (children and adults) because of their physiological differences. The calculation process for the health risk value is given by Eq. 1-2, where the ADD represents the average daily intake dose, C is the observed concentration of PTEs, and THI and TCR are the total non-carcinogenic and carcinogenic health risks, respectively. The introduction and reference values of the other exposure parameters are listed in Tables S7 and S8.

ADDing=C×IngR×ED×EFBW×ATca/nc×10−6ADDder=C×SA×SL×ABF×ED×EFBW×ATca/nc×10−6ADDinh=C×InhR×ED×EFPEF×BW×ATca/nc#1

THI=HQi=ADDingRfDing+ADDderRfDder+ADDinhRfDinhTCR=CRi=(ADDing×SFing+ADDder×SFder+ADDinh×SFinh)#2

When the non-carcinogenic hazard quotient (HQ) and carcinogenic hazard quotient (CR) caused by a single substance or their total risk value (THI or TCR) exceed 1.0 and 10-4, respectively. It is considered an unacceptable non-carcinogenic risk or carcinogenic risk.

Table S1 Methods and detection limits of PTEs in river sediments (mg/kg).

| HMs | Methods | Detect limits |
| --- | --- | --- |
| As | ICP-MS | 0.2 |
| Cr | ICP-MS | 2.0 |
| Cd | ICP-MS | 0.03 |
| Hg | AFS | 0.0003 |
| Pb | ICP-MS | 1.0 |
| Cu | ICP-MS | 0.7 |
| Ni | ICP-MS | 2.0 |
| Zn | ICP-MS | 5.0 |

Table S2 Statistical result of the geo-accumulation index.

| Parameters | Igeo |  |  |  |  |  |  |  | INI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | As | Cd | Cr | Hg | Pb | Cu | Ni | Zn |  |
| mean | -1.18 | 0.90 | 0.52 | -0.76 | 0.44 | 0.94 | -0.02 | 0.67 | 1.80 |
| min | -3.54 | -1.71 | -1.37 | -3.44 | -2.33 | -1.90 | -1.60 | -1.73 | 0.70 |
| max | 1.85 | 4.21 | 2.32 | 2.52 | 4.83 | 2.26 | 1.80 | 2.78 | 3.76 |
| <0 | 71.79% | 43.59% | 17.95% | 66.67% | 58.97% | 12.82% | 41.03% | 17.95% | - |
| 0~0.5 | - |  |  |  |  |  |  |  | 0.00% |
| 0.5~1 |  |  |  |  |  |  |  |  | 20.51% |
| 0~1 | 7.69% | 17.95% | 58.97% | 25.64% | 2.56% | 23.08% | 51.28% | 48.72% | 20.51% |
| 1~2 | 20.51% | 10.26% | 17.95% | 5.13% | 5.13% | 58.97% | 7.69% | 12.82% | 53.85% |
| 2~3 | 0.00% | 10.26% | 5.13% | 2.56% | 7.69% | 5.13% | 0.00% | 20.51% | 5.13% |
| 3~4 | 0.00% | 15.38% | 0.00% | 0.00% | 10.26% | 0.00% | 0.00% | 0.00% | 20.51% |
| 4~5 | 0.00% | 2.56% | 0.00% | 0.00% | 15.38% | 0.00% | 0.00% | 0.00% | 0.00% |
| >5 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

Table S3 Statistical result of the potential ecological risk index.

| Parameters | Er |  |  |  |  |  |  |  | PERI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | As | Cd | Cr | Hg | Pb | Cu | Ni | Zn |  |
| mean | 13.33 | 169.23 | 4.95 | 55.19 | 38.49 | 16.48 | 8.66 | 3.20 | 309.54 |
| min | 1.29 | 13.76 | 1.16 | 5.52 | 1.49 | 2.01 | 2.48 | 0.45 | 31.69 |
| max | 54.18 | 833.94 | 14.94 | 344.83 | 212.93 | 35.85 | 26.12 | 10.32 | 1214.30 |
| <40 | 87.18% | 35.90% | 100.00% | 43.59% | 71.79% | 100.00% | 100.00% | 100.00% | - |
| 40~80 | 12.82% | 23.08% | 0.00% | 33.33% | 7.69% | 0.00% | 0.00% | 0.00% |  |
| 80~160 | 0.00% | 10.26% | 0.00% | 20.51% | 17.95% | 0.00% | 0.00% | 0.00% |  |
| 160~320 | 0.00% | 10.26% | 0.00% | 0.00% | 2.56% | 0.00% | 0.00% | 0.00% |  |
| >320 | 0.00% | 20.51% | 0.00% | 2.56% | 0.00% | 0.00% | 0.00% | 0.00% |  |
| <150 | - |  |  |  |  |  |  |  | 51.28% |
| 150~300 |  |  |  |  |  |  |  |  | 15.38% |
| 300~600 |  |  |  |  |  |  |  |  | 12.82% |
| >600 |  |  |  |  |  |  |  |  | 20.51% |

Table S4 Statistical result of the environmental capacity index.

| Parameters | Pi |  |  |  |  |  |  |  | PI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | As | Cd | Cr | Hg | Pb | Cu | Ni | Zn |  |
| Mean | 0.30 | -4.00 | 0.45 | 0.99 | -1.47 | -0.98 | 0.74 | -0.02 | -0.50 |
| Min | -10.76 | -44.15 | -1.28 | 0.87 | -14.81 | -4.32 | -0.24 | -4.22 | -7.15 |
| Max | 3.43 | 1.31 | 1.15 | 1.01 | 1.22 | 1.52 | 1.15 | 1.23 | 1.44 |
| <0 | 30.77% | 41.03% | 12.82% | 0.00% | 38.46% | 79.49% | 10.26% | 30.77% | 30.77% |
| 0~0.3 | 0.00% | 2.56% | 17.95% | 0.00% | 0.00% | 5.13% | 0.00% | 5.13% | 10.26% |
| 0.3~0.7 | 2.56% | 15.38% | 38.46% | 0.00% | 0.00% | 2.56% | 23.08% | 28.21% | 23.08% |
| 0.7~1.0 | 5.13% | 28.21% | 20.51% | 56.41% | 5.13% | 7.69% | 41.03% | 28.21% | 17.95% |
| >1 | 61.54% | 12.82% | 10.26% | 43.59% | 56.41% | 5.13% | 25.64% | 7.69% | 17.95% |

Table S5 Statistical results of contribution rate of different PTEs to the THI.

| Parameters |  | As | Cd | Cr | Hg | Pb | Cu | Ni | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Children | Min | 23.20% | 0.20% | 0.01% | 0.03% | 10.16% | 0.15% | 0.23% | 0.16% |
|  | Mean | 40.46% | 0.70% | 0.29% | 0.20% | 43.18% | 3.91% | 10.70% | 0.57% |
|  | Max | 81.90% | 1.51% | 1.08% | 1.01% | 75.73% | 18.11% | 42.47% | 1.38% |
| Adults | Min | 37.90% | 0.24% | 0.01% | 0.03% | 4.91% | 0.10% | 0.30% | 0.09% |
|  | Mean | 55.74% | 1.12% | 0.55% | 0.20% | 28.67% | 2.39% | 10.98% | 0.35% |
|  | Max | 88.85% | 2.36% | 2.01% | 0.82% | 58.38% | 11.74% | 42.85% | 0.85% |

Table S6 Statistical results of contribution rate of different routes to the health risks.

| Parameters |  | THI |  |  | TCR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ingestion | Dermal contact | Inhalation | Ingestion | Dermal contact | Inhalation |
| Children | Min | 72.42% | 8.68% | 0.08% | 68.33% | 31.67% | 0.00% |
|  | Mean | 84.00% | 15.65% | 0.36% | 68.33% | 31.67% | 0.00% |
|  | Max | 91.24% | 27.21% | 0.98% | 68.33% | 31.67% | 0.00% |
| Adults | Min | 32.79% | 33.65% | 0.40% | 28.68% | 71.31% | 0.01% |
|  | Mean | 50.25% | 48.39% | 1.36% | 28.68% | 71.31% | 0.01% |
|  | Max | 65.96% | 66.10% | 3.63% | 28.68% | 71.31% | 0.01% |

Table S7 Reference values of exposure parameters in the health risk calculation.

| Parameter | Description | Unit | Values |  | Probabilistic Distribution | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Children | Adult |  |  |
| IngR | Ingestion rate | mg/d | 103 | 30 | Triangular (66,103,161)/(4,30,52) | (Huang et al., 2021; Yang et al., 2024) |
| InhR | Inhalation rate | mg/d | 7.5 | 14.5 | Normal (7.5,0.75)/(14.5,0.145) | (USEPA 1996; MEPPRC, 2014) |
| EF | Exposure frequency | d/a | 345 | 345 | Triangular (180,315,365) | (Shen et al., 2024; Yang et al., 2024) |
| ED | Exposure duration | a | 6 | 24 | Uniform | (USEPA, 2011) |
| BW | Body weight | kg | 19.2 | 61.8 | Log-normal (19.2,0.19)/(61.8,0.62) | (MEPPRC, 2014) |
| AT (nc) | Average exposure time | d | 365*ED |  | Point | (MEPPRC, 2014) |
| AT (ca) |  |  | 365*70=25550 |  |  |  |
| SA | Surface area of skin | cm2 | 2448 | 5075 | Point | (MEPPRC, 2014) |
| SL | Skin adherence factor | mg/(cm2·d) | 0.65 | 0.49 | Log-normal (0.65,1.2)/(0.49,0.54) | (Huang et al., 2021; Yang et al., 2024) |
| ABF (nc) | Dermal absorption factor | unitless | 0.001 |  | Point | (Yang et al., 2024; Zuo et al., 2022) |
| ABF (ca) |  |  | 0.01 |  |  |  |
| ABF(As) |  |  | 0.03 |  |  |  |
| PEF | Particle emission factor | m3/kg | 1.36E+09 |  | Point | (USEPA, 2011; MEEP, 2016） |

Table S8 Carcinogenic slope factors and non-carcinogenic reference doses of PTEs* (mg/(kg·d)).

| PTEs | SF |  |  | RfD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ingestion | Inhalation | Dermal contact | Ingestion | Inhalation | Dermal contact |
| As | 1.50 | 1.50 | 1.50 | 3.00E-4 | 3.52E-6 | 3.00E-4 |
| Cd | - | - | - | 1.00E-3 | 2.35E-06 | 2.5E-05 |
| Cr | - | - | - | 1.50 | - | 1.95E-2 |
| Hg | - | - | - | 3.00E-4 | 7.04E-5 | 2.10E-5 |
| Pb | - | - | - | 1.40E-3 | - | 1.40E-3 |
| Cu | - | - | - | 4.00E-2 | - | 4.00E-2 |
| Ni | - | - | - | 2.00E-2 | 2.11E-5 | 8.00E-4 |
| Zn | - | - | - | 3.00E-1 | - | 3.00E-1 |

*The reference values are from the Technical guidelines for risk assessment of soil contamination of land for construction (HJ 25.3-2019)

Reference

MEEPRC, Technical guidelines for risk assessment of soil contamination of land for construction (HJ 25.3-2019). Ministry of Ecology and Environment of the People’s Republic of China.

MEPPRC, Technical Guidelines for Risk Assessment of Contaminated Sites, HJ 25.3-2014, 2014, Ministry of Environmental Protection of the People's Republic of China; Beijing, (in Chinese).

US EPA (1996). Soil Screening Guidance: Technical Background Document, 1996, Superfund US EPA.

US EPA (2001). Supplemental Guidance for Developing Soil Screening Levels for Superfund Sites. OSWER 9355.4-24 Office of Solid Waste and Emergency Response

US EPA, Exposure Factors Handbook: 2011 Edition (2011), National Center for Environmental Assessment Office of Research and Development U.S. Environmental Protection Agency; Washington, 20460, (EPA/600/R-09/052F).

Huang J, Wu Y, Sun J, Li X, Geng X, Zhao M, Sun T, Fan Z (2021). Health risk assessment of heavy metal(loid)s in park soils of the largest megacity in China by using monte carlo simulation coupled with positive matrix factorization model. Journal of Hazardous Materials, 415, 125629.

Shen C, Huang S, Wang M, Wu J, Su J, Lin K, Chen X, He T, Li Y, Sha C, Liu M (2024). Source-oriented health risk assessment and priority control factor analysis of heavy metals in urban soil of shanghai. Journal of Hazardous Materials, 480, 135859.

Yang Y, Lu X, Yu B, Wang Z, Wang L, Lei K, Zuo L, Fan P, Liang T (2024). Exploring the environmental risks and seasonal variations of potentially toxic elements (PTEs) in fine road dust in resource-based cities based on monte carlo simulation, geo-detector and random forest model. Journal of Hazardous Materials, 473, 134708.

Zuo L, Lu X, Fan P, Wang L, Yu B, Lei K, Yang Y, Chen Y (2022). Concentrations, sources and ecological–health risks of potentially toxic elements in finer road dust from a megacity in north China. Journal of Cleaner Production, 358, 132036.