Life cycle assessment of hydrogen production pathways to support hydrogen decarbonization policies in a Canadian context

Supplementary information

Contents

S1.	Life cycle inventory (LCI) dataset availability	1

- Life cycle inventory (LCI) dataset availability

LCI datasets generated in this study are provided in Tables S1-S7. Furthermore, these LCI datasets can be made available upon request using NRC sustainability datahub. These datasets are compatible openLCA software platform and EcoInvent database as background data. The following data quality criteria order was used for flow-based data quality (reliability, completeness, time representativeness, geographical representativeness, technological representativeness). The following data quality criteria order was used for process-based data quality (process completeness, process review).

Table S1 Life cycle inventory of hydrogen production from alkaline electrolysis

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Desiccant, activated carbon | 6.3E–9 | kg | (3,5,1,3,2) |  |
| Catalyst, alumina | 4.9E–7 | kg | (3,5,1,3,2) |  |
| Catalyst, palladium | 2.5E–8 | kg | (3,5,1,3,2) |  |
| Deionized water required | 12.9 | kg | (3,5,1,3,2) |  |
| Electricity required | 50.4 | kWh | (3,5,1,3,2) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,1,3,2) |  |
| Oxygen produced | 7.9 | kg | (3,5,1,3,2) |  |
| Wastewater produced | 3.9 | kg | (3,5,1,3,2) |  |

Table S2 Life cycle inventory of hydrogen production from proton exchange membrane electrolysis

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Deionized water required | 9.1 | kg | (3,5,1,3,2) |  |
| Electricity required | 55.3 | kWh | (3,5,1,3,2) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,1,3,2) |  |
| Oxygen produced | 8.0 | kg | (3,5,1,3,2) |  |

Table S3 Life cycle inventory of hydrogen production from biomass gasification with carbon capture

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Deionized water required | 8.3 | kg | (3,5,1,1,2) |  |
| Electricity required | 7.7 | kWh | (3,5,1,1,2) |  |
| Wood chips, wet | 17.3 | kg | (3,5,1,1,2) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,1,3,2) |  |
| CO2, liquid, co-product | 16.5 | kg | (3,5,1,3,2) |  |
| CO2, biogenic | 4.5 | kg | (3,5,1,3,2) |  |
| CO, biogenic | 1.0E–6 | kg | (3,5,1,3,2) |  |
| Electricity, co-product | 0.3 | kWh | (3,5,1,3,2) |  |
| Methane, biogenic | 2.0E–6 | kg | (3,5,1,3,2) |  |
| MDEA, emissions to air | 1.8E–4 | kg | (3,5,1,3,2) |  |
| Nitrogen | 25.9 | kg | (3,5,1,3,2) |  |
| Oxygen | 0.1 | kg | (3,5,1,3,2) |  |
| Wastewater | 4.7 | kg | (3,5,1,3,2) |  |
| Water vapor, emissions to air | 2.6 | kg | (3,5,1,3,2) |  |

Table S4 Life cycle inventory of hydrogen production from steam methane reforming without carbon capture

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Activated carbon | 4.32E–4 | kg | (3,5,3,3,1) |  |
| Alumina, catalysts | 3.0E–6 | kg | (3,5,3,3,1) |  |
| Copper, catalysts | 9.0E–6 | kg | (3,5,3,3,1) |  |
| Nickel, catalysts | 3.0E–6 | kg | (3,5,3,3,1) |  |
| Electricity | 0.002 | kWh | (3,5,3,3,1) |  |
| Natural gas, high pressure | 3.6 | m3 | (3,5,3,3,1) |  |
| Deionized water | 10.0 | kg | (3,5,3,3,1) |  |
| Zinc, catalysts | 1.0E–5 | kg | (3,5,3,3,1) |  |
| Zinc oxide, sulfur guard bed materials | 1.0E–6 | kg | (3,5,3,3,1) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,3,3,1) |  |
| CO2, fossil | 9.33269 | kg | (3,5,3,3,1) |  |
| CO, fossil | 2.62E–4 | kg | (3,5,3,3,1) |  |
| Dinitrogen monoxide | 1.13E–6 | kg | (3,5,3,3,1) |  |
| Hydrogen, emissions to air | 1.4E–5 | kg | (3,5,3,3,1) |  |
| Nitric oxide | 0.0175 | kg | (3,5,3,3,1) |  |
| Nitrogen | 21.301634 | kg | (3,5,3,3,1) |  |
| Nitrogen dioxide | 4.27E–5 | kg | (3,5,3,3,1) |  |
| Oxygen | 1.090829 | kg | (3,5,3,3,1) |  |
| SO2 | 1.8E–4 | kg | (3,5,3,3,1) |  |
| Water vapor, emissions to air | 7.5 | kg | (3,5,3,3,1) |  |

Table S5 Life cycle inventory of hydrogen production from steam methane reforming with carbon capture

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Activated carbon | 4.3E–4 | kg | (3,5,3,3,1) |  |
| Alumina, catalysts | 1.7E–5 | kg | (3,5,3,3,1) |  |
| Copper, catalysts | 9.0E–6 | kg | (3,5,3,3,1) |  |
| Nickel, catalysts | 3.0E–6 | kg | (3,5,3,3,1) |  |
| Electricity | 2.3 | kWh | (3,5,3,3,1) |  |
| Heat | 32.5 | MJ | (3,5,3,3,1) |  |
| MDEA | 0.01 | kg | (3,5,3,3,1) |  |
| Natural gas, high pressure | 4.8 | m3 | (3,5,3,3,1) |  |
| Deionized water | 110.6 | kg | (3,5,3,3,1) |  |
| Zinc, catalysts | 1.0E–6 | kg | (3,5,3,3,1) |  |
| Zinc oxide, sulfur guard bed materials | 1.0E–5 | kg | (3,5,3,3,1) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,3,3,1) |  |
| CO2, fossil | 0.9 | kg | (3,5,3,3,1) |  |
| CO2, co-product | 8.4 | kg | (3,5,3,3,1) |  |
| Nitric oxide | 0.01 | kg | (3,5,3,3,1) |  |
| Nitrogen | 21.3 | kg | (3,5,3,3,1) |  |
| Oxygen | 1.1 | kg | (3,5,3,3,1) |  |
| SO2 | 0.001 | kg | (3,5,3,3,1) |  |
| Water vapor, emissions to air | 7.5 | kg | (3,5,3,3,1) |  |

Table S6 Life cycle inventory of hydrogen production from autothermal reforming without carbon capture

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Electricity | 2.5 | kWh | (3,5,1,2,2) |  |
| Natural gas, high pressure | 4.6 | m3 | (3,5,1,2,2) |  |
| Deionized water | 2.1 | kg | (3,5,1,2,2) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,1,2,2) |  |
| CO2, fossil | 8.9 | kg | (3,5,1,2,2) |  |
| Nitrogen | 12.6 | kg | (3,5,1,2,2) |  |
| SO2 | 0.02 | kg | (3,5,1,2,2) |  |
| Water vapor, emissions to air | 0.005 | kg | (3,5,1,2,2) |  |

Table S7 Life cycle inventory of hydrogen production from autothermal reforming with carbon capture

| Flow name | Value | Unit | Data quality (flows) | Data quality (process) |
| --- | --- | --- | --- | --- |
| Input flows |  |  |  | (1,4) |
| Electricity | 2.4 | kWh | (3,5,1,2,2) |  |
| Natural gas, high pressure | 4.902 | m3 | (3,5,1,2,2) |  |
| Deionized water | 1.9 | kg | (3,5,1,2,2) |  |
| Output flows |  |  |  |  |
| Hydrogen produced | 1 | kg | (3,5,1,2,2) |  |
| CO2, co-product | 8.0 | kg |  |  |
| CO2, fossil | 1.5 | kg | (3,5,1,2,2) |  |
| Nitrogen | 20.2 | kg | (3,5,1,2,2) |  |
| SO2 | 0.003 | kg | (3,5,1,2,2) |  |
| Water vapor, emissions to air | 0.6 | kg | (3,5,1,2,2) |  |