In-cylinder NOx active reduction via ammonia post-injection

in ammonia direct-injection engines

Supplementary Information

1 Experimental method

1.1 Experimental testing method

Table S1 lists the key devices used in experiment. The instantaneous pressure in the test cylinder was recorded by a crank angle resolved sensor (Kistler 6125C) and collected through a charge amplifier (Kistler 5018A1003). The CO, THC, and CO2 concentrations in the exhaust were measured by an exhaust analyzer (Horiba MEXA-7100DEGR). And the NO, NO2, N2O, and unburned NH3 concentrations were measured by an FTIR (Bruker Omega5). The sampling time of FTIR was set to 64 s and FTIR was operated continuously during each measurement period, which means that the pollutant emission values presented in this study are averaged values of at least 500 engine cycles for each operating point.

Table S1. The key devices in the experimental system.

| Name | Type | Measurement uncertainty |
| --- | --- | --- |
| Dynamometer | DW140 | ± 1 r/min |
| In-cylinder pressure sensor | Kistler 6125C | ≤ 0.03 %FSO |
| Charge amplifier | Kistler 5018A1003 | - |
| Photoelectric encoder | Kistler 2614A4 | ± 0.5°CA |
| Air flow meter | E+H T-mass 65F | ± 0.01 kg/h |
| Ammonia booster pump | HiFluid HFG100 | - |
| Ammonia injector | Bosch J6A00-1112100-A38 | - |
| Ammonia flow meter | RHEONIK RHM015 | < 0.05% |
| N-heptane injector | DELPHI 25334150 | - |
| N-heptane consumption meter | Yike FCM-01 | < 1% |
| Exhaust Analyzer | Horiba MEXA-7100DEGR | < 0.5 % |
| FTIR | Bruker Omega5 | < 2 % |

1.2 Data analysis method

Based on the measured in-cylinder pressure, the in-cylinder temperature, heat release rate (HRR), indicated mean effective pressure (IMEP), indicated thermal efficiency (ITE), and coefficient of variation (COV) were calculated. The data analysis methods and parameter definitions are illustrated as follow.

1.2.1 In-cylinder temperature

The in-cylinder temperature refers to the average temperature of the substance inside the cylinder, which is calculated by the gas state equation:

T=pVMR#S1

where p is the in-cylinder pressure, V is the instantaneous volume of the cylinder, M is the moles of in-cylinder substance, and R is the universal gas constant. The moles of in-cylinder substance M are the sum of the moles of air M1 and the moles of combustion products M2:

M=M1+M2=Ma1+0.065X1+γα#S2

where Ma is the moles of in-cylinder substance at the start of compression, γ is the residual gas coefficient, α is the excess air coefficient, and X is the burned fuel proportion at the moment.

1.2.2 Heat release rate (HRR)

The HRR of fuel combustion was calculated based on the measured in-cylinder pressure [1]. The HRR curves presented in this paper are the ensemble-averaged curves of 100 engine cycles at each operating point. According to the energy conservation inside the cylinder, the HRR (dQR/dφ) is the sum of internal energy change rate of the in-cylinder substance, the change rate of pressure work on the piston, and the rate of heat transfer:

dQRdφ=dUdφ+dWdφ+dQwdφ#S3

where QR is the heat released by fuel combustion, U is the internal energy of the in-cylinder working substance, W is the pressure work on the piston, Qw is the heat transfer between in-cylinder substance and chamber wall, and φ is the crank angle.

Within the duration of ∆φ, the above equation can be expressed as:

∆QR=∆U+∆W+∆Qw#S4

∆U, ∆W, and ∆Qw is calculated by:

∆U=M∙cv∙T−Ma∙cv∙Ta#S5

where cv is the average molar specific heat capacity at constant volume, Ta is the in-cylinder temperature at the start of compression.

∆W=p1+p22×V2−V1#S6

where p1 and p2 is the in-cylinder pressure at the beginning and the end of the duration, respectively. V1 and V2 is the cylinder volume at the beginning and the end of the duration, respectively.

∆Qw=∆φ6ni=13αg∙AiT−Twi#S7

where n is the engine speed, αg is the instantaneous average heat transfer coefficient, Ai and Twi are the area and temperature of each chamber wall, respectively. i = 1, 2, 3, which refers head, piston, and cylinder. Currently, the instantaneous average heat transfer coefficient αg is calculated based on the Woschni formula [2].

1.2.3 Indicated mean effective pressure (IMEP)

IMEP is calculated based on the indicated cycle work and the cylinder displacement:

IMEP=Wiπ4∙D2∙S#S8

where D is the cylinder bore, and S is the stroke, and Wi is the indicated cycle work, which is calculated as follows:

Wi=IVCEVOp∙dVdφ∙dφ#S9

There are two calculation methods for Wi and IMEP: calculated by the in-cylinder pressure curve of compression and expansion strokes (from intake valve close to exhaust valve open) and calculated by the in-cylinder pressure curve of entire cycle. In the current study, due to a compressor was used to pressurize the intake air for the test cylinder, the real working state of the engine cannot be reflected. Therefore, the calculation of IMEP and ITE in this paper all use the former method.

1.2.4 Indicated thermal efficiency (ITE)

The indicated thermal efficiency was calculated by dividing the indicated cycle work by the injected fuel energy:

ITE=Wimammonia∙LHVammonia+mn−heptane∙LHVn−heptane#S10

where mammonia and mn−heptane are the injected mass of ammonia and n-heptane, respectively. LHVammonia and LHVn−heptane are the lower heating values of ammonia and n-heptane, respectively.

1.2.5 Coefficient of variation (COV)

The COV in this paper refers to the coefficient of variation of the IMEP during the measured 100 cycles, which is calculated by:

COV=i=1NIMEPi−IMEPmean2NIMEPmean×100%#S11

where N is the total number of cycles, IMEPmean is the average IMEP value for N cycles. COV is an important parameter for evaluating the combustion stability. It is generally considered that the COV of SI engines should not exceed 10%, while that of CI engines should not exceed 5%.

2 Detailed results of chemical kinetics analysis

2.1 Chemical kinetics calculation under constrain volume conditions

Table S2. The calculation methods and model setup.

| Item | Value |
| --- | --- |
| Solver | CHEMKIN |
| Solver type | Constrain volume and solve energy equation |
| Pressure (bar) | 50 |
| Temperature (K) | 1000, 1500, 2000 |
| Equivalence ratio (-) | 0.5, 1.0, 1.5 |
| Reactant mixture | Ammonia/air |

Figure S1. ROP analysis results of NO.

Figure S2. ROP analysis results of NO2.

2.2 Chemical kinetics calculation under constrain pressure and temperature conditions

Table S3. The calculation methods and model setup.

| Item | Value |
| --- | --- |
| Solver | CHEMKIN |
| Solver type | Constrain pressure and temperature |
| Pressure (bar) | 50 |
| Temperature (K) | 1500, 2000 |
| Equivalence ratio (-) | 0.5, 1.0, 1.5 |
| Reactant mixture | Ammonia/air |

Figure S3. ROP analysis results of NO.

Figure S4. ROP analysis results of NO2.

References

- Heywood J.B. Internal Combustion Engine Fundamentals. McGraw-Hill; 1988.

- Woschni G. A Universally Applicable Equation for the Instantaneous Heat Transfer Coefficient in the Internal Combustion Engine, SAE Technical Paper Series, (1967) 670931.