Supplementary information

Table S1 LCI for renewable energy mix

| Renewable energy mix |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Renewable Electricity Mix-NO INVERTER | 1 kWh |  |
| Input | Electricity, low voltage {RoW-ZA water} ¦ electricity production, photovoltaic, 570kWp open ground installation, multi-Si ¦ APOS, U | 0.8 kWh | Modified by removing the inverter, electronic-grade silicon from casted silicon production mix inputs, silicon carbide from silicon wafer production inputs and polystyrene from silicon wafer production inputs. |
|  | Electricity, high voltage {ZA}¦ electricity production, wind, >3MW turbine, onshore ¦ APOS, U | 0.2 kWh | Material of construction replaced with South African material |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 2.48 g | Represent steel recycling process. UNEP (2020), BIR (2020) |
|  | Aluminium scrap, post-consumer, prepared for melting {RoW-}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U | 0.597 g | Represent aluminium recycling process. UNEP (2020) |
|  | Copper scrap, sorted, pressed {RoW}¦ treatment of copper scrap by electrolytic refining ¦ APOS, U | 0.039 g | Represent copper recycling process. UNEP (2020) International Copper Association, (2020). |
|  | Nickel (waste treatment) {GLO}¦ recycling of nickel scrap ¦ APOS, U | 3.96E-6 g | Represent nickel recycling process. UNEP (2020) Reck (2015) |
| Notes: The solar PV in the Ecoinvent database for South Africa was based on a 2009 source with outdated technologies solar PV and was modified as described. |  |  |  |

Table S2 LCI for water purification

| Water purification |  |  | Amount | Modification |
| --- | --- | --- | --- | --- |
| Output | Treated water |  | 9 kg |  |
| Input | Water works, capacity 6.23E10l/year {GLO}¦ water works construction, capacity 6.23E10l/year, seawater reverse osmosis, ultrafiltration pretreatment ¦ APOS, U |  | 4.82E-12 p | Represent SWRO process. Material of construction replaced with South Africa material |
|  | Water, process, salt, ocean |  | 21.4 kg |  |
|  | Renewable Electricity Mix-NO INVERTER |  | 0.0369 kWh |  |
| Emissions to Water | Water, ZA |  | 11.6 l | Ortega Méndez et al 2012, suggest the composition of brine and this was calculated and modelled based on the water rejected/effluent from SWRO. This was added to emissions to water (see below) |
|  | Sodium |  | 232.0746 g |  |
|  | Chloride |  | 437.999 g |  |
|  | Sulphate |  | 68.32586 g |  |
|  | Magnesium |  | 27.69292 g |  |
|  | Calcium |  | 13.05658 g |  |
|  | Potassium (I) |  | 10.9709 g |  |
|  | Bicarbonate, ion |  | 3.43542 g |  |
|  | Silicon dioxide |  | 0.5394 g |  |
|  | Carbonate |  | 0.062 g |  |
|  | Nitrate |  | 0.15376 g |  |
|  | Nitrite |  | 0.00062 g |  |
|  | Ammonium, ion |  | 0.00186 g |  |
|  | Iron (II) |  | 0.00124 g |  |
| Waste treatment | Aluminium scrap, post-consumer, prepared for melting {RoW-}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U |  | 0.003 g | Represent aluminium recycling process UNEP (2020) |
|  | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U |  | 0.00825 g | Represent steel recycling process. UNEP (2020) |

Table S3 LCI for hydrogen production

| Hydrogen production |  |  | Amount | Modification |
| --- | --- | --- | --- | --- |
| Output | Hydrogen gas |  | 1 kg |  |
|  | Oxygen gas |  | 8 kg |  |
| Input | Treated water |  | 9 kg |  |
|  | Electrolyser |  |  |  |
|  | Consisting of: |  |  | Electrolyser unit was modelled according to Zhao et al. (2020) that provides the raw materials to construct a 1 m2 electrolyser with a production of 19 tonne/m2 H2 over its lifetime of 15 years. |
|  |  | Nickel, class 1 {ZA}¦ platinum group metal, extraction and refinery operations ¦ APOS, U | 0.366 kg |  |
|  |  | Steel, stainless 304, flat rolled coil/kg/RNA | 0.123 kg |  |
|  |  | Zirconium oxide {GLO}¦ market for ¦ APOS, U | 5.01×10-3 kg |  |
|  |  | Polysulfone {GLO}¦ market for ¦ APOS, U | 8.8×10-4 kg |  |
|  |  | Electricity, medium voltage {ZA}¦ market for ¦ APOS, U | 7.26×10-5 KWh |  |
|  |  | Polyphenylene sulfide {GLO}¦ market for ¦ APOS, U | 0.102 kg |  |
|  | Hydrogen, liquid {RER}¦ potassium hydroxide production ¦ APOS, U |  | 3.7E-5 kg | None |
|  | Chemical factory, organics {RoW}¦ construction ¦ APOS, U |  | 4.0E-10 p | Represent the building that house the electrolyser. Material of construction replaced with South Africa material |
|  | Transport, freight, lorry 7.5-16 metric ton, EURO2 {ZA}¦ market for transport, freight, lorry 7.5-16 metric ton, EURO2 ¦ APOS, U |  | 0.0207 tkm | Diesel used replaced with South Africa diesel. |
|  | Renewable Electricity Mix-NO INVERTER |  | 49.11 kWh |  |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U |  | 2.99 g | Represent aluminium recycling process UNEP (2020) BIR (2019). |
|  | Aluminium scrap, post-consumer, prepared for melting {RoW}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U |  | 0.353 g | Represent aluminium recycling process UNEP (2020) |
|  | Copper scrap, sorted, pressed {RoW}¦ treatment of copper scrap by electrolytic refining ¦ APOS, U |  | 0.12 g | Represent copper recycling process. UNEP (2020) International Copper Association (2020) |
|  | Nickel (waste treatment) {GLO}¦ recycling of nickel scrap ¦ APOS, U |  | 0.000115 g | Represent nickel recycling process. UNEP (2020) Reck (2015) |

Table S4 LCI for hydrogen storage

| Hydrogen storage |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Hydrogen gas | 1 kg |  |
| Input | Liquid storage tank, chemicals, organics {RoW}¦ production //Kolobe//¦ APOS, U | 7.74E-12 | Material of construction replaced with South African material |
|  | Renewable Electricity Mix-NO INVERTER | 1.7 kWh |  |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 0.0996 g | Represent steel recycling process. UNEP (2020), BIR (2019) |
|  | Copper scrap, sorted, pressed {RoW}¦ treatment of copper scrap by electrolytic refining ¦ APOS, U | 0.000284 g | Represent copper recycling process. UNEP (2020) International Copper Association (2020) |
|  | Aluminium scrap, post-consumer, prepared for melting {RoW}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U | 0.000823 g | Represent aluminium recycling process UNEP (2020) |
| Note: 14 days of storage and 0.1% hydrogen loss per day (Andersen and Grönkvist 2019) |  |  |  |

Table S5 LCI for nitrogen production

| Nitrogen production |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Nitrogen liquid | 0.234 kg |  |
| Input | Argon-40/kg | 0.00985 kg |  |
|  | Nitrogen, atmospheric | 0.531 kg |  |
|  | Oxygen | 0.163 kg |  |
|  | Water, cooling, unspecified natural origin, RoW | 0.0214 kg |  |
|  | Air separation facility {GLO}¦ market for ¦ APOS, U | 4.432E-10 p | Material of construction replaced with South Africa material (Aluminium) |
|  | Renewable Electricity Mix-NO INVERTER | 0.563 kWh |  |
| Emissions to air | Nitrogen, atmospheric | 0.296 kg | Added to emissions to air |
|  | Argon-40/kg | 5.5E-5 kg | Added to emissions to air |
|  | Oxygen | 0.162 kg | Added to emissions to air |
|  | Water/m3 | 0.00828 M3 | None |
| Emissions to Water | Water, RoW | 0.0131 M3 | None |
| Waste treatment | Aluminium scrap, post-consumer, prepared for melting {RoW}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U | 0.00286 g | Represent aluminium recycling process. UNEP (2020) |
|  | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 0.0201 g | Represent steel recycling process. UNEP (2020), BIR (2020) |  |
| Notes: Oxygen and argon were added to emissions to air. 2% w/w argon was added to the nitrogen stream. Electricity supply to ASU was replaced with on-site renewable |  |  |  |  |

Table 6 LCI for nitrogen storage

| Liquid nitrogen storage |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Liquid nitrogen stored | 4.6 kg |  |
| Input | Liquid nitrogen | 4.6 kg |  |
|  | Liquid storage tank, chemicals, organics {RoW}¦ production ¦ APOS, | 7.11E-9 p | Material of construction replaced with South African material |
|  | Renewable Electricity Mix-NO INVERTER | 0.431 kWh |  |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 2.25 g | Represent steel recycling process. UNEP (2020), BIR (2020) |

Table S8 LCI for ammonia production

| Ammonia production |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Liquid ammonia | 5.3 kg |  |
| Input | Chemical factory, organics {RoW}¦ construction ¦ APOS, U | 2.12E-9 p | Electricity supply was replaced with on-site renewable energy |
|  | Aluminium oxide, metallurgical {RoW}¦ market for aluminium oxide, metallurgical ¦ APOS, U | 1.422E-9 kg | None |
|  | Hydrogen, liquid {RER}¦ potassium hydroxide production ¦ APOS, U | 4.74E-10 kg | None |
|  | Magnetite {GLO}¦ market for ¦ APOS, U | 4.55E-8 kg | None |
|  | Nitrogen gas | 4.6 kg |  |
|  | Hydrogen gas | 1 kg |  |
|  | Renewable Electricity Mix-NO INVERTER | 2.33 kWh |  |
| Emissions to air | Nitrogen, atmospheric | 0.178 kg | None |
|  | Argon-40/kg | 0.178 kg | None |
|  | Hydrogen | 0.05 kg | None |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 14.2 g | Represent steel recycling process. UNEP (2020) BIR (2019) |
|  | Copper scrap, sorted, pressed {RoW}¦ treatment of copper scrap by electrolytic refining ¦ APOS, U | 0.568 g | Represent copper recycling process. UNEP (2020) International Copper Association, (2020). |
|  | Aluminium scrap, post-consumer, prepared for melting {RoW}¦ treatment of aluminium scrap, post-consumer, prepared for recycling, at remelter ¦ APOS, U | 1.68 g | Represent aluminium recycling process. UNEP (2020) |
| Notes: Chemical factory, for the ammonia synthesis (Haber-Bosch) includes land use, building and facilities (including dismantling) of an average chemical plant. It represents the building and reactor needed to produce ammonia. The chemical factory has an annual production volume of 50E-6 kg of product. and a 50-year lifespan. |  |  |  |

Table S7 LCI for ammonia storage

| Liquid ammonia storage |  | Amount | Modification |
| --- | --- | --- | --- |
| Output | Liquid ammonia stored | 5.3 kg |  |
| Input | Liquid ammonia | 5.3 kg |  |
|  | Liquid storage tank, chemicals, organics {RoW}¦ production ¦ APOS, U | 10E-9 P | Material of construction replaced with South African material |
|  | Renewable Electricity Mix-NO INVERTER | 0.000376 kWh |  |
| Waste treatment | Steel and iron (waste treatment) {GLO}¦ recycling of steel and iron by (EAF) ¦ APOS, U | 2.74 g | Represent aluminium recycling process BIR (2019). UNEP (2020) |