## Supplementary Instruction

## S1: Detailed Experimental Data Description

MIT Dataset (Controlled Measurement Conditions): The MIT dataset comprises 124 LFP 18650 cylindrical cells. These cells were subjected to a constant-current (CC) discharge process at 30 °C. The highly controlled and ideal measurement environment of this dataset provides a reliable baseline for validation, enabling a precise assessment of the fundamental performance metrics under stable conditions.

CALCE Dataset (Periodic Measurement Scenarios): The CALCE dataset utilizes a CX2-3 prismatic LCO cell, which undergoes periodic pulse discharge interspersed with rest periods. This testing protocol closely mimicked the operational conditions of urban electric vehicles, where batteries experience repetitive charge-discharge cycles with intermittent idle periods. As such, the dataset effectively represents the performance characteristics of batteries during cyclic operations in real-world urban transportation scenarios.

Oxford Dataset (Complex Dynamic Measurement): The Oxford dataset consists of six LCO pouch cells tested at 40°C. The cells were discharged following Artemis dynamic profiles that simulate urban, rural, and highway driving conditions. Characterized by high-frequency variations, this challenging measurement environment poses significant tests to the proposed methodology, allowing for a comprehensive evaluation of its adaptability and accuracy in handling complex, dynamic real-world situations. Table S1 summarizes the material systems, types, and specifications of the tested batteries.

Table S1. Specification of the tested datasets

| Dataset | Type | Battery shape | Normal capacity | Voltage range |
| --- | --- | --- | --- | --- |
| MIT | LFP | Cylindrical | 1.1Ah | 2.0-3.6 V |
| CALCE | LCO | Rectangular | 1.35Ah | 2.7-4.2 V |
| Oxford | LCO | Pouch | 0.75Ah | 2.7-4.2 V |

## S2: Parameter Sensitivity Analysis of Segmentation Parameter Np

To validate the rationality of the segmentation parameter Np = 30 used in the main text and clarify its impact on the extraction of health features K and b and their relationship with SOH estimation performance, a parameter sensitivity analysis was conducted using the MIT dataset as an example. The MIT dataset was selected due to its well - controlled measurement environment, which helps accurately quantify the relationship between Np and the correlation of features K and b with SOH.

Four candidate values of Np (20, 25, 30, and 40) were selected. The features K and b were reextracted following the method described in Section 3.2 of the main text, and the Pearson correlation coefficients between these features and the actual SOH were calculated. The correlation coefficients of features K and b with SOH under different Np values are shown in Table S2 and Figure S1.

Table S2. The Pearson correlation coefficients for different segmentation

| Np | K | b |
| --- | --- | --- |
| 20 | 0.899 | 0.31 |
| 25 | 0.974 | - 0.985 |
| 30 | 0.986 | - 0.951 |
| 40 | 0.986 | - 0.968 |

Fig. S1 The Pearson correlation coefficients between features K, b and SOH for different Np values

## S3: Network Architecture and Training Details

### S3.1 Complete Network Mathematical Formulations

TCN Branch Detailed Implementation:

Residual block structure:

Receptive field calculation:

where k=5 is convolutional kernel size and N=3 is number of TCN blocks.

Transformer Branch Complete Formulation:

Positional encoding:

Input processing:

Multi-head attention mechanism:

Feed-forward network:

Transformer layer computation:

Complete Attention-Gated Fusion:

Gating mechanism:

Cross-attention computation:

Final fusion:

Output processing:

where GAP denotes global average pooling.

### S3.2 Complete Network Mathematical Formulations

Table S3. Network architecture parameters

| Component | Parameter | Value | Description |
| --- | --- | --- | --- |
| TCN Branch | Number of blocks | 3 | Cascaded residual blocks |
|  | Kernel size | 5 | 1D convolution kernel |
|  | Dilation rates | [1, 2, 4] | Exponential dilation |
|  | Filters | [64, 64, 64] | Feature channels |
|  | Dropout rate | 0.2 | Regularization |
| Transformer | Model dimension | 64 | Hidden dimension |
|  | Number of heads | 8 | Multi-head attention |
|  | Number of layers | 2 | Encoder layers |
|  | Feed-forward dim | 256 | FFN dimension |
|  | Dropout rate | 0.1 | Attention dropout |
| Fusion Module | Gate conv filters | [32, 16] | Gating mechanism |
|  | Cross-attention heads | 4 | Bidirectional attention |
|  | MLP dimensions | [128, 64] | Fusion processing |
| Output Layer | Dimensions | [128, 64, 1] | Final prediction |
|  | Activation | [ReLU, ReLU, Linear] | Layer activations |

### S3.3 Training Optimization Parameters

Table S4. Training configuration

| Parameter | Value | Description |
| --- | --- | --- |
| Batch size | 32 | Mini-batch size |
| Learning rate | 0.001 | Initial learning rate |
| Optimizer | Adam | Optimization algorithm |
| Weight decay | 1e-5 | L2 regularization |
| L1 regularization | 1e-6 | Sparsity penalty |
| Max epochs | 200 | Training iterations |
| Early stopping | 20 | Patience epochs |
| Learning rate decay | 0.5 | Decay factor |
| Decay patience | 10 | Plateau patience |