Appendices for:

Machine learning-enhanced metaheuristic optimization of lead rubber bearings for inter-story isolated buildings under seismic load

Sukamal Kanta GHOSH*, Indrajeet KASHYAP, Diptesh DAS

Department of Civil Engineering, National Institute of Technology Durgapur, Durgapur 713209, India

*Corresponding Author. Email address: sukamalkantaghosh@gmail.com

Appendix A

Table A1. Important characteristics of the selected ground motion records

| RSN | Event | Year | Station | Component | Mw | Site class | Fault mechanism | Rjb (km) | PGA (g) | PGV (cm/s) | PGD (cm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Imperial Valley-02 | 1940 | El Centro Array #9 | 180 | 6.95 | D | strike slip | 6.09 | 0.281 | 30.92 | 8.66 |
| 15 | Kern County | 1952 | Taft Lincoln School | 111 | 7.36 | C | reverse | 38.42 | 0.180 | 18.62 | 9.35 |
| 28 | Parkfield | 1966 | Cholame-Shandon Array #12 | 320 | 6.19 | C | strike slip | 17.64 | 0.063 | 7.34 | 4.26 |
| 77 | San Fernando | 1971 | Pacoima Dam (upper left abut) | 164 | 6.61 | A | reverse | 0 | 1.219 | 114.41 | 39.00 |
| 81 | San Fernando | 1971 | Pearblossom Pump | 0 | 6.61 | C | reverse | 35.54 | 0.104 | 4.95 | 1.26 |
| 87 | San Fernando | 1971 | Santa Anita Dam | 3 | 6.61 | C | reverse | 30.7 | 0.155 | 4.71 | 0.88 |
| 140 | Tabas, Iran | 1978 | Ferdows | T | 7.35 | D | reverse | 89.76 | 0.105 | 7.13 | 4.38 |
| 143 | Tabas, Iran | 1978 | Tabas | T | 7.35 | B | reverse | 1.79 | 0.862 | 123.34 | 93.56 |
| 148 | Coyote Lake | 1979 | Gilroy Array #3 | 50 | 5.74 | D | strike slip | 6.75 | 0.252 | 17.62 | 4.71 |
| 149 | Coyote Lake | 1979 | Gilroy Array #4 | 360 | 5.74 | D | strike slip | 4.79 | 0.252 | 31.82 | 5.24 |
| 150 | Coyote Lake | 1979 | Gilroy Array #6 | 320 | 5.74 | C | strike slip | 0.42 | 0.319 | 25.38 | 4.38 |
| 158 | Imperial Valley-06 | 1979 | Aeropuerto Mexicali | 315 | 6.53 | D | strike slip | 0 | 0.271 | 24.19 | 3.71 |
| 160 | Imperial Valley-06 | 1979 | Bonds Corner | 230 | 6.53 | D | strike slip | 0.44 | 0.777 | 44.92 | 15.09 |
| 161 | Imperial Valley-06 | 1979 | Brawley Airport | 315 | 6.53 | D | strike slip | 8.54 | 0.220 | 40.92 | 16.24 |
| 166 | Imperial Valley-06 | 1979 | Coachella Canal #4 | 45 | 6.53 | D | strike slip | 49.1 | 0.116 | 12.83 | 2.58 |
| 170 | Imperial Valley-06 | 1979 | EC County Center FF | 2 | 6.53 | D | strike slip | 7.31 | 0.212 | 38.42 | 16.98 |
| 171 | Imperial Valley-06 | 1979 | El Centro-Meloland Geot. Array | 0 | 6.53 | D | strike slip | 0.07 | 0.317 | 72.91 | 34.90 |
| 173 | Imperial Valley-06 | 1979 | El Centro Array #10 | 50 | 6.53 | D | strike slip | 8.6 | 0.173 | 50.66 | 35.38 |
| 175 | Imperial Valley-06 | 1979 | El Centro Array #12 | 230 | 6.53 | D | strike slip | 17.94 | 0.118 | 22.98 | 13.34 |
| 180 | Imperial Valley-06 | 1979 | El Centro Array #5 | 140 | 6.53 | D | strike slip | 1.76 | 0.529 | 48.89 | 48.86 |
| 181 | Imperial Valley-06 | 1979 | El Centro Array #6 | 230 | 6.53 | D | strike slip | 0 | 0.449 | 113.50 | 72.85 |
| 182 | Imperial Valley-06 | 1979 | El Centro Array #7 | 140 | 6.53 | D | strike slip | 0.56 | 0.341 | 51.65 | 27.98 |
| 183 | Imperial Valley-06 | 1979 | El Centro Array #8 | 230 | 6.53 | D | strike slip | 3.86 | 0.610 | 54.46 | 41.75 |
| 185 | Imperial Valley-06 | 1979 | Holtville Post Office | 315 | 6.53 | D | strike slip | 5.35 | 0.221 | 51.43 | 35.81 |
| 285 | Irpinia, Italy-01 | 1980 | Bagnoli Irpinio | 270 | 6.9 | C | normal | 8.14 | 0.190 | 34.69 | 13.74 |
| 338 | Coalinga-01 | 1983 | Parkfield-Fault Zone 14 | 0 | 6.36 | D | reverse | 28.11 | 0.262 | 34.90 | 8.25 |
| 369 | Coalinga-01 | 1983 | Slack Canyon | 45 | 6.36 | C | reverse | 25.98 | 0.174 | 16.21 | 4.17 |
| 451 | Morgan Hill | 1984 | Coyote Lake Dam-Southwest Abutment | 230 | 6.19 | C | strike slip | 0.18 | 0.713 | 52.88 | 12.75 |
| 457 | Morgan Hill | 1984 | Gilroy Array #3 | 90 | 6.19 | D | strike slip | 13.01 | 0.201 | 13.30 | 3.66 |
| 459 | Morgan Hill | 1984 | Gilroy Array #6 | 90 | 6.19 | C | strike slip | 9.85 | 0.292 | 36.47 | 5.94 |
| 499 | Hollister-04 | 1986 | Hollister Differential Array #3 | 345 | 5.45 | D | strike slip | 13.11 | 0.095 | 9.46 | 2.93 |
| 517 | N. Palm Springs | 1986 | Desert Hot Springs | 90 | 6.06 | D | reverse oblique | 0.99 | 0.271 | 16.17 | 3.10 |
| 529 | N. Palm Springs | 1986 | North Palm Springs | 300 | 6.06 | D | reverse oblique | 0 | 0.668 | 27.92 | 4.91 |
| 549 | Chalfant Valley-02 | 1986 | Bishop-LADWP South St | 270 | 6.19 | D | strike slip | 14.38 | 0.176 | 19.53 | 7.09 |
| 566 | Kalamata, Greece-02 | 1986 | Kalamata (bsmt) (2nd trigger) | NS | 5.4 | C | normal | 4 | 0.161 | 12.78 | 1.31 |
| 568 | San Salvador | 1986 | Geotech Investig Center | 180 | 5.8 | C | strike slip | 2.14 | 0.421 | 62.26 | 13.08 |
| 605 | Whittier Narrows-01 | 1987 | Canyon Country-W Lost Cany | 270 | 5.99 | D | reverse oblique | 44.88 | 0.100 | 7.18 | 0.77 |
| 719 | Superstition Hills-02 | 1987 | Brawley Airport | 225 | 6.54 | D | strike slip | 17.03 | 0.144 | 9.72 | 2.85 |
| 723 | Superstition Hills-02 | 1987 | Parachute Test Site | 315 | 6.54 | D | strike slip | 0.95 | 0.384 | 53.03 | 17.81 |
| 735 | Loma Prieta | 1989 | APEEL 7-Pulgas | 0 | 6.93 | C | reverse oblique | 41.68 | 0.157 | 15.65 | 8.26 |
| 741 | Loma Prieta | 1989 | BRAN | 90 | 6.93 | C | reverse oblique | 3.85 | 0.502 | 44.47 | 5.05 |
| 752 | Loma Prieta | 1989 | Capitola | 90 | 6.93 | D | reverse oblique | 8.65 | 0.439 | 29.60 | 4.91 |
| 753 | Loma Prieta | 1989 | Corralitos | 0 | 6.93 | C | reverse oblique | 0.16 | 0.645 | 55.94 | 9.44 |
| 755 | Loma Prieta | 1989 | Coyote Lake Dam-Southwest Abutment | 195 | 6.93 | C | reverse oblique | 19.97 | 0.152 | 15.75 | 5.77 |
| 762 | Loma Prieta | 1989 | Fremont-Mission San Jose | 90 | 6.93 | C | reverse oblique | 39.32 | 0.106 | 9.64 | 5.46 |
| 763 | Loma Prieta | 1989 | Gilroy-Gavilan Coll. | 67 | 6.93 | C | reverse oblique | 9.19 | 0.359 | 31.07 | 10.91 |
| 765 | Loma Prieta | 1989 | Gilroy Array #1 | 90 | 6.93 | B | reverse oblique | 8.84 | 0.485 | 32.43 | 15.65 |
| 766 | Loma Prieta | 1989 | Gilroy Array #2 | 90 | 6.93 | D | reverse oblique | 10.38 | 0.323 | 40.35 | 18.45 |
| 768 | Loma Prieta | 1989 | Gilroy Array #4 | 0 | 6.93 | D | reverse oblique | 13.81 | 0.419 | 40.12 | 7.98 |
| 787 | Loma Prieta | 1989 | Palo Alto-SLAC Lab | 360 | 6.93 | C | reverse oblique | 30.62 | 0.277 | 31.27 | 11.51 |
| 790 | Loma Prieta | 1989 | Richmond City Hall | 280 | 6.93 | D | reverse oblique | 87.78 | 0.106 | 14.82 | 4.71 |
| 802 | Loma Prieta | 1989 | Saratoga-Aloha Ave | 0 | 6.93 | C | reverse oblique | 7.58 | 0.514 | 41.56 | 16.32 |
| 803 | Loma Prieta | 1989 | Saratoga-W Valley Coll. | 0 | 6.93 | D | reverse oblique | 8.48 | 0.257 | 42.04 | 24.26 |
| 807 | Loma Prieta | 1989 | Sunol-Forest Fire Station | 90 | 6.93 | C | reverse oblique | 47.41 | 0.084 | 9.08 | 4.14 |
| 821 | Erzican, Turkey | 1992 | Erzincan | EW | 6.69 | D | strike slip | 0 | 0.496 | 78.12 | 28.02 |
| 828 | Cape Mendocino | 1992 | Petrolia | 90 | 7.01 | C | reverse | 0 | 0.662 | 88.47 | 33.20 |
| 832 | Landers | 1992 | Amboy | 0 | 7.28 | C | strike slip | 69.21 | 0.115 | 18.20 | 10.51 |
| 836 | Landers | 1992 | Baker Fire Station | 140 | 7.28 | D | strike slip | 87.94 | 0.106 | 10.96 | 7.96 |
| 838 | Landers | 1992 | Barstow | 0 | 7.28 | C | strike slip | 34.86 | 0.130 | 20.44 | 14.64 |
| 848 | Landers | 1992 | Coolwater | LN | 7.28 | D | strike slip | 19.74 | 0.284 | 27.60 | 18.20 |
| 864 | Landers | 1992 | Joshua Tree | 0 | 7.28 | C | strike slip | 11.03 | 0.645 | 55.94 | 9.44 |
| 879 | Landers | 1992 | Lucerne | 345 | 7.28 | B | strike slip | 2.19 | 0.789 | 28.09 | 25.52 |
| 882 | Landers | 1992 | North Palm Springs | 90 | 7.28 | D | strike slip | 26.84 | 0.134 | 14.52 | 5.70 |
| 900 | Landers | 1992 | Yermo Fire Station | 360 | 7.28 | D | strike slip | 23.62 | 0.152 | 29.08 | 23.13 |
| 949 | Northridge-01 | 1994 | Arleta-Nordhoff Fire Sta | 360 | 6.69 | D | reverse | 3.3 | 0.308 | 22.98 | 8.86 |
| 959 | Northridge-01 | 1994 | Canoga Park-Topanga Can | 106 | 6.69 | D | reverse | 0 | 0.358 | 33.77 | 8.38 |
| 960 | Northridge-01 | 1994 | Canyon Country-W Lost Cany | 0 | 6.69 | D | reverse | 11.39 | 0.404 | 44.36 | 11.26 |
| 968 | Northridge-01 | 1994 | Downey-Co Maint Bldg | 90 | 6.69 | D | reverse | 43.2 | 0.158 | 13.64 | 2.24 |
| 982 | Northridge-01 | 1994 | Jensen Filter Plant Administrative Building | 22 | 6.69 | C | reverse | 0 | 0.411 | 111.41 | 44.61 |
| 983 | Northridge-01 | 1994 | Jensen Filter Plant Generator Building | 292 | 6.69 | C | reverse | 0 | 0.995 | 67.31 | 24.32 |
| 1004 | Northridge-01 | 1994 | LA-Sepulveda VA Hospital | 270 | 6.69 | C | reverse | 0 | 0.753 | 77.63 | 11.84 |
| 1013 | Northridge-01 | 1994 | LA Dam | 334 | 6.69 | C | reverse | 0 | 0.324 | 47.35 | 24.56 |
| 1019 | Northridge-01 | 1994 | Lake Hughes #1 | 90 | 6.69 | C | reverse | 35.46 | 0.077 | 9.45 | 2.33 |
| 1028 | Northridge-01 | 1994 | Leona Valley #2 | 0 | 6.69 | C | reverse | 36.9 | 0.091 | 7.59 | 1.75 |
| 1031 | Northridge-01 | 1994 | Leona Valley #5-Ritter | 90 | 6.69 | C | reverse | 37.47 | 0.092 | 10.43 | 2.58 |
| 1032 | Northridge-01 | 1994 | Leona Valley #6 | 90 | 6.69 | D | reverse | 37.7 | 0.178 | 14.34 | 1.95 |
| 1042 | Northridge-01 | 1994 | N Hollywood-Coldwater Can | 180 | 6.69 | D | reverse | 7.89 | 0.309 | 21.44 | 9.02 |
| 1044 | Northridge-01 | 1994 | Newhall-Fire Sta | 360 | 6.69 | D | reverse | 3.16 | 0.590 | 96.54 | 34.32 |
| 1045 | Northridge-01 | 1994 | Newhall-W Pico Canyon Rd. | 316 | 6.69 | D | reverse | 2.11 | 0.357 | 59.20 | 16.03 |
| 1050 | Northridge-01 | 1994 | Pacoima Dam (downstr) | 175 | 6.69 | A | reverse | 4.92 | 0.416 | 44.26 | 5.01 |
| 1051 | Northridge-01 | 1994 | Pacoima Dam (upper left) | 194 | 6.69 | A | reverse | 4.92 | 1.285 | 103.33 | 22.17 |
| 1052 | Northridge-01 | 1994 | Pacoima Kagel Canyon | 90 | 6.69 | C | reverse | 5.26 | 0.301 | 30.79 | 10.78 |
| 1054 | Northridge-01 | 1994 | Pardee-SCE | L | 6.69 | D | reverse | 5.54 | 0.557 | 76.03 | 14.44 |
| 1055 | Northridge-01 | 1994 | Pasadena-N Sierra Madre | 180 | 6.69 | C | reverse | 35.77 | 0.263 | 14.09 | 1.42 |
| 1062 | Northridge-01 | 1994 | Rancho Palos Verdes-Luconia | 95 | 6.69 | C | reverse | 50.47 | 0.125 | 7.27 | 1.24 |
| 1063 | Northridge-01 | 1994 | Rinaldi Receiving Sta | 228 | 6.69 | D | reverse | 0 | 0.874 | 147.92 | 41.85 |
| 1070 | Northridge-01 | 1994 | San Gabriel-E Grand Ave | 270 | 6.69 | C | reverse | 38.86 | 0.257 | 11.77 | 2.27 |
| 1080 | Northridge-01 | 1994 | Simi Valley-Katherine Rd | 0 | 6.69 | C | reverse | 0 | 0.804 | 50.23 | 6.26 |
| 1082 | Northridge-01 | 1994 | Sun Valley-Roscoe Blvd | 90 | 6.69 | D | reverse | 5.59 | 0.447 | 42.73 | 9.09 |
| 1083 | Northridge-01 | 1994 | Sunland-Mt Gleason Ave | 170 | 6.69 | C | reverse | 12.38 | 0.133 | 15.72 | 4.25 |
| 1084 | Northridge-01 | 1994 | Sylmar-Converter Sta | 142 | 6.69 | D | reverse | 0 | 0.923 | 88.48 | 22.18 |
| 1085 | Northridge-01 | 1994 | Sylmar-Converter Sta East | 281 | 6.69 | C | reverse | 0 | 0.449 | 60.38 | 18.79 |
| 1086 | Northridge-01 | 1994 | Sylmar-Olive View Med FF | 360 | 6.69 | C | reverse | 1.74 | 0.843 | 129.31 | 32.11 |
| 1100 | Kobe, Japan | 1995 | Abeno | 0 | 6.9 | D | strike slip | 24.85 | 0.221 | 21.24 | 7.97 |
| 1106 | Kobe, Japan | 1995 | KJMA | 0 | 6.9 | D | strike slip | 0.94 | 0.834 | 91.06 | 21.07 |
| 1114 | Kobe, Japan | 1995 | Port Island (0 m) | 0 | 6.9 | D | strike slip | 3.31 | 0.348 | 90.63 | 39.29 |
| 1119 | Kobe, Japan | 1995 | Takarazuka | 0 | 6.9 | D | strike slip | 0 | 0.614 | 86.21 | 20.82 |
| 1120 | Kobe, Japan | 1995 | Takatori | 90 | 6.9 | D | strike slip | 1.46 | 0.671 | 122.90 | 29.60 |
| 1158 | Kocaeli, Turkey | 1999 | Duzce | 180 | 7.51 | D | strike slip | 13.6 | 0.312 | 58.84 | 44.04 |
| 1165 | Kocaeli, Turkey | 1999 | Izmit | 180 | 7.51 | B | strike slip | 3.62 | 0.165 | 22.32 | 11.84 |
| 1176 | Kocaeli, Turkey | 1999 | Yarimca | 60 | 7.51 | D | strike slip | 1.38 | 0.227 | 69.68 | 62.29 |
| 1193 | Chi-Chi, Taiwan | 1999 | CHY024 | N | 7.62 | C | reverse oblique | 9.62 | 0.165 | 43.50 | 29.81 |
| 1197 | Chi-Chi, Taiwan | 1999 | CHY028 | N | 7.62 | C | reverse oblique | 3.12 | 0.760 | 85.71 | 27.87 |
| 1198 | Chi-Chi, Taiwan | 1999 | CHY029 | E | 7.62 | C | reverse oblique | 10.96 | 0.289 | 35.24 | 11.11 |
| 1244 | Chi-Chi, Taiwan | 1999 | CHY101 | N | 7.62 | D | reverse oblique | 9.94 | 0.398 | 109.16 | 74.02 |
| 1481 | Chi-Chi, Taiwan | 1999 | TCU038 | N | 7.62 | D | reverse oblique | 25.42 | 0.144 | 38.75 | 44.37 |
| 1483 | Chi-Chi, Taiwan | 1999 | TCU040 | E | 7.62 | C | reverse oblique | 22.06 | 0.162 | 56.78 | 44.03 |
| 1489 | Chi-Chi, Taiwan | 1999 | TCU049 | N | 7.62 | C | reverse oblique | 3.76 | 0.244 | 62.28 | 49.45 |
| 1491 | Chi-Chi, Taiwan | 1999 | TCU051 | E | 7.62 | D | reverse oblique | 7.64 | 0.160 | 53.81 | 73.85 |
| 1492 | Chi-Chi, Taiwan | 1999 | TCU052 | N | 7.62 | C | reverse oblique | 0 | 0.358 | 151.13 | 210.33 |
| 1493 | Chi-Chi, Taiwan | 1999 | TCU053 | N | 7.62 | C | reverse oblique | 5.95 | 0.229 | 39.58 | 64.98 |
| 1494 | Chi-Chi, Taiwan | 1999 | TCU054 | E | 7.62 | C | reverse oblique | 5.28 | 0.146 | 46.00 | 81.92 |
| 1496 | Chi-Chi, Taiwan | 1999 | TCU056 | E | 7.62 | C | reverse oblique | 10.48 | 0.156 | 42.86 | 54.89 |
| 1501 | Chi-Chi, Taiwan | 1999 | TCU063 | E | 7.62 | C | reverse oblique | 9.78 | 0.182 | 43.96 | 43.66 |
| 1503 | Chi-Chi, Taiwan | 1999 | TCU065 | E | 7.62 | D | reverse oblique | 0.57 | 0.790 | 125.28 | 108.67 |
| 1505 | Chi-Chi, Taiwan | 1999 | TCU068 | N | 7.62 | C | reverse oblique | 0 | 0.371 | 263.97 | 421.46 |
| 1507 | Chi-Chi, Taiwan | 1999 | TCU071 | E | 7.62 | C | reverse oblique | 0 | 0.529 | 52.28 | 16.05 |
| 1510 | Chi-Chi, Taiwan | 1999 | TCU075 | E | 7.62 | C | reverse oblique | 0.89 | 0.332 | 109.50 | 96.56 |
| 1511 | Chi-Chi, Taiwan | 1999 | TCU076 | E | 7.62 | C | reverse oblique | 2.74 | 0.344 | 51.82 | 33.26 |
| 1512 | Chi-Chi, Taiwan | 1999 | TCU078 | N | 7.62 | C | reverse oblique | 0 | 0.307 | 31.62 | 7.49 |
| 1513 | Chi-Chi, Taiwan | 1999 | TCU079 | E | 7.62 | C | reverse oblique | 0 | 0.592 | 70.50 | 7.54 |
| 1515 | Chi-Chi, Taiwan | 1999 | TCU082 | N | 7.62 | C | reverse oblique | 5.16 | 0.186 | 40.74 | 44.23 |
| 1519 | Chi-Chi, Taiwan | 1999 | TCU087 | N | 7.62 | C | reverse oblique | 6.98 | 0.114 | 40.46 | 34.06 |
| 1521 | Chi-Chi, Taiwan | 1999 | TCU089 | N | 7.62 | C | reverse oblique | 0 | 0.229 | 33.13 | 25.81 |
| 1528 | Chi-Chi, Taiwan | 1999 | TCU101 | N | 7.62 | C | reverse oblique | 2.11 | 0.258 | 50.95 | 35.94 |
| 1529 | Chi-Chi, Taiwan | 1999 | TCU102 | E | 7.62 | C | reverse oblique | 1.49 | 0.304 | 91.67 | 104.48 |
| 1530 | Chi-Chi, Taiwan | 1999 | TCU103 | E | 7.62 | C | reverse oblique | 6.08 | 0.129 | 70.22 | 68.35 |
| 1548 | Chi-Chi, Taiwan | 1999 | TCU128 | N | 7.62 | C | reverse oblique | 13.13 | 0.167 | 62.61 | 52.27 |
| 1550 | Chi-Chi, Taiwan | 1999 | TCU136 | W | 7.62 | C | reverse oblique | 8.27 | 0.170 | 45.42 | 60.10 |
| 1602 | Duzce, Turkey | 1999 | Bolu | 0 | 7.14 | D | strike slip | 12.02 | 0.739 | 55.91 | 25.57 |
| 1614 | Duzce, Turkey | 1999 | Lamont 1061 | N | 7.14 | C | strike slip | 11.46 | 0.101 | 11.19 | 7.67 |
| 1633 | Manjil, Iran | 1990 | Abbar | T | 7.37 | C | strike slip | 12.55 | 0.497 | 50.56 | 23.79 |
| 1787 | Hector Mine | 1999 | Hector | 90 | 7.13 | C | strike slip | 10.35 | 0.328 | 44.75 | 10.69 |
| 3317 | Chi-Chi, Taiwan-06 | 1999 | CHY101 | N | 6.3 | D | reverse | 34.55 | 0.147 | 20.44 | 11.34 |
| 3548 | Loma Prieta | 1989 | Los Gatos-Lexington Dam | 90 | 6.93 | B | reverse oblique | 3.22 | 0.411 | 95.73 | 30.27 |
| 3750 | Cape Mendocino | 1992 | Loleta Fire Station | 270 | 7.01 | C | reverse | 23.46 | 0.265 | 35.51 | 33.89 |
| 3757 | Landers | 1992 | North Palm Springs Fire Sta #36 | 90 | 7.28 | C | strike slip | 26.95 | 0.139 | 14.60 | 7.15 |
| 3759 | Landers | 1992 | Whitewater Trout Farm | 180 | 7.28 | C | strike slip | 27.05 | 0.123 | 9.77 | 1.58 |
| 4040 | Bam, Iran | 2003 | Bam | L | 6.6 | C | strike slip | 0.05 | 0.808 | 124.06 | 33.93 |
| 4065 | Parkfield-02, CA | 2004 | PARKFIELD-EADES | 360 | 6 | C | strike slip | 1.37 | 0.391 | 25.81 | 4.95 |
| 4097 | Parkfield-02, CA | 2004 | Slack Canyon | 90 | 6 | C | strike slip | 1.6 | 0.211 | 25.93 | 6.76 |
| 4103 | Parkfield-02, CA | 2004 | Parkfield-Cholame 4W | 90 | 6 | C | strike slip | 3.3 | 0.575 | 32.28 | 4.22 |
| 4107 | Parkfield-02, CA | 2004 | Parkfield-Fault Zone 1 | 90 | 6 | E | strike slip | 0.02 | 0.605 | 63.74 | 8.46 |
| 4113 | Parkfield-02, CA | 2004 | Parkfield-Fault Zone 9 | 360 | 6 | C | strike slip | 1.22 | 0.097 | 14.32 | 3.12 |
| 4115 | Parkfield-02, CA | 2004 | Parkfield-Fault Zone 12 | 360 | 6 | D | strike slip | 0.88 | 0.307 | 46.87 | 10.31 |
| 4126 | Parkfield-02, CA | 2004 | Parkfield-Stone Corral 1E | 90 | 6 | D | strike slip | 2.85 | 0.683 | 35.97 | 5.46 |
| 4348 | Umbria Marche, Italy | 1997 | Castelnuovo-Assisi | 0 | 6 | D | normal | 17.28 | 0.173 | 14.23 | 2.50 |
| 4451 | Montenegro, Yugoslavia | 1979 | Bar-Skupstina Opstine | 0 | 7.1 | C | reverse | 0 | 0.372 | 41.22 | 10.40 |
| 4458 | Montenegro, Yugoslavia | 1979 | Ulcinj-Hotel Olimpic | 90 | 7.1 | D | reverse | 3.97 | 0.248 | 51.58 | 19.21 |
| 4480 | L'Aquila, Italy | 2009 | L'Aquila-V. Aterno-Centro Valle | North | 6.3 | C | normal | 0 | 0.556 | 42.73 | 3.30 |
| 4482 | L'Aquila, Italy | 2009 | L'Aquila-V. Aterno -F. Aterno | East | 6.3 | C | normal | 0 | 0.402 | 32.00 | 5.27 |
| 4483 | L'Aquila, Italy | 2009 | L'Aquila-Parking | East | 6.3 | C | normal | 0 | 0.336 | 32.35 | 7.97 |
| 5274 | Chuetsu-oki, Japan | 2007 | NIG028 | EW | 6.8 | C | reverse | 15.32 | 0.131 | 11.39 | 4.70 |
| 6890 | Darfield, New Zealand | 2010 | Christchurch Cashmere High School | N10E | 7 | D | strike slip | 17.64 | 0.229 | 50.35 | 34.78 |
| 6911 | Darfield, New Zealand | 2010 | HORC | N18E | 7 | D | strike slip | 7.29 | 0.450 | 105.87 | 52.93 |
| 6962 | Darfield, New Zealand | 2010 | ROLC | S61W | 7 | D | strike slip | 0 | 0.325 | 56.98 | 21.06 |
| 8119 | Christchurch, New Zealand | 2011 | Pages Road Pumping Station | W | 6.2 | D | reverse oblique | 1.92 | 0.670 | 96.60 | 40.41 |
| 8123 | Christchurch, New Zealand | 2011 | Christchurch Resthaven | N02E | 6.2 | E | reverse oblique | 5.11 | 0.371 | 48.23 | 18.36 |
| 8142 | Christchurch, New Zealand | 2011 | TPLC | N27W | 6.2 | D | reverse oblique | 16.6 | 0.126 | 11.74 | 7.55 |
| 8164 | Duzce, Turkey | 1999 | IRIGM 487 | NS | 7.14 | C | strike slip | 2.65 | 0.282 | 28.95 | 16.63 |
| 51 | San Fernando | 1971 | 2516 Via Tejon PV | 65 | 6.61 | D | reverse | 55.2 | 0.026 | 6.03 | 10.21 |
| 88 | San Fernando | 1971 | Santa Felita Dam (Outlet) | 172 | 6.61 | C | reverse | 24.69 | 0.155 | 10.74 | 9.28 |
| 159 | Imperial Valley-06 | 1979 | Agrarias | 3 | 6.53 | D | strike slip | 0 | 0.287 | 34.92 | 9.36 |
| 179 | Imperial Valley-06 | 1979 | El Centro Array #4 | 140 | 6.53 | D | strike slip | 4.9 | 0.484 | 39.62 | 25.12 |
| 184 | Imperial Valley-06 | 1979 | El Centro Differential Array | 270 | 6.53 | D | strike slip | 5.09 | 0.353 | 75.54 | 57.12 |
| 256 | Mammoth Lakes-07 | 1980 | USC McGee Creek Inn | 0 | 4.73 | C | strike slip | 0.8 | 0.296 | 8.18 | 0.93 |
| 268 | Victoria, Mexico | 1980 | SAHOP Casa Flores | 10 | 6.33 | D | strike slip | 39.1 | 0.101 | 8.51 | 1.98 |
| 1198 | Chi-Chi, Taiwan | 1999 | CHY029 | E | 7.62 | C | reverse oblique | 10.96 | 0.289 | 35.24 | 11.11 |
| 1495 | Chi-Chi, Taiwan | 1999 | TCU055 | E | 7.62 | D | reverse oblique | 6.34 | 0.239 | 33.12 | 11.37 |
| 2114 | Denali, Alaska | 2002 | TAPS Pump Station #10 | 47 | 7.9 | D | strike slip | 0.18 | 0.333 | 115.66 | 53.41 |
| 3584 | Taiwan SMART1(25) | 1983 | SMART1 I10 | EW | 6.5 | D | reverse | 95.69 | 0.024 | 3.57 | 1.02 |
| 4101 | Parkfield-02, CA | 2004 | Parkfield-Cholame 3E | 90 | 6 | C | strike slip | 4.95 | 0.519 | 23.41 | 3.15 |
| 4102 | Parkfield-02, CA | 2004 | Parkfield-Cholame 3W | 90 | 6 | D | strike slip | 2.55 | 0.326 | 28.77 | 4.68 |
| 5783 | Iwate, Japan | 2008 | Semine Kurihara City | EW | 6.9 | C | reverse | 28.9 | 0.142 | 18.44 | 13.15 |
| 6927 | Darfield, New Zealand | 2010 | LINC | N23E | 7 | D | strike slip | 5.07 | 0.461 | 108.69 | 66.65 |